B54W101 35 00

MATERIAL SAFETY DATA SHEET

Section 1 — PRODUCT AND COMPANY IDENTIFICATION

PRODUCT NUMBER B54W101

PRODUCT NAME

Industrial Enamel, Pure White MANUFACTURER'S NAME

THE SHERWIN-WILLIAMS COMPANY

101 Prospect Avenue N.W.

Cleveland, OH 44115 DATE OF PREPARATION

02-JUL-06

HMIS CODES Health

F1ammabi1ity Reactivity

EMERGENCY TELEPHONE NO. (216) 566-2917

INFORMATION TELEPHONE NO. (216) 566-2902

2*

2

0

| Section 2 — COMPOSITION/ INFORMATION ON INGREDIENTS % by WT CAS No. INGREDIENT UNITS VAPOR PRESSURE |  |  |
| --- | --- | --- |
| 39 64742-88-7 Mineral Spirits |  |  |
| ACGIH TLV | 100 | ppm 2 mm |
| OSHA PEL | 100 | ppm |
| 0.1 100-41-4 Ethylbenzene |  |  |
| ACGIH TLV | 100 | ppm 7 . 1 mm |
| ACGIH TLV | 125 | ppm STEL |
| OSHA PEL | 100 | ppm |
| OSHA PEL | 125 | ppm STEL |
| 5 14807-96-6 Talc |  |  |
| ACGIH TLV | 2 | mg/m3 as Resp. Dust |
| OSHA PEL | 2 | mg/m3 as Resp. Dust |
| 15 13463-67-7 Titanium Dioxide |  |  |
| ACGIH TLV | 10 | mg/m3 as Dust |
| OSHA PEL | 10 | mg/m3 Total Dust |
| OSHA PEL | 5 | mg/m3 Respirable Fraction |

Section 3 — HAZARDS IDENTIFICATION

ROUTES OF EXPOSURE

INHALATION of vapor or spray mist.

EYE or SKIN contact with the product, vapor or spray mist. EFFECTS OF OVEREXPOSURE

EYES:  Irritation.

SKIN:  Prolonged or repeated exposure may cause irritation.

INHALATION:  Irritation of the upper respiratory system.

May cause nervous system depression.  Extreme overexposure may result in unconsciousness and possibly death. SIGNS AND SYMPTOMS OF OVEREXPOSURE

Headache, dizziness, nausea, and loss of coordination are indications of excessive exposure to vapors or spray mists.

Redness and itching or burning sensation may indicate eye or excessive skin exposure.

Continued on page 2

B54W101	page 2

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE

None generally recognized. 3ANCER INFORMATIpN

For complete discussion of toxicology data refer to Section 11.

Section 4 — FIRST AID MEASURES

EYES:  Flush eyes with large amounts of water for 15 minutes.

Get medical attention. SKIN:  Wash affected area thoroughly with soap and water.

Remove contaminated clothing and launder before re-use. INHALATION:  If affected, remove from exposure.  Restore breathing.

Keep warm and quiet. INGESTION:  Do not induce vomiting.

Get medical attention immediately.

Section 5 — FIRE FIGHTING MEASURES

FLASH POINT	LEL      UEL

101 F PMCC	1.0      6.0

FLAMMABILITY CLASSIFICATION

Combustible, Flash above 99 and below 200 F EXTINGUISHING MEDIA

Carbon Dioxide, Dry Chemical, Foam UNUSUAL FIRE AND EXPLOSION HAZARDS

Closed containers may explode when exposed to extreme heat.

Application to hot surfaces requires special precautions.

During emergency conditions overexposure to decomposition products may vcause a health hazard.  Symptoms may not be immediately apparent.  Obtain nedical attention. SPECIAL FIRE FIGHTING PROCEDURES

Full protective equipment including self-contained breathing apparatus should be used.

Water spray may be ineffective.  If water is used, fog nozzles are preferable.  Water may be used to cool closed containers to prevent pressure build-up and possible autoignition or explosion when exposed to extreme heat.

Section 6 — ACCIDENTAL RELEASE MEASURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED Remove all sources of ignition.  Ventilate the area. Remove with inert absorbent.

Section 7 — HANDLING AND STORAGE

STORAGE CATEGORY

DOL Storage Class II PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE

Contents are COMBUSTIBLE.  Keep away from heat and open flame.

Consult NFPA Code.  Use approved Bonding and Grounding procedures.

Keep container closed when not in use.  Transfer only to approved containers with complete and appropriate labeling.  Do not take internally, Keep out of the reach of children.

Continued on page 3

B54W101	page 3

Section 8 — EXPOSURE CONTROLS/PERSONAL PROTECTION

'RECAUTIONS TO BE TAKEN IN USE

Use only with adequate ventilation.

Avoid contact with skin and eyes.  Avoid breathing vapor and spray mist.

Wash hands after using.

This coating may contain materials classified as nuisance particulates [listed "as Dust" in Section 2) which may be present at hazardous levels )nly during sanding or abrading of the dried film.  If no specific dusts ire listed in Section 2, the applicable limits for nuisance dusts are ACGIH :LV 10 mg/m3 (total dust), 3 mg/m3 (respirable fraction), OSHA PEL 15 mg/m3 [total dust), 5 mg/m3 (respirable fraction). VENTILATION

Local exhaust preferable.  General exhaust acceptable if the exposure to materials in Section 2 is maintained below applicable exposure limits, lefer to OSHA Standards 1910.94, 1910.107, 1910.108. RESPIRATORY PROTECTION

If personal exposure cannot be controlled below applicable limits by ventilation, wear a properly fitted organic vapor/particulate respirator ipproved by NIOSH/MSHA for protection against materials in Section 2.

When sanding or abrading the dried film, wear a dust/mist respirator ipproved by NIOSH/MSHA for dust which may be generated from this product, anderlying paint, or the abrasive. PROTECTIVE GLOVES

Wear gloves which are recommended by glove supplier for protection igainst materials in Section 2. CYE PROTECTION

Wear safety spectacles with unperforated sideshields. )THER PRECAUTIONS

Intentional misuse by deliberately concentrating and inhaling the :ontents can be harmful or fatal.

Section 9 — PHYSICAL AND CHEMICAL PROPERTIES

PRODUCT WEIGHT	8.83  Ib/gal    1057 g/1

SPECIFIC GRAVITY	1.06

BOILING POINT	300 - 395 F     148 - 201 C

MELTING POINT	Not Available

VOLATILE VOLUME	57  %

EVAPORATION RATE	Slower than ether

VAPOR DENSITY	Heavier than air

SOLUBILITY IN WATER         N.A.

VOLATILE ORGANIC	COMPOUNDS (VOC Theoretical - As Packaged)

3.67 Ib/gal	440 g/1   Less Water and Federally Exempt Solvents

3.67 Ib/gal	440 g/1   Emitted VOC

Section 10 — STABILITY AND REACTIVITY

STABILITY — Stable 30NDITIONS TO AVOID

None known. INCOMPATIBILITY

None known.

lontinued on page 4

Section 11 — TOXICOLOGICAL INFORMATION

CHRONIC HEALTH HAZARDS

Ethylbenzene is classified by IARC as possibly carcinogenic to humans (2B) based on inadequate evidence in humans and sufficient evidence in laboratory animals.   Lifetime inhalation exposure of rats and mice to high ethylbenzene concentrations resulted in increases in certain types of cancer, including kidney tumors in rats and lung and liver tumors in mice. These effects were not observed in animals exposed to lower concentrations. There is no evidence that ethylbenzene causes cancer in humans.

Prolonged overexposure to solvent ingredients in Section 2 may cause adverse effects to the liver and urinary systems.

Rats exposed to titanium dioxide dust at 250 mg./m3 developed lung cancer, however, such exposure levels are not attainable in the workplace.

Reports have associated repeated and prolonged overexposure to solvents with permanent brain and nervous system damage.

TOXICOLOGY DATA

CAS No.     Ingredient Name

| 64742-88-7 | Mineral Spirits |  |  |  |
| --- | --- | --- | --- | --- |
|  | LC50 | RAT | 4HR | Not Available |
|  | LD50 | RAT |  | Not Available |
| 100-41-4 | Ethylbenzene |  |  |  |
|  | LC50 | RAT | 4HR | Not Available |
|  | LD50 | RAT |  | 3500 mg/kg |
| 14807-96-6 | Talc |  |  |  |
|  | LC50 | RAT | 4HR | Not Available |
|  | LD50 | RAT |  | Not Available |
| 13463-67-7 | Titanium Dioxide |  |  |  |
|  | LC50 | RAT | 4HR | Not Available |
|  | LD50 | RAT |  | Not Available |

Section 12 — ECOLOGICAL INFORMATION

ECOTOXICOLOGICAL INFORMATION No data available.

Section 13 — DISPOSAL CONSIDERATIONS

WASTE DISPOSAL METHOD

Waste from this product may be hazardous as defined under the Resource Conservation and Recovery Act (RCRA) 40 CFR 261.

Waste must be tested for ignitability to determine the applicable EPA hazardous waste numbers.

Incinerate in approved facility.  Do not incinerate closed container. Dispose of in accordance with Federal, State/Provincial, and Local regulations regarding pollution.

Continued on page 5 ^

' B54W101	page 5

Section 14 — TRANSPORT INFORMATION

f*>

No data available.

Section 15 — REGULATORY INFORMATION SARA 313 (40 CFR 372.65C) SUPPLIER NOTIFICATION

CAS No.       CHEMICAL/COMPOUND	% by WT   % Element

100-41-4 Ethylbenzene	0.1

CALIFORNIA PROP9SITION 65

WARNING:  This product contains chemicals known to the State of California to cause cancer and birth defects or other reproductive harm. TSCA CERTIFICATION

All chemicals in this product are listed, or are exempt from listing, on the TSCA Inventory.

Section 16 — OTHER INFORMATION

This product has been classified in accordance with the hazard criteria of the Canadian Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

The above information pertains to this product as currently