)

MATERIAL

SAFETY

DATA

SHEET

The information contained herein is based on data available at the time of preparation of this data sheet and which The Glidden Company believes to be reliable. However, no warranty is expressed or implied regarding the accuracy of this data. The Glidden Company shall not be responsible for the use of this information, or of any product, method or apparatus mentioned and you must make your own determination of its suitability and completeness for your own use, for the protection of the environment, and the health and safety of your employees and users of this material.

Complies with OSHA hazard communication standard 29CFR1910.1200.

| 1225 Series, 86, HMIS Rating | 87 |
| --- | --- |
| Health | 2 |
| Flammability | 4 |
| Reactivity | 0 |

1245 HMIS Rating

Health	*3

Flammability	4

Reactivity	0

"See Health Hazard Data for chronic health information.

Prepared November, 1988

THE GLIDDEN COMPANY

925 Euclid Avenue Cleveland, Ohio 44115

Emergency Telephone No. (216)826-5566

)

RUSTMASTER® Enamel No. 1225 Series; WOODMASTER® Polyurethane Nos. 86,87

FIRE & EXPLOSION HAZARD DATA

DOT Proper Shipping Name: Consumer Commodity

Hazard Class: ORM-D

Extinguishing Media: Dry chemical or foam

Unusual Fire and Explosion Hazards: Do not spray near open flame. Keep at room temperature as exposure to direct sunlight or other heat may cause bursting.

Special Fire Fighting Procedures: Water may be used to cool and protect exposed containers.

HEALTH HAZARD DATA

Primary Route(s) of Exposure: Inhalation

Skin Contact

Effects of Overexposure:

Inhalation: Irritation of respiratory tract. Prolonged inhalation may lead to fatigue, drowsiness, dizziness and/or lightheadedness, headache, uncoordination, nausea, vomiting, coughing, central nervous system depression, respiratory problems, intoxication, confusion, anesthetic effect or narcosis, difficulty of breathing.

Skin Contact: Irritation of skin. Prolonged or repeated contact can cause dermatitis, defatting. May be absorbed through skin.

Eye Contact: Irritation of eyes. Prolonged or repeated contact can cause blurred vision, tearing of eyes, redness of eyes, severe eye irritation.

Ingestion: Amounts ingested incidental to consumer and industrial handling are not likely to cause injury; however, ingestion of larger amounts may cause lung inflammation and damage due to aspiration of material into lungs.

Notice: Reports have associated repeated and prolonged occupational over-exposure to solvents with permanent brain and nervous system damage. Intentional misuse by deliberately concentrating and inhaling the contents may be harmful or fatal.

Emergency and First Aid Procedures:

Inhalation: Remove to fresh air. Restore and support continued breathing. Get

emergency medical attention. Have trained person give oxygen if necessary.

Get medical help for any breathing difficulty. Skin Contact: Wash off quickly with plenty of water, then soap and water;

remove contaminated clothing. Wash contaminated clothing before reuse. Eye Contact: Flush immediately with large amounts of water, especially under

lids, for at least 1 5 minutes. Obtain emergency medical treatment. Ingestion: If swallowed, obtain medical treatment immediately.

Medical Conditions Aggravated By Exposure: This product is not expected to aggravate existing medical conditions; however, ingredients contained in this product have been reported to aggravate preexisting eye, skin, respiratory disorders, lung disorders, asthma-like conditions, cardiac abnormalities, kidney disorders, liver disorders, nervous system disorders.

Supplemental Health Information: This product contains crystalline silica, which is considered a hazard by inhalation. The International Agency for Research on Cancer (IARC) has determined that there is sufficient evidence for the car-cinogenicity of crystalline silica to experimental animals, and limited evidence for the carcinogenicity of crystalline silica to humans. Crystalline silica is also a known cause of silicosis, a noncancerous lung disease. Exposure to materials in this product have been associated with possible cardiac abnormalities, blood abnormalities, liver damage, kidney damage. Excessive inhalation of solvent vapors under uncontrolled conditions may lead to unconsciousness, respiratory failure, asphyxiation, and even death.

| CHEMICAL NAME/COMMON NAME | CAS. NO. | ACGI 8-HOUR TWA | 1-TLV STEl | OSHA-PEL 8-HOUR TWA | LEL | V.P. |
| --- | --- | --- | --- | --- | --- | --- |
| Alkyd ResinfAlkyd Resin | 68072-13-9 | Not Established | Not Established | Not Established | N.A. | N.A. |
| Calcium Carbonate/limestone | 1317-653 | 10 mg/m3 | Not Established | 15 mgjm3 | N.A. | N.A. |
| Solvent Naphtha (Petroleum), Light Aliph.fVM&P Naphtha | 64742-89-8 | 300 ppm | Not Established | 500 ppm* | .9 | 11 @68"F. |
| Titanium DioxideiTitanium Dioxide, Rutile | 13463-67-7 | 10 mg/m3 | Not Established | 15mg/m3 | N.A. | N.A. |
| Magnesium Silicate HydratelTalc | 14807-96-6 | 2 mg/m4 | Not Established | 20 mppcf | N.A. | N.A. |
| Benzene, Dimethyl-IXylene | 1330-20-7 | 100 ppm | 150 ppm | 100 ppm | 1.0 | 5.1@68PF. |
| Basic Zinc MolybdatelBasic Zinc Molybdate | 13767-32-3 | 10mg¦mJ | Not Established | 15 mg/m3 | N.A. | N.A. |
| Acrylic Resin/Acrylic Resin | 9010-88-2 | Not Established | Not Established | Not Established | N.A. | N.A. |
| Aluminum Powder, Flake/Aluminum Powder | 7429-90-5 | 15mg/m3 | Not Established | 15mglm3 | N.A. | N.A. |
| Bronze Powder/Bronze Powder | 7440-50-8 | Not Established | Not Established | Not Established | N.A. | N.A. |
| Ethanol, 2-Butoxy-iButyl Cellosolve | 111-76-2 | Skin 25 ppm | Not Established | Skin 50 ppm | 1.1 | .76 @ 68°F. |
| Yellow Iron Oxide/Yellow Iron Oxide | 51274-00-1 | 10mg/m3 | Not Established | 1 5 mg/m3 | N.A. | N.A. |
| Metal Oxide/Red Iron Oxide | 1309-37-1 | 10 mg/m3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| Alkyd ResinfAlkyd Resin | 68015-39-4 | Not Established | Not Established | Not Established | N.A. | N.A. |
| 1 -Propanol, 2 Methyl-llsobutanol | 78-83-1 | 50 ppm | Not Established | 100 ppm | 1.2 | 8.0® 68'F. |
| Ethylbenzene/Ethylbenzene | 100-41-4 | 100 ppm | 125 ppm | 100 ppm | 1.0 | 7.1 @ 68GF. |
| Graphite/Graphite | 7782-42-5 | 2.5 mg/m't | Not Established | 15 mppcf | N.A. | N.A. |
| Silicon Dioxide/Silica, Crystalline, Quartz | 14808-60-7 | 0.1 mg/nrt | Not Established | (10mg/m3)/(%Si02 + 2)t | N.A. | N.A. |
| Alkyd ResinfAlkyd Resin | Supplier Confidential | Not Established | Not Established | Not Established | N.A. | N.A. |
| Distillates (Petroleum), Hydro-Treated, LightfMineral Spirits | 64742-47-8 | 100 ppm § | Not Established | 500 ppm§ | 0.7 | 3.0 @ 68'F. |
| Solvent Naphtha (Petroleum), Medium, Aiiph. /Mineral Spirits | 64742-88-7 | 100 ppm§ | Not Established | 500 ppm§ | 1.0 | 5@77°F. |
| Amorphous Silicon DioxidelSilica, Gel, Amorphous | 7631-86-9 | 10mg)m3 | Not Established | (80 mg/m3l/(% SiO,) | N.A. | N.A. |
| Alkyd ResinfAlkyd Resin | 68459-17-6 | Not Established | Not Established | Not Established | N.A. | N.A. |
| Polyurethane Alkyd/Polyurethane Alkyd | 67989-28-0 | Not Established | Not Established | Not Established | N.A. | N.A. |
| 2-Propanonef Acetone | 67-64-1 | 750 ppm | 1000 ppm | 1000 ppm | 2.6 | 186® 68'F. |
| Benzene, Methyl/Toluene | 108-88-3 | 100 ppm | 150 ppm | 200 ppm A | 1.2 | 22 @ 68°F. |
| Propane: PropellantfPropane: Propellant | 74-98-6 | 1000 ppm | Not Established | 1000 ppm | 2.3 | Gas @ 68°F. |
| Isobutane: Propellantflsobutane: Propellant | 75-28-5 | 800 ppm | Not Established | Not Established | 1.8 | Gas @ 68'F. |
| n-Butane: Propellantln Butane: Propellant | 106-97-8 | 800 ppm | Not Established | Not Established | 1.9 | Gas @ 68°F. |

LEL-The lower explosive limit is the lowest concentration (% of volatiles in air) that will produce a flash of fire when an ignition source is present.

V.P.-Vapor pressure in millimeters of mercury at the indicated temperature, mppcf—Millions of particles per cubic foot. N.A.-Not applicable. mg|m3-Milligrams per cubic meter. ppm-Parts per million. Wespirable Oust §As Stoddard solvent

Skin-Denotes that additional exposure, over and above airborne exposure, may result from skin absorption. * As Petroleum Distillates A Acceptable ceiling concentration-300 ppm Acceptable maximum peak above the acceptance ceiling concentration for an 8-hour shift -500 ppm/10 minutes IARC listing due to Crystalline Silica.

Carcinogenicity Listed By: NTP? No     IARC Monograph? Yes     OSHA Regulated? No

NO. 1225 LINE, 86, 87

| PRODUCT CODE NO. | 1225 | 1226 | 1227 | 1228 | 1229 | 1230 | 1231 | 1232 | 1233 | 1234 | 1235 | 1236 | 1237 | 1238 | 1239 | 1240 | 1241 | 1242 | 1243 | 1244 | 1245 | 1246 |  | 86 | 87 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ingredients, Wt. % |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alkyd Resin | 12-17 | 4-9 | 6-11 | 4-9 | 3-8 | 11-16 | 7-12 |  | 7-12 | 11-16 | 9-14 | 10-15 | 12-17 | 11-16 | 12-17 | 6-11 | 10-15 | 9-14 | 9-14 |  |  | 8-13 |  |  |  |
| • Calcium Carbonate | 1-5 |  | 1-5 |  | 1-5 |  |  |  |  | 1-5 |  |  |  |  | 1-5 |  | 1-5 | 1-5 | 1-5 |  |  | 1-5 |  |  |  |
| • Solvent Naphtha (Petroleum!, Light, Aliph. | 8-13 | 2-7 | 2-7 | 2-7 | 27 | 7-12 | 1-5 |  | 4-9 | 6-11 | 5-10 | 6-11 | 7-12 | 7-12 | 7-12 | 3-8 | 6-11 | 5-10 | 5-10 | 2-7 |  | 4-9 |  |  |  |
| • Titanium Dioxide |  |  | 4-9 |  | 2-7 |  | 2-7 |  |  | 1-5 | 1-5 | 1-5 |  |  |  | 6-11 | 1-5 | 1-5 | 1-5 |  |  | 2-7 |  |  |  |
| • Magnesium Silicate Hydrate |  |  |  | 4-9 | 2-7 | 1-5 | 1-5 |  |  |  |  |  |  | 1-5 |  |  |  |  |  | 4-9 | 1-5 |  |  |  |  |
| • Benzene, Dimethyl ID |  |  |  |  |  |  | 8-13 |  |  |  |  |  |  |  |  |  |  |  |  | 1-5 | 11-16 |  |  |  |  |
| • Basic Zinc Molybdate (1) |  |  |  |  |  |  | 1-5 |  |  |  |  |  |  |  |  |  |  |  |  | 1-5 |  |  |  |  |  |
| • Acrylic Resin |  |  |  |  |  |  |  | 3-8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Aluminum PowderfFlake |  |  |  |  |  |  |  | 1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Bronze Powder (1) |  |  |  |  |  |  |  |  | 1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Ethanol, 2-Butoxy-(1) |  |  |  |  |  |  |  |  |  |  | 1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Yellow Iron Oxide |  |  |  |  |  |  |  |  |  |  |  | 1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Metal Oxide |  |  |  |  |  |  |  |  |  |  |  |  | 1-5 |  |  |  |  |  |  | 1-5 |  |  |  |  |  |
| Alkyd Resin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3-8 |  |  |  |  |  |
| • 1-Propanol, 2-Methyl- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1-5 |  |  |  |  |  |
| • Ethylbenzened) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1-5 |  |  |  |  |  |
| • Graphite |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1-5 |  |  |  |  |
| • Silicon Dioxide |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3-8 |  |  |  |  |
| Alkyd Resin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4-9 |  |  |  |  |
| • Distillates (Petroleum), Hydro-Treated, Light |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8-13 | 8-13 |
| • Solvent Naphtha (Petroleum) Medium, Aliph. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5-10 | 49 |
| • Amorphous Silicon Dioxide |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1-5 |
| Alkyd Resin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6-11 | 4-9 |
| Polyurethane Alkyd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6-11 | 4-9 |
| • 2-Propanone¦1) | 18-23 | 40-45 | 28-33 | 36-41 | 32-37 | 2328 | 25-30 | 40-45 | 31-36 | 26-31 | 22-27 | 21-26 | 14-19 | 15-20 | 21-26 | 27-32 | 19-24 | 26-31 | 26-31 | 31-36 | 25-30 | 28-33 |  | 34-39 | 33-38 |
| •Benzene, Methyl (1) | 18-23 | 12-17 | 12-17 | 11 16 | 10-15 | 14-19 | 712 | 16-21 | 14-19 | 12-17 | 15-20 | 17-22 | 23-28 | 22-27 | 1621 | 13-18 | 17-22 | 12-17 | 13-18 | 6-11 | 9-14 | 12-17 |  |  |  |
| • Propane: Propellant | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 1722 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 | 17-22 |  | 17-22 | 1722 |
| • Isobutane: Propellant | 27 | 2-7 | 2-7 | 27 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 | 2-7 |  | 2-7 | 27 |
| • n-Butane: Propellant | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 4-9 | 49 | 4-9 | 49 | 4-9 | 4-9 | 4-9 |  | 4-9 | 4-9 |
| Physical Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| % Volatile by Vol. | 85.0 for all products on this sheet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wt. per Gallon, Ibs. | Not applicable for any product on this sheet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Boiling Range, "F. | 133-290 | 133-290 | 133-290 | 133-290 | 133-290 | 133-290 | 133-293 | 133-231 | 133-290 | 133-290 | 133-343 | 133-290 | 133-290 | 133-290 | 133-290 | 133-290 | 133-290 | 133-290 | 133290 | 133-293 | 133-293 | 133-290 |  | 133-374 | 133-374 |
| Flash Point, "F. | Propellant < 20 for all products on this sheet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lower Explosive Limit | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 1.2 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 09 | 0.9 | 0.9 | 1.0 | 0.9 |  | 0.7 | 0.7 |

• Hazardous Ingredients as defined by OSHA, ACGIH or The Glidden Company.

- SARA SECTION 313 CHEMICAL

- SARA SECTION 302 EHS

No. 331 75A

REACTIVITY DATA

Stability: Stable

Incompatibility: Oxidizers Acids Bases Aldehydes Alkylhalides Halogens Amines

Conditions to Avoid: Elevated temperatures

Contact with oxidizing agent

Hazardous Decomposition Products: Carbon Monoxide

Carbon Dioxide

Hazardous Polymerization: Will not occur

SPILL OR LEAK PROCEDURES

Steps to be taken in case material is released or spilled: Comply with all applicable health and environmental regulations. Eliminate all sources of ignition. Ventilate area. Spills may be collected with non-combustible absorbent materials.

Waste Disposal: Dispose in accordance with all applicable regulations. Avoid discharge to natural waters. Do not incinerate closed containers.

SPECIAL PROTECTION INFORMATION

Respiratory Protection: Control environmental concentrations below applicable standards. Where respiratory protection is required, use only NIOSH/MSHA approved respirators in accordance with OS HA Standard 29 CFR 1910.134.

Ventilation: Provide dilution ventilation or local exhaust to prevent build-up of vapors.

Personal Protective Equipment: Eye Wash

Safety Shower

Safety Glasses or Goggles

Impervious Gloves

SPECIAL PRECAUTIONS

Handling and Storage: Store below 100°F. Keep containers closed and away from heat and open flame or storage in direct sunlight.

Other Precautions: Use only with adequate ventilation. Do not take internally. Keep out of reach of children. Avoid contact with skin and eyes, and breathing of vapors. Wash hands thoroughly after handling, especially before eating or smoking. Keep containers tightly closed and upright when not in use. Avoid conditions which result in formation of inhalable particles such as spraying or abrading (sanding) painted surfaces. If such conditions cannot be avoided, use appropriate respiratory protection as directed under Special Protection Information. Empty containers may contain hazardous residue. Do not puncture or incinerate containers. Do not spray near open flame or fire.

P.9