FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

MSDS #:    480.00

Revision Date: November 25,2002

Section 1 — Chemical Product and Company Identification

Magnesium Sulfate

Flinn Scientific, Inc. P.O. Box 219 Batavia.IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Magnesium Sulfate anhydrous or heptahydrate

Synonym: epsom salts

CAS#: 7487-88-9 (anhydrous), 10034-99-8 (heptahydrate)

Section 3 — Hazards Identification

White, odorless powder.

Avoid inhalation. May irritate eyes and respiratory tract.

Avoid body tissue contact.

flinn Ar-A-GLANCE

Health-0 Flammabiliry-0 Reactivity-0 Exposure- 1 Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable solid.

NFPA code

None Established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and

phosphates.

Store in a cool dry place.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Magnesium Sulfate

MSDS #:    480.00

Revision Date:   November 25, 2002

Section 9 — Physical and Chemical Properties

White, odorless powder.

Solubility: Very soluble in water. Soluble in glycerol.

Anhydrous decomposes at 1124 C.

Formula: MgSO4 7H2O or MgSO4 (Anhydrous)

Formula Weight: 246.50 or 120.37

Specific Gravity: 2.65 (for anhydrous) 1.678 (for hydrous).

Section 10 — Stability and Reactivity

Shelf Life: Poor; substance hygroscopic.

Section 11 —Toxicoloqical Information

Acute effects: Irritant, abdominal pain

Chronic effects: N.A.

Target organs: Central nervous system, GI system

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N. A. = Not available, not all health aspects of this substance have been folly investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231-298-2).

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemicals—Your Best Choice

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   BatavialL 60510

(800) 452-1261 Fax (866) 452-1436