MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

CHROMATOGRAPHY SOLVENT	Replaces: 09/12/07

CAROLINA BIOLOGICAL	Printed: 05/20/08

1.	PRODUCT DESCRIPTION

Product Name:      Chromatography Solvent

Product Code(s):   74-6865, 74-6470, 74-6475, 84-0546, 10-1036, 68-8950

68-8951, 68-8953
Size:	120 ml

Chemical Name:     N/A
CAS Number:        See section 2
Formula:	See section 2

Synonyms:	Petroleum Ether:Acetone Solution

Distributor:       Carolina Biological Supply Company

2700 York Road

Burlington, NC 27215

Chemical Information:  800-227-1150 (8am-5pm (ET) M-F) Chemtrec (Transportation Spill Response 24 hours): 800-424-9300

2.	COMPOSITION/INFORMATION ON INGREDIENTS

Principal Hazardous Components:  Petroleum Ether (CAS#8032-32-4) 90%

Acetone (CAS#67-64-1) 10% TLV and PEL units:  Petroleum Ether

ACGIH-TLV  300 ppm (TWA) Acetone

ACGIH-TLV  750 ppm (STEL) 500 ppm (TWA) OSHA-PEL 1000 ppm (TWA)

3.	HAZARD IDENTIFICATION

Emergency Overview:  Irritating to skin and eyes.  May cause dizziness,

nausea, respiratory and cardiac effects.  May be harmful if inhaled or

swallowed.  Extremely flammable.

Potential Health Effects:

Eyes:  May cause irritation.

Skin:  May cause irritation.

Ingestion:  May cause gastrointestinal discomfort.

Inhalation:  May cause irritation to respiratory tract.

4.	FIRST AID MEASURES

Emergency and First Aid Procedures:

Eyes - Flush with water for at least 15 minutes, raising and lowering

eyelids occasionally.  Get medical attention if irritation persists.

Skin - Thoroughly wash exposed area for at least 15 minutes.  Remove

contaminated clothing.  Launder contaminated clothing before reuse.  Get

medical attention if irritation persists.

Ingestion - If swallowed, if conscious, give plenty of water.

Immediately call a physician or poison control center.  Never give

anything by mouth to an unconscious person.

Inhalation - Remove to fresh air.  Give oxygen if breathing is

difficult; give artificial respiration if breathing has stopped.  Keep

person warm and quiet, and get medical attention.

5.	FIREFIGHTING PROCEDURES

Flash Point(Method Used):  Petroleum Ether: -18 C cc

Acetone:         -20 C cc NFPA Rating:      Health:  1

Fire:  3

Reactivity:  0 Extinguisher Media:

Page 1 of 4

MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

CHROMATOGRAPHY SOLVENT	Replaces: 09/12/07

CAROLINA BIOLOGICAL	Printed: 05/20/08

Use dry chemical, C02 or appropriate foam. Flammable Limits in Air % by Volume: Petroleum Ether: LEL: 1.1  UEL: 5.9

Acetone:  LEL: 2.15  UEL: 13.0

Autoignition Temperature:  No information available Special Firefighting Procedures:

Firefighters should wear full protective equipment and NIOSH approved self-contained breathing apparatus.

Unusual Fire and Explosion Hazards:  Highly flammable, confined material can explode on heating.

6.	SPILL OR LEAK PROCEDURES

Steps to be Taken in Case Material is Released or Spilled:

Ventilate area of spill.  Eliminate all sources of ignition. Remove all non-essential personnel from area.  Clean-up personnel should wear proper protective equipment and clothing.  Absorb material with suitable absorbent and containerize for disposal.

7.	SPECIAL PRECAUTIONS

Precautions to be Taken in Handling or Storing: Keep container tightly closed. Store in a cool area away from ignition sources and oxidizers. Other Precautions: Do not breathe vapors.

8.	SPECIAL PROTECTION INFORMATION

Respiratory Protection(Specify Type) :

A NIOSH/MSHA chemical cartridge respirator should be worn if PEL or TLV is exceeded. Ventilation:

Local Exhaust:         Yes
Mechanical(General):   Recommended
Special :	No

Other:	No

Protective Gloves: Nitrile or equivalent Eye Protection:

Splash proof chemical safety goggles should be worn at all times. Other Protective Clothing or Equipment:

Lab apron, eye wash, and safety shower.

9.	PHYSICAL DATA

Molecular Weight:	Product is a mixture

Melting Point:	-73C(Petroleum Ether) -94.8C(Acetone)

Boiling Point:	35-60C(Petroleum Ether) 56C (Acetone)

Vapor Pressure:	No information available

Vapor Density(Air=l):	2.5 (Petroleum Ether)   2.0 (Acetone)

Specific Gravity(H20=l):	0.6 (Petroleum Ether)  0.79 (Acetone)

Percent Volatile by Volume:	90-100%

Evaporation Rate(BuAc=l):	6.82(Petroleum Ether)  14.4 (Acetone)

Solubility in Water:	Insoluble

Appearance and Odor:	Clear, colorless liquid; gasoline-like

odor

10.	REACTIVITY DATA

Stability:  Stable

Conditions to Avoid:  Vapor contact with spark, flame, or hot surface Incompatibility(Materials to Avoid):  Strong oxidizing agents Hazardous Decomposition Products:  COx

Page 2 of 4

MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

CHROMATOGRAPHY SOLVENT	Replaces: 09/12/07

CAROLINA BIOLOGICAL	Printed: 05/20/08

Hazardous Polymerization:  Will not occur

11.	TOXICITY DATA

Toxicity Data:

Petroleum Ether: inh-rat LC50:  3400 ppm/4H

Acetone: orl-rat LD50: 10700 mg/kg Effects of Overexposure:

Acute:  See section 3

Chronic: Not listed as causing cancer by NTP, IARC or OSHA. Conditions Aggravated by Overexposure:  No information available Target Organs:  Lungs, central nervous system and kidneys. Primary Route(s) of Entry: Inhalation, Ingestion

12.	ECOLOGICAL DATA

EPA Waste Numbers:  D001, U002

13.	DISPOSAL INFORMATION

Waste Disposal Methods:  Dispose in accordance with all applicable

Federal, State and Local regulations.

Always contact a permitted waste disposer (TSD) to assure compliance.

14.	TRANSPORT INFORMATION

Flammable  Liquid,   n.o.s.    (Petroleum  Ether,   Acetone),   3,   UN1993,II

15.	REGULATORY INFORMATION

EPA TSCA Status:  On TSCA Inventory

Hazard Category for SARA Section 311/312 Reporting:  Acute, Fire

|  |  | SARA | Sec. 313 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SARA EHS | Chemicals |  | CERCLA | RCRA |
| Product or | Sec. 302 | Name | Chemical | Sec. 103 | Sec . |
| Components | TPQ | List | Category | RQ Ibs. | 261.33 |
| Petroleum Ether Acetone | No No | No Yes | No No | No 5000 | No Yes |

16. ADDITIONAL INFORMATION

The information provided in this Material Safety Data Sheet represents a compilation of data drawn directly from various sources available to us. Carolina Biological Supply makes no representation or guarantee as to the suitability of this information to a particular application of the substance covered in the Material Safety Data Sheet.  Any employer must carefully assess the applicability of any information contained herein in regards to the particular use to which the employer puts the material. Glossary

ACGIH	American Conference of Governmental Industrial Hygienists

CAS Number..Chemical Abstracts Service Number

CERCLA	Comprehensive Environmental Response, Compensation, and

Liability Act

DOT	U.S. Department of Transportation

IARC	International Agency of Research on Cancer

mppcf	million particles per cubic foot

N/A	Not Available

NTP	National Toxicology Program

OSHA	Occupational Safety and Health Administration

Page 3 of 4

MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

CHROMATOGRAPHY SOLVENT	Replaces: 09/12/0

CAROLINA BIOLOGICAL	Printed: 05/20/08

PEL	Permissible Exposure Limit

ppm	parts per million

RCRA	Resource Conservation and Recovery Act

SARA	Superfund Amendments and Reauthorization Act

TLV	Threshold Limit Value

TSCA	Toxic Substances Control Act