FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:     860.00

Revision Date:    March 12, 200!

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Zinc Acetate

MSDS #:     860.00

Revision Date:   March 12, 2001

Section 1 —Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

Zinc Acetate

Flinn Scientific, Inc. P.O. Box 219 Batav.a, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

White crystalline plates with distinct acetic acid odor. Solubility: Water and alcohol. Formula: Zn(C2H3O2}2 2H2O Formula Weight: 219.50

Specific Gravity:   1.735

Melting Point: 200 C (decomposes)

| Section 2 — Composition. Information on Inaredients |  |
| --- | --- |
| Zinc Acetate, dihydrate Synonym: zinc diacetate CAS#; 5970-45-6 |  |
| Section 3 — Hazards Identification |  |
| White crystalline plates with distinct acetic acid odor. Moderately toxic by ingestion. Irritating to eyes. Avoid all body tissue contact. | flinn at-a-glance Health-2 FIarnmability-0 Reactivity-0 Exposure- 1 Storage- 1 0 is low hazard, 3 is high hazard |
| Section 4 — First Aid Measures |  |

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye; Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable soiid.	NFPA code

When heated to decomposition, emits toxic fumes of ZnO.	None established

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and SCBA with full facepiece operated in positive pressure mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and !3 for further information.

Section 7-—Handling and Storage			 n

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halid.es, sulfates, sulfttes, thiosulfates and phosphates. Store m a cool, dry place. Store in a Flinn Chem-Saf bag. Use and dispense in a hood.

Section 10 — Stability and Reactivity

Avoid contact with strong oxidizing agents. Shelf Life: Good, if kept tightly closed.

Section 11 — lexicological Infonriajjon

Acute effects Toxic, eye irritant Chronic effects N A Target organs N A.

ORL-RAT LD50: 794 mg/kg IHL-RAT LC50: N.A, SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of (his substance have been fullv investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name. Not regulated Hazard Class. N/A UN Number N/A N/A - Nol xpplicabk

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (209-I70-2).

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flin Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relatir thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no leg responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

Need a Chemical Fast?— Order from Flinn

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510 (800)452-1261 Fax (866) 452-1436

C 2002 Ftmn Scientific, Inc. All Rights Reserved.

PAGE1 OF 2

© 2002 Flmn Scientific. Inc   All Riehts Reserved.