SSS05011 MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLE, NORTH CAROLINA 28070

- MATERIAL SAFETY DATA SHEET -

THESE MATERIAL SAFETY DATA SHEETS ARE SENT TO YOU IN COMPLIANCE WITH 29 CFR 1910.1200 WHICH BECAME EFFECTIVE NOVEMBER 25, 1985.

IDENTITY AND DISTRIBUTOR'S INFORMATION

nfpa rating: Health, 2    Flammabi1ity, 3

Reactivity, 0 Reactivity, 0

Special, 0 HMis RATING: Health, 2    Flammabi1ity, 3

Personal Protection, B MANUFACTURED FOR:  Triple S

2 Executive Park Drive Bill erica, ma 01862

DOT HAZARD CLASSIFICATION:  ORM-D

IDENTITY (TRADE NAME AS USED ON LABEL): SSS CENSOR VANDAL MARK

REMOVER

date prepared:   05/07/1997       prepared by: dl

manufacturer's msds number: a00178  revision - 11

distributor's msds reference:   21720

information calls: (978) 667-7900

emergency response number : 1 (800) 255-3924

notice: judgment based on indirect test data

section 1 - material identification and information
components	sara    osha     acgih

CHEMICAL AND COMMON NAMES      III List   PEL (ppm)   TLV (ppm)

(Hazardous Components 1% or greater; Carcinogens 0.1% or greater)
Tolulene	Yes        200       100

(CAS # 108-88-3)

Acetone	No       1000       750

(CAS # 67-64-1)

Ethylene Glycol N-Butyl Ether   Yes         25        25 (CAS # 111-76-2)

Propane	No       1000      1000

(CAS # 74-98-6)

Methyl Ethyl Ketone	Yes        200       200

(CAS # 78-93-3)

Carcinogen Reference Source:  None of the above listed.

-1-

SECTION 2 - PHYSICAL/CHEMICAL CHARACTERISTICS BOILING POINT: N/A

specific gravity, (H20=l): Concentrate only = 0.84 VAPOR PRESSURE: PSIG @ 70 deg. F. (Aerosols): Max. 60 VAPOR PRESSURE (Non-Aerosols) (mm Hg and Temperature): N/A

VAPOR DENSITY (Ai r=l): N/E

EVAPORATION RATE, (     =1): N/E

SOLUBILITY IN WATER: Partial

WATER REACTIVE: No

APPEARANCE AND ODOR: Clear liquid, aromatic solvent odor.

SECTION 3 - FIRE AND EXPLOSION HAZARD DATA

FLAMMABILITY as per USA FLAME PROJECTION TEST (Aerosols):

EXTREMELY FLAMMABLE AUTO IGNITION TEMPERATURE:  N/E FLAMMABILITY LIMITS IN AIR BY % IN VOLUME:

% LEL: N/E        % UEL: N/E

Page 1

SSS05011

:LASH POINT AND METHOD USED (non-aerosols): N/A IXTINGUISHER MEDIA: Foam, dry chemical, carbon dioxide, water. iPEClAL fire fighting procedure: self-contained breathing apparatus.

JNUSUAL FIRE AND EXPLOSION HAZARDS: DO not expose aerO59ls to

temperatures above 130 degrees F. or the container may rupture.

SECTION 4 - REACTIVITY HAZARD DATA

stability: Stable

HAZARDOUS POLYMERIZATION: Will not occur

^COMPATIBILITY (MATERIALS TO AVOID): Acids and strong oxidizers.

:onditions to avoid: Open flame, welding arcs, heat, sparks.

HAZARDOUS DECOMPOSITION PRODUCTS: CO, C02.

SECTION 5 - HEALTH HAZARD DATA

DRIMARY ROUTES OF ENTRY: Inhalation, Skin Absorption <\CUTE EFFECTS:

inhalation: Excessive inhalation of vapors can be

harmful and may cause headache, dizziness, asphyxia, anesthetic effects and possible unconsciousness. Eye contact: Irritati9n. Skin Contact: Irritation, ingestion: Possible chemical pneumonitis if aspirated

into lungs.  Nausea.

chronic effects : (Effects due to excessive exposure to the raw materials of this mixture) May cause cardiac abnormality, liver abnormalities, kidney and/or lung damage.  Lab animals have experienced anemia, liver, kidney, lung and blood damage to Glycol Ether EB.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE: May

aggravate existing eye, skin or upper respiratory conditions.

EMERGENCY FIRST AID PROCEDURES:

EYE CONTACT: Flush with water for 15 minutes.  If irritated,

seek medical attention. SKIN CONTACT: Wash with soap and water,  if irritated, seek

medical attention. INHALATION: Remove to fresh air.  Resuscitate if necessary.

Get medical attention. INGESTION: DO NOT induce vomiting.  Drink two large glasses of

water. Get immediate medical attention. -2-

SECTION 6 - CONTROL AND PROTECTIVE MEASURES

RESPIRATORY PROTECTION (specify type): if vapor concentration

exceeds tlv, use respirator approved by U.S. Bureau of Mines for organic vapor.

PROTECTIVE GLOVES: Rubber gloves.

eye protection: Safety glasses recommended.

ventilation requirements: Adequate ventilation to keep vapor concentration below TLV.

OTHER PROTECTIVE CLOTHING AND EQUIPMENT: None

hygienic work PRACTICES: wash with soap and water before handling food.  Remove contaminated clothing.

SECTION  7 - PRECAUTIONS FOR SAFE HANDLING AND USE STEPS TO BE TAKEN IF MATERIAL IS SPILLED OR RELEASED:

SSS05011

Absorb with suitable medium,  incinerate or landfill according to local, state or Federal regulations. Prevent from entering drains or sewers.

waste DISPOSAL METHODS: Aerosol cans when vented to atmospheric pressure through normal use, pose no disposal hazard.

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE: DO not puncture

or incinerate containers. do not store at temperatures above 130 degrees F.

OTHER PRECAUTIONS AND/OR SPECIAL HAZARDS: KEEP OUT OF REACH OF

children. Avoid food contamination. Avoid breathing vapors. Remove ignition sources.

we believe the statements, technical information and recommendations contained herein are reliable, but they are given without warranty or guarantee of any kind.