MATERIAL SAFETY DATA SHEET

(comlies with OSHA Communication Standard 29 CFR 1910.1209 Dept. of Labor)

CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

PRODUCT NAME Aero Chem Caluking Tool Coolant

Revision Date :  9/10/97

MANUFACTURER/SUPPLIER’S NAME: Aerosol Systems, 9150 Valley View Road, Mandedoneia, OH  44056

TELEPHONE NUMBER (330) 487-4195

Product .		Composition		ACGIH TLV		OSHA PEL

CAS REG NO. UNITS	wgt%		TWA      STEL		TWA    STEL

Perchloroethylene

127-18-4		90-100		25          100		100   na   ppm

Carbon Dioxide

124-38-9		1-10		50000    300000	5000  na   ppm

HEALTH HAZARD DATA (ACUTE AND CHRONIC)

May cause dizziness or narcosis in high vapor concentrations.  Will cause defatting of skin.  Effects are reversible.  Long term exposure (years) to high concentrations of vapor may cause lung, liver or kidney damage.  The solvents listed have been reported to affect the pentral nervous system.  Aspiration hazard if swallowed.  Eye and skin irritant.  May irritate respiratory tract.  Deliberately concentrating and inhaling the vapor of the contents may be harmful or fatal.

Other:  Perchlorothylene may cause spleen and brain damage.

Carcinogenicity:  Perchloroethylene is listed as a carcinogen by IARC and NTP.  See Regulatory Section.

First Aid:  Give oxygen.  Do not induce vomiting.  Wash eyes and skin with water for 15 minutes.  Get medical help.  Never administer adrenalin following overexposure.

Routes of Entry:  Ingestion, inhalation, and skin

Signs and symptoms of Overexposure:  Inhalation – difficulty in breathing.  Skin – redness.  Ingestion – vomiting

Medial Conditions Aggravated by Exposure:  Heart disease, respiratory disorders

Boiling Point 250 degrees F

Vapor Pressure at 70 degrees F		80-85

Vapor Density (air=1)	4.0

Evaporation Rate (Butyl Acetate=1)	>1

Specific Gravity (water=1)	1.6

Solubility: nil

Appearance:  Clear

Odor:	Ethereal

pH:  Not applicable

Stability:	Stable

Incompatibility:	Strong oxidizers

Hazardous Thermal Decomposition/Combustion Products:  Carbon dioxide, hydrogen chloride.  Phosgene

Hazardous Polymerization:  Will not occur

California Proposition as:

Warning:  This product contains a chemical known to the State of California to cause cancer.

Ozone depleting substances:  None

HMIS:  Flammability – 0, Reactivity – 0, Health – 1

NFPA:  Flammability – 0, Reactivity – 0, Health – 2

SARA 313 Title IV – Toxic Chemical List:  Perchloroethylene (127-18-4)

DOT Shipping Name:  Aerosol, UN 1950

Respiratory Protection Equipment:  Use NIOSH approved respirator if TLV limit is exceeded

Ventilation:  Local exhaust – yes

Protective Gloves:  None required if spraying

Eye and Face Protection:  Safety glasses

Other Protective Equipment:  Long sleeves and long pants

Safe Handling:  Do not smoke while using.  Wash hands after use.

Flash Point:  None

Lower Explosion Limit:  ND

Upper Explosion Limit:  ND

Extinguishing Media:  Water fog, carbon dioxide, foam, dry chemical

Unusual Fire and Explosion Hazards:  Heated cans my burst.

Special Fire Fighting Procedures:  Keep containers cool.  Use equipment or shielding required to protect against bursting or venting containers.

Spill Procedures:  Use absorbent sweeping compound to soak up materials.  Put into container.  Dispose as hazardous waste.

Waste Disposal Methods:  Dispose as hazardous waste in accordance with EPA RCRA

Precautions to be taken in handling and storage:  Keep away from heat, sparks, or open flame.  Store at Temperatures below 120 degrees F.

Other Information:  When spraying more than one half can continuously or more then one can consecutively use NIOSH NP 300 level.