FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:      790.00

Revision Date:      February 24,2003

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Sudan 111

MSDS #:    790.00

Revision Date:   February 24, 2003

Section 9 — Physical and Chemical properties

Section 1 — Chemical Product and Company Identification

Sudan III

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (81X1)424-9300

Section 2 — Composition, Information on Ingredients

Sudan III

Synonym: solvent red 23. C.I. 26100

CAS#: 85-86-9

Section 3 — Hazards Identification

Reddish-brown powder. Odorless.

Substance not considered hazardous. However, not al! health aspects of this substance have been thoroughly

investigated.

Section 4 — First Aid Measures

flinn at-a-glance

Health-0

Flammability-0

Reactivity-0

Exposure-Q

Storage-0

Q is low hazard, 3 is high hazard

Melting Point: 199C(dec)

Reddish-brown powder. Odorless.

Solubility: Chloroform, acetic acid, acetone, and, only slightly

in alcohol. Not water.

Formula: C22H16N4O

Formula Weight: 352.40

Section 10 — Stability and Reactivity

Avoid contact with strong oxidizers and strong reducing agents. Shelf Life, indefinite.

S ecti o n 11 —• Tox Jcoiogi c a 11 jnfor matjon

ORL-RAT LD50; N.A. IHL-RAT LC50: N.A. SKN-RBTLD50:N.A.

Acute effects. N.A. Chronic effects: N.A. Target organs: N.A

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Call a physician, seek medical attention for further treatment, observation and support after first aid.

inhalation: Remove lo fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

internal: Rinse out mouth, give 1 lo 2 cups of water or milk, induce vomiting. Call a physician or poison control at once.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 5 — Fire Fighting Measures

Non flammable solid.

NFPA code None established

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A N/A = Not applicable

Section 15 — Regulatory Information

Section 6 — Accidental Release Measures

TSCA-listed: EfNECS-Iisted (201-638-4).

Restrict unprotected personnel from area. Sweep up. place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7—Handling and Storage

Flinn Suggested Chemical Storage Pattern: Organic #9. Store with dyes, indicators, and stains. Store in a cool, dry place.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory' chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8 — Exposure Controls, Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gioves and chemical-resistant apron.

Flinn MSDS Prevent Chemical Accidents

tlinn@f1innsci.com   www.flinnsci.com P.O. Box 219       Batavia IL 60510 (800)452-1261   Fa* (866) 452-1436

© 2003 Flinn Scientific, Inc. All Rights Reserved

page 1 of 2

O 2003 Flmn Scientific, Inc.  All Rights Reserved.

page 2 of 2