Commercial Service Produc

.* ^ <-.-r.,—r-. « s^. •*-<., r-	Winton Hill Technical Cente

>OCTER & GAMBLE	6071 Center Hill Road -

Cincinnati. Ohio 45224

MATERIAL SAFETY DATA SHEET

Issue Date: 1/94

|  | SECTION | I |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Emergency TclephoneJ&unber Procter & | Gamble Operator | 1-513-983-1100 |  |  |  |
| Identity: Foodservi^ Joy^Brand Codes 08426, 08163, 08415, |  | 02789, 08628, 08683, 04126 and |  | 03197) |  |
| Ingredients/ChemicaLr-Name: Cleaning and alcohol), water, stabilizing agents, colorant | sudsing agents (anionic and nonionic and perfume. |  | surfactants), dispensing |  | aid (ethyl |
| Other N.A. |  |  |  |  |  |

SECTION n - HAZARDOUS INGREDIENTS/IDENTITY INFORMATION

Hazardous Ingredients as defined by OSHA, 29 CFR 1910.1200.

ACGIH	OSHA	Other Limits

Chemical Name	Common Name	CAS No.        TLV	PEL	Recommended

Ethanol	Ethyl Alcohol	64-17-5	1000 ppm         1000 ppm	N.K.

1900 mg/m3

• .".",.

Toxic chemicals required to be reported by SARA, Section 313 of Title HI and of 40 CFR 372.

Chemical Category	Percent Bv Weight

Glycol ether	12%

DOT Classification: Combustible liquid not regulated in packaging of less thanl 10 gallons. (UN 1993)

| SECTION m - PHYSICAL/CHEMICAL CHARACTERISTICS |  |  |
| --- | --- | --- |
| Boiling Point (°F) | N.A. | ¦ Specific Gravity (H-^O = 1): 1.02 |
| Vapor Pressure (mm Hg): | N.A. | ¦ Percent Volatile by Volume (%): 65% |
| Vapor Density (Air=l): | N.A. | ¦ Evaporation Rate (nBuOAc=l): N.A. |
| Solubility in Water: | Appreciable | I Appearance and Odor Yellow liquid, lemon odor |

| SECTION | p/ - FLAMMABinTY AND REACTIVITY |  |
| --- | --- | --- |
| Flash Point (Method Used): 132°F closed cup. Although this product has a flashpoint below 200°F (closed cup), it is an aqueous solution containing rthyl alcoholwhich does not sustain combustion. |  |  |
| Explosive Limits: LEL: N.A. UEL: | N.A |  |
| Extinguishing Media: Use CO^, water | or dry chemical. |  |
| Special Fire Fighting Procedures: None | Known |  |
| Unusual Fire Hazards: None Known |  |  |
| Stability ¦ Unstable: ¦ Stable: X | ¦ Conditions to Avoid: None 1 | Known |
| Incompatibility (Materials to avoid): | Products containing chlorine. |  |
| Hazardous Decomposition/By Products: | Chloramincs in small quantities. |  |
| Hazardous ¦ May Occur: ¦ Conditions to Avoid: None Polymerization ¦ Will Not Occur X ¦ |  | Known |

6006C-0111N

SECTION V — HEALTH AND SAFETY DATA

Routes of Entry

_0ral, eye contact,. skin contact, inhalation

Health Hazards (Acute and Chronic)

None. Known

Signs and SYmptomsr  Expected to be enemic if accidentally ingested may cause nausea and or diarrhea.  May

May cause mild transient: eye irritation if accidently eye contact oddurs.^

Inhalation or granules may cause coughing, sore throat, •wheezing or transient"
shortness of breath.	•

Medical conditions generally aggravated by exposure

None

Emergency and First Aid Procedures

Eye Contact:    Flush thoroughly with water for 15 minutes.    Ingestion:    Dilute with

water or milk and treat symptomatically.    Skin Irritation:   Discontinue use, apply a cold compress to relieve irritation.    Inhalation:    Leave dusty area.

Other

N.A.

SECTION VI — PRECAUTIONS FOR SAFE HANDLING AND USE

Precaustion to be taken in handling and storing

Avoid moisure to prevent caking

Other precautions.  - None

Steps to be taken in case material is relesed or spilled

Sweep up and dispose

Waste Disposal Method

Contains biodegradable surfactants.  IF permitted, flush small quantities down sewer drain with large excess of water or dispose of at a landfill.  Disposal is to be performed in compliance with all regulations.

«*;

«*£

't»

| SECTION VII — SPECIAL PROTECTION INFORMATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Respiratory Protection None reouired with |  |  | normal | use |
|  | U3CU. EXHAUST None recuired with |  |  | SPKIAL normal use None |
|  | MEchanicl | Acceptable |  | None |
| en !>«QTECTtoN None recuired |  | With | normal | I Protective Gloves _ _ , _, Plastic or rubber gloves for extended wear • |
| OTXtR WK3TKTIVE 6QUIPVK !3cne recuired |  | •NT With | rcrrral | use. r |
| /* |  |  |  |  |