PIRS

Page 1 of4

MATERIAL SAFETY DATA SHEET:GROUND ASSAULT (GAL). O/G

DATE OF ISSUE: 11/13/2003

Product Code Number

1182

Chemical Name & Synonyms:

N/A

Chemical Family:

PETROLEUM MIXTURE

Manufacturer's Name:

MANTEK, DIVISION OF NCH CORP.

Address:

BOX 152170

IRVING, TEXAS 75015

Prepared By:

L Boynton/Chemist

SUPERCEDES: 04/23/2003

SECTION I - GENERAL INFORMATION

Trade Name & Synonyms:

GROUND ASSAULT (GAL), O/G Formula Mixture: X

Emergency Phone Number

800-424-9300

SECTION II - HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS
Chemical Name (Ingredients)	Hazard         TLV	PEL	STEL	CAS #

PERMETHRIN	IRRITANT    N/E1.	N/E 2.	N/E	52645-53-1

ALIPHATIC PETROLEUM DISTILLATES	IRRITANT    5 MG/M3S 1      5 MG/M3$ 2     10MG/M3$ 1      64742-47-8

$ OIL MIST VALUES

SECTION III - PHYSICAL DATA

Boiling Point (f):

Vapor Pressure (MM HG):

Vapor Density (Air=1):

PH@100%:

% Volatile by Volume:

H20 Solubility:

>200°

0.06

6.8

7.5 @ 5%

69

EMULSIFIABLE

Specific Gravity (H20=1):

Color:

Odor:

Clarity

Evaporation Rate (BU A/C=1):

Viscosity:

0.939

GOLDEN

PETROLEUM

TRANSPARENT

0.01

NON-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD
Flash Point:	Flammable Limits:	LEL:	UEL:

>200°F/T.C.C.	ALIPHATIC PET DISTILLATES	0.6%	7.0%

Extinguishing Media:

Foam:X	Alcohol Foam:	CO2:X	Dry Chemical:X	Water Spray:	Other:

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. COOL FIRE-EXPOSED CONTAINERS WITH WATER SPRAY TO PREVENT BURSTING.

Unusual Fire and Explosion Hazards:

THE USE OF WATER SPRAY (FOG) WHILE EFFECTIVE, MAY CAUSE FROTHING AND FOAMING. NEVER USE A WATER JET AS THIS WILL JUST SPREAD THE FIRE.

NFPA Hazard Rating:	(0=lnsignificant 1=Slight 2=Moderate 3=High 4=Extreme)

Health:1	Flammability:1	Instability:!	Special:

SECTION V - HEALTH HAZARD DATA

Threshold Limit Value: 5MG/M3ASOILMIST1. Effects of Overexposure:

-Acute(Short Term Exposure)

Page 2 of 4

r

EYE CONTACT: CAUSES IRRITATION SEEN AS TEARING, REDNESS, AND BLURRED VISION. SKIN CONTACT: MAY CAUSE IRRITATION SEEN AS ITCHING AND REDNESS. PRODUCT MAY BE ABSORBED THROUGH THE SKIN IN HARMFUL AMOUNTS. INHALATION: MAY CAUSE RESPIRATORY IRRITATION SEEN AS COUGHING AND SNEEZING. AT LOW LEVELS, NO HARMFUL EFFECTS ARE EXPECTED. AT HIGH VAPOR CONCENTRATIONS, INHALATION MAY CAUSE CENTRAL NERVOUS SYSTEM EFFECTS SUCH AS HEADACHE, DIZZINESS, DROWSINESS, WEAKNESS, UNCONCIOUSNESS, POSSIBLE ANESTHETIC EFFECTS FROM CENTRAL NERVOUS SYSTEM DEPRESSION, AND MAY BE FATAL. INGESTION: MAY CAUSE IRRITATION WITH POSSIBLE CRAMPS, NAUSEA, VOMITING, AND DIARRHEA. MAY RESULT IN EFFECTS SIMILAR TO INHALATION. INGESTION AND SUBSEQUENT VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL.

-Chronic (Long Term Exposure)

ON RARE OCCASIONS, PROLONGED AND REPEATED EXPOSURE TO HYDROCARBON MIST POSES A RISK OF CHRONIC LUNG INFLAMMATION. THIS CONDITION IS USUALLY ASYMPTOMATIC AS A RESULT OF REPEATED SMALL ASPIRATIONS. SHORTNESS OF BREATH AND COUGHING ARE THE MOST COMMON SYMPTOMS. ASPIRATION MAY LEAD TO PULMONARY EDEMA AND HEMORRHAGE AND MAY BE FATAL. SIGNS OF LUNG INVOLVEMENT INCLUDE INCREASED RESPIRATION AND HEART RATES AS WELL AS A BLUISH DISCOLORATION OF THE SKIN. CHRONIC SKIN CONTACT MAY PROMOTE DERMATITIS AND OIL ACNE. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA, AND DERMATITIS. TARGET ORGANS: CENTRAL NERVOUS SYSTEM. THE PRIMARY ROUTES OF EXPOSURE ARE SKIN AND EYE CONTACT.

Primary Routes of Entry:	lnhalation:X	Ingestion:	Absorption:X

Emergency and First Aid Procedures: -Inhalation:

REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

-Eye Contact:

RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Skin Contact:

WASH AFFECTED AREAS WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Ingestion:

GIVE 1 CUP OF WATER, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION. DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

-Notes to Physician:

INGESTION AND SUBSEQUENT VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL. DEPENDING ON THE AMOUNT INGESTED AND RETAINED AS WELL AS THE TOXICITY OF THE PRODUCT, GASTRIC LAVAGE SHOULD BE CONSIDERED. KEEP PATIENT'S HEAD BELOW HIPS TO PREVENT PULMONARY ASPIRATION. IF COMATOSE, A CUFFED ENDOTRACHAEL TUBE WILL PREVENT ASPIRATION. FOR SKIN EFFECTS, A HIGHLY EFFICIENT THERAPEUTIC AGENT FOR PYRETHROID EXPOSURE IS TOPICAL APPLICATION OF TOCOPHEROL ACETATE, (VITAMIN E) ACETATE.

Page 4 of 4

ALL PESTICIDE HANDLERS (MIXERS, LOADERS, AND APPLICATORS) MUST WEAR A WATERPROOF APRON OR LONG-SLEEVED SHIRT AND LONG PANTS, SOCKS, AND SHOES.

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:	lndoors:X	Outdoors:	Heated:	Refrigerated:/

Minimum Temperature:40°F	Maximum Temperature: 100°F

Precautions to be taken in Handling and Storing:

DO NOT ALLOW PESTICIDE TO CONTAMINATE FOOD, FEED, OR NATURAL BODIES OF WATER. ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER. KEEP THE CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. EMPTY CONTAINERS MAY CONTAIN PRODUCT RESIDUES WHICH MAY EXHIBIT THE HAZARDS OF THE PRODUCT.

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS. TRACES OF FREE ETHYLENE OXIDE MAY BE PRESENT IN THIS PRODUCT AND COULD ACCUMULATE IN THE HEADSPACE OF STORAGE AND TRANSPORT VESSELS.

SECTION XI - REGULATORY INFORMATION
Chemical Name	CAS Number	Upper % Limit

PERMETHRIN	52645-53-1	25.6

Those Ingredients listed above are subject to the reporting requirements of 313 of Title III& of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR part 372.

PLEASE CALL 1-800-527-9919 FOR ADDITIONAL INFORMATION IF YOU ARE A CALIFORNIA CUSTOMER. THIS MSDS IS NOT INTENDED FOR USERS IN THE STATE OF CALIFORNIA

SECTION XII • REFERENCES

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND BIOLOGICAL EXPOSURE

- OSHA PEL.

- VENDOR'S MSDS.

- REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFOWeb, 2003.

ALL THE COMPONENTS OF THIS PRODUCT ARE IN COMPLIANCE WITH THE TOXIC SUBSTANCES CONTROL ACT (T

IRR: IRRIT

LEL:LOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATI

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORI MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, stc

tavmrs

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:
IARC: No	NTP: No	OSHA: No	ACGIH: No	OTHER: No

VOC CONTENT: 0% BY WEIGHT; 0% BY VOLUME; 0 G/L

PERMETHRIN

ORL-RAT LD50: 383 MG/KG 4. IHL-RAT LC50: 485 MG/M3 4. SKN-RAT LD50: 1750 MG/KG 4. SKN-RBT SDT: 500 MG/24H MILD 4.

A STATISTICALLY SIGNIFICANT INCREASE IN THE INCIDENCE OF LUNG AND LIVER TUMORS WAS OBSERVED T

ALIPHATIC PETROLEUM DISTILLATES IHL-RAT LC50: >290 PPM    3.

PFGGY TAYNF.S

ORL-RAT LD50: >10 G/KG	3.

SKN-RBT LD50: >3 G/KG	3.

SKN SENSITIZER: NO	3.

SKN IRRITATION: SLIGHT	3.

EYE IRRITATION: SLIGHT	3.

THIS HYDROCARBON WAS ADMINISTERED ORALLY 5 DAYS/WEEK TO MALE AND FEMALE RATS AT 100, 500 OR 1 HYDROCARBON MISTS ARE REPORTED TO HAVE LOW ACUTE AND SUB-ACUTE TOXICITIES IN ANIMALS. EFFECTS THIS PRODUCT IS FORMULATED WITH MINERAL OILS WHICH ARE CONSIDERED TO BE SEVERELY REFINED AND

SECTION VII - REACTIVITY DATA

Stability:	Stable:X	Unstable:

Conditions to Avoid: NONE KNOWN. Incompatibility (Materials to Avoid):

STRONG OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE; ACIDS AND BASES.

Hazardous Decomposition Products:

OXIDES OF CARBON AND NITROGEN.

Hazardous Polymerization:	May Occur:	Will Not OccurX

Conditions to Avoid: N/A

SECTION VIII - SPILL OR LEAK PROCEDURES Steps to be Taken if Material is Released or Spilled:

WEAR APPROPRIATE PROTECTIVE CLOTHING. USE CARE AS SPILLS MAY BE SLIPPERY. DIKE AND CONTAIN SPILL. ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. IF NECESSARY WASH AREA WITH WATER AND PICK UP WASH WATER FOR DISPOSAL. PREVENT PRODUCT FROM CONTAMINATING SOIL OR FROM ENTERING SEWAGE AND DRAINAGE SYSTEMS AND BODIES OF WATER.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS. Neutralizing Agent:

N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE EXCESSIVE LEVELS OF VAPORS OR MISTS. LOCAL VENTILATION IS PREFERRED, BECAUSE IT PREVENTS DISPERSION INTO WORK AREAS BY CONTROLLING IT AT ITS SOURCE.

Respiratory Protection:

RESPIRATORS SHOULD BE SELECTED BY AND USED UNDER THE DIRECTION OF A TRAINED HEALTH AND SAFETY PROFESSIONAL FOLLOWING REQUIREMENTS FOUND IN OSHA'S RESPIRATOR STANDARD (29 CFR 1910.134) AND ANSI'S STANDARD FOR RESPIRATORY PROTECTION (Z88.2-1992). USE ONE OF THE FOLLOWING MINE SAFETY AND HEALTH ADMINISTRATION (MSHA)/NATIONAL INSTITUTE FOR OCCUPATIONAL SAFETY AND HEALTH ADMINISTRATION (NIOSH) RESPIRATOR TYPES WITH APPROVAL NUMBER PREFIXES: TC-21C (DUST/MIST FILTERING) TC-19C (SUPPLIED-AIR) TC-13F (SELF-CONTAINED BREATHING APPARATUS (SCBA) TC-14G (ORGANIC-VAPOR REMOVING WITH PRE-FILTER FOR REMOVING PESTICIDES)

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES IF REPEATED OR PROLONGED SKIN CONTACT IS LIKELY. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR HAND PROTECTION, 29 CFR 1910.138. Eye Protection:

SAFETY GLASSES WITH SIDE SHIELDS IF THE METHOD OF USE PRESENTS THE LIKELIHOOD OF EYE CONTACT. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR EYE AND FACE PROTECTION, 29 CFR 1910.133. Other Protection: