flinn scientific, inc.

^   Material Safety Data Sheet (MSDS)		msds * 432.00

: (MSDS)

•""•"•••'••	Revision Date: December 21, 2007

SECTION 1 — CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Lead

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

SECTION 2 — COMPOSITION. INFORMATION ON INGREDIENTS

Lead

CAS#: 7439-92-1

SECTION 3 — HAZARDS IDENTIFICATION

Heavy, ductile, gray solid. Forms: foil, sheets, shot, strips, and wire. Odorless.	flinn Ai-A-GLANCE

Lead as a powder or dust is toxic by ingestion or inhalation. Lead and lead compounds are possible	Health-0

carcinogens. Avoid ingestion and inhalation.	Flammability-0

Emits highly toxic fumes of Pb when heated.	Reactivity-0

Chronic exposure to inorganic lead via inhalation or ingestion can result in accumulation in and	.Exposure-0

damage to the soft tissues and bones.	Storage-0

0 is low hazard, 3 is high hazard SECTION 4 — FIRST AID MEASURES

Call a physician, seek medical attention for further treatment, observation and support after first aid. ^_^    inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately. Eye: Immediately flush with fresh water for 15 minutes. External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. After vomiting, give mixture of 2 Tbs. of activated charcoal mixed with one cup of water. Call a physician or poison control at once.

SECTION 5 — FIRE FIGHTING MEASURES

Finely divided lead dust is flammable.	NFPA code
Emits highly toxic fumes of Pb when heated.	None

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE	Established
and SCBA with full facepiece operated in positive pressure mode.

SECTION 6 — ACCIDENTAL RELEASE MEASURES

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

SECTION 7 — HANDLING AND STORAGE

Flinn Suggested Chemical Storage Pattern: Inorganic #1. Store with metals and metal hydrides. Use fume hood when handling powder form.

SECTION 8 — EXPOSURE CONTROLS. PERSONAL PROTECTION

\void contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. 7se ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 0.15 mg/m3 (ACGIH)

flinn scientific, inc.

Material Safety Data Sheet     Lead

MSDS#:   432.00

Revision Date: December 21,2007

CTION 9 — PHYSICAL AND CHEMICAL PROPERTIES

Heavy, ductile, gray solid.

Solubility: Soluble in dilute nitric acid. Insoluble in water.

Lead wire also contains 1% antimony (CAS #7440-36-0)

Formula: Pb

Formula Weight: 207.19

Specific Gravity: 11.35 Melting Point: 327.4 C

SECTION 10 — STABILITY AND REACTIVITY

Avoid strong acids, ammonium nitrate, hydrogen peroxide, sodium azide, zirconium, sodium acetylide and chlorine. Shelf Life: Indefinite.

SECTION 11 — TOXICOLOGICAL INFORMATION

Acute effects: N.A.

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

Chronic effects: Anemia, reproductive hazard

Target organs: Nerves, blood, kidneys, female/male reproductive

system

N.A. = Not available, not all health aspects of this substance have been fully investigated.

SECTION 12 — ECOLOGICAL INFORMATION

Data not yet available.

SECTION 13 — DISPOSAL CONSIDERATIONS

"Sase consult with state and local regulations.

:nn Suggested Disposal Method #27d is one option.

SECTION 14 —TRANSPORT INFORMATION

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A N/A = Not applicable

SECTION 15 — REGULATORY INFORMATION

TSCA-listed, EINECS-listed (231-100-4), RCRA code D008.

SECTION 16 — OTHER INFORMATION

This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific, Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. The data should not be confused with local, state, federal or insurance mandates, regulations, or requirements and CONSTITUTE NO WARRANTY. Any use of this data and information must be determined by the science instructor to be in accordance with applicable local, state or federal laws and regulations. The conditions or methods of handling, storage, use and disposal of the product(s) described are beyond the control of Flinn Scientific, Inc. and may be beyond our knowledge. FOR THIS AND OTHER REASONS, WE DO NOT ASSUME RESPONSIBILITY AND EXPRESSLY DISCLAIM LIABILITY FOR LOSS, DAMAGE OR EXPENSE ARISING OUT OF OR IN ANY WAY CONNECTED WITH THE HANDLING, STORAGE, USE OR DISPOSAL OF THIS PRODUCT(S).

Consult your copy of the Flinn Chemical Catalog/Reference Manual for additional information about laboratory chemicals

©Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2