MNT13510 MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLE, NORTH CAROLINA 28070

- MATERIAL SAFETY DATA SHEET -

THESE MATERIAL SAFETY DATA SHEETS ARE SENT TO YOU IN COMPLIANCE WITH 29 CFR 1910.1200 WHICH BECAME EFFECTIVE NOVEMBER 25, 1985. EMERGENCY PHONE:  (704) 875-6513 MSDS REFERENCE:    13510 REVISION DATE:     9/27/93

SECTION 1 - IDENTIFICATION

manufacturer:     Maintenance supply Service corporation

Post Office box 498 Huntersville, North Carolina 28078

PRODUCT:         Alkaline surfactants

trade name:      Jet stream Concentrate

SECTION 2 - HAZARDOUS INGREDIENTS
COMPOSITION	%	TLV

Tetrapotassium pyrophospate	8.0    Not Established

Trisodium phosphate	8.0    Not Established

Dipropylene glycol monomethyl ether     2.4     100 PPM

SECTION 3 - PHYSICAL DATA

BOILING POINT: 212 degrees F.

solubility in water: Complete

SPECIFIC GRAVITY, (H20=l): 1.21

PERCENT VOLATILE BY VOLUME: 75

P H : 13.5

appearance and ODOR: clear liquid with chemical odor

SECTION 4 - FIRE AND EXPLOSION HAZARDS

NOT FLAMMABLE

SECTION 5 - HEALTH HAZARD DATA THRESHOLD LIMIT VALUE: Not Established

primary routes OF entry: Skin absorption; ingestion

EFFECTS OF OVEREXPOSURE:

EYES: Severe irritant SKIN: Severe irritant

INGESTION: severe gastrointestinal distress MEDICAL CONDITIONS AGGRAVATED BY OVEREXPOSURE: Prior skin,

gastrointestinal disorders EMERGENCY FIRST AID PROCEDURE:

EYES: Flush with water for 15 minutes; see a doctor

-1-

immediately. SKIN: Flush with water for 15 minutes.  If irritation

persists, contact a physician. ingestion: Drink large quantities of milk or water.

DO NOT induce vomiting. Go immediately to emergency room of hospital.

Chemicals in this product are not currently listed as carcinogens.

SECTION 6 - REACTIVITY DATA

stability: stable

Page 1

MNT13510 HAZARDOUS POLYMERIZATION: Will not OCCUP

incompatibility: high temperature, acids

SECTION  7 - SPILL OR LEAK PROCEDURES

SPILL OR LEAK PRECAUTIONS:

SMALL SPILLS:  Flush into drain with large quantities of

water.

use absorbent material   to soak up material. Dispose of waste at approved landfill   in accordance with  state  regulations. Follow State and  Local   regulations for disposal of large non-reclaimable spills.

LARGE SPILLS:

waste disposal:

SECTION 8 - SPECIAL PROTECTION

protective GLOVES:  Use when handling concentrate

eye protection: Use safety glasses or goggles in handling

concentrate. OTHER PROTECTIVE EQUIPMENT: Safety apron is recommended when

handling concentrate.

SECTION 9 - PRECAUTIONS:  HANDLING, STORAGE AND USAGE PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE: Avoid

temperature extremes in storage; st9re away from acids; keep container lid sealed when not in use.

other PRECAUTIONS: if concentrate is spilled on clothes, wash thoroughly before reuse.

SECTION 10 - REGULATORY CONTROLS DEPARTMENT OF TRANSPORTATION:

DOT PROPER SHIPPING NAME: Caustic Alkali Liquid N.O.S (contains

sodium hydroxide, dipropylene glycol monomethyl ether) 8, UN 1719, PG II -2-