MATERIAL SAFETY DATA SHEtT: ( 4O46 79 -O476@>-UO 136644-5068

FREE W/MOLY AEROSOL

~DATE OF ISSUE 07/29/98

SECTION I - GENERAL INFORMATION

SUPERSEDES 07/22/97

CHEMICAL NAME & SYNONYMS	TRADE NAME & SYNONYMS

N/A	FREE W/MOLY AEROSOL

CHEMICAL FAMILY :	FORMULA

ALIPHATIC PETROLEUM DISTILLATE	X <--MIXTURE_

MANUFACTURE'S NAME:

CERTIFIED LABS, DIV. OF NCH CORP.

ADDRESS (NUMBER, STREET, CITY, STATE & ZIP CODE)

BOX 15217O

IRVING,	TEXAS

75015

PREPARED BY:	- PRODUCT CODE NUMBER  " EMERGENCY TELEPHONE NUMBER

G MARSHALL/CHEMIST	5O68	972-438-1381

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

CHEMICAL NAME (INGREDIENTS)  : ALIPHATIC PETROLEUM DISTILLATE

HAZARD	>COMB / IRR "  TLV	> 10O PPM  1 *  PEL	>5OO PPM  2 "

STEL(TWA)*->NOT EST.       CAS»-->8O52-41-3

ETHYL ACETATE

HAZARD	>FLAMM/ IRR "  TLV	>4OO PPM  1 "  PEL	>4OOPPM   2 "

STEL(TWA)*->NJOT EST.       CAS»- ->141 - 78-6

SODIUM PETROLEUM SULFONATE

HAZARD	>IRRITANT    "  TLV	>5 MG/M3 $1 ~  PEL	>5 MG/M3 $2 "

STEL(TWA)*->1O MG/M3 $ "  CAS#-->686O8-26-4 ~

LIGHT NAPHTHENIC HYDROTREATED DISTILLATE

HAZARD	>IRRITANT   ~  TLV	>5 MG/M3 $1 *  PEL	>5 MG/M3 $2 "

STEL(TWA)*->1O MG/M3 $ ~  CAS#-->64742-53-6 "

ISOBUTANE

HAZARD	>FLAMMABLE  *  TLV	>NOT EST.  1 "  PEL	>NOT EST. 2 ~

STEL(TWA)*->NOT EST.    ~  CAS*-->75-28-5

MATERIAL SAFETY DATA SHEET:    FREE W/MOLY AEROSOL	PAGE :  2

(CONTINUED) - SECTION II - HAZARDOUS INGREDIENTS

PROPANE

HAZARD	>ASPHYXIANT '  TLV	>25OO PPM 1 "  PEL	> 10OO PPM 2 ~

STEL(TWA)*->NOT EST.       CAS =- ->74-98-6

$ OIL MIST VALUES

HAZARD	>	~  TLV	>	"  PEL	>

STEL(TWA)*->	'  CASi*-->

SECTION III - PHYSICAL DATA
BOILING PT.  (F)    * 160	- SPEC. GRAVITY (H2O=1)" O.86O

VAPOR PR.  (MM HG) - 13.6	" COLOR	~ AMBER

VAPOR DENSITY      ~ 1.8	" ODOR	* SOLVENT

PH. 9 1OO%         ~ N/A	- CLARITY	" TRANSPARENT

% VOLATILE BY VOL - 79	" EVAPORATION RATE      ~ O 6

~ ( BU A/C = 1 )

H20 SOLUBILITY     - NEGLIGIBLE
VISCOSITY	- NON-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

FLASH POINT: BO F.	- FLAMMABLE LIMITS         ~   LEL     ~   UEL

T.C.C.	"ETHYL ACETATE	"2.2°/°       ~ 1 1 . 0%

EXTINGUISHING MEDIA "ALCOHOL"	DRY	WATER

X <--FOAM	<--FOAM   X <--C02   X <--CHEMICAL  X <--SPRAY   <--OTHER

SPECIAL FIRE FIGHTING PROCEDURES

SPRAY WATER ON AEROSOL CONTAINERS TO COOL. AVOIDING RUPTURING.  WEAR

FULL PROTECTIVE CLOTHING INCLUDING A SUPPLIED AIR DEVICE.

UNUSUAL FIRE AND EXPLOSION HAZARDS

MATERIAL SAFETY DATA SHEET:    FREE W/MOLY AEROSOL	PAGE :   3

(CONTINUED) - SECTION IV - FIRE AND EXPLOSION HAZARD

VAPORS ARE HEAVIER THAN AIR AND MAY TRAVEL TO DISTANT SOURCES OF IGNITION

AND FLASHBACK.    PRODUCT MAY PRODUCE A FLOATING FIRE HAZARD AS LIQUID

FLOATS ON WATER. FLAME EXTENSION IS > 18 INCHES. BURNBACK IS 5-6 INCHES.	^—.

AEROSOL LEVEL (NFPA SOB)  : 3

NFPA HAZARD RATING  (O= INS I GNI F ICANT ; 1 =SL I GHT ; 2 = MODERATE ; 3=HIGH ; 4=EXTREME ) : 1 <--HEALTH   3 <--FLAMMABILITY   O <--REACTIVITY    <--SPECIAL

SECTION V - HEALTH HAZARD DATA

THRESHOLD LIMIT VALUE:

NOT ESTABLISHED FOR MIXTURE. SEE SECTION II.

EFFECTS OF OVEREXPOSURE :

-	ACUTE -  (SHORT TERM EXPOSURE)
EYE CONTACT: CAUSES IRRITATION SEEN AS TEARING. REDNESS, AND

BURNING SENSATION.   PROLONGED CONTACT MAY LEAD TO SEVERE IRRITATION.  MAY

POSSIBLY CAUSE FINE VASCULAR LESSIONS OF THE CORNEA.

SKIN CONTACT: MAY CAUSE IRRITATION SEEN AS ITCHING AND REDNESS.   MATERIAL

MAY BE ABSORBED ACROSS MARKEDLY ABRADED SKIN WHEN CONTACT IS SUSTAINED.

INHALATION:   INHALATION MAY CAUSE CENTRAL NERVOUS SYSTEM EFFECTS SEEN AS

DIZZINESS, WEAKNESS. FATIGUE. NAUSEA. HEADACHE. POSSIBLE UNCONSCIOUSNESS.

AND EVEN DEATH.

INGESTION:  CAUSES GASTROINTESTINAL IRRITATION WITH POSSIBLE NAUSEA.

VOMITING. EUPHORIA, STAGGERING GAIT. LACK OF CORDINATION. AND OTHER EFFECTS

SIMILAR TO INHALATION.  INGESTION AND SUBSEQUENT VOMITING OF

THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN

CAUSE DAMAGE AND MAY BE FATAL.

-	CHRONIC -  (LONG TERM EXPOSURE)

PROLONGED OR REPEATED CONTACT MAY RESULT IN OIL ACNE, WHICH IS CHARACTERIZED BY BLACKHEADS WITH POSSIBLE SECONDARY INFECTION. CONSTITUENTS OF THIS PRODUCT HAVE BEEN ASSOCIATED WITH PHOTOSENSITIVITY. AN ABNORMAL SENSITIVITY OF THE SKIN TO SUNLIGHT.

LONG TERM REPEATED ORAL EXPOSURE MAY RESULT IN THE DEVELOPMENT OF

PROGRESSIVE LIVER INJURY WITH FIBROSIS.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY

AND SKIN CONDITIONS SUCH AS ASTHMA. EMPHYSEMA. AND DERMATITIS.

ON RARE OCCASIONS, PROLONGED AND REPEATED EXPOSURE TO OIL MIST POSES A

MATERIAL SAFETY DATA SHEET:    FREE W/MOLY AEROSOL	PAGE :  4

(CONTINUED) - SECTION V - HEALTH HAZARD DATA

RISK OF CHRONIC LUNG INFLAMMATION. THIS CONDITION IS USUALLY ASYMPTOMATIC A A RESULT OF REPEATED SMALL ASPIRATIONS. SHORTNESS OF BREATH AND COUGHING ARE THE MOST COMMON SYMPTOMS. ASPIRATION MAY LEAD TO PULMONARY EDEMA AND HEMORRHAGE AND MAY BE FATAL. SIGNS OF LUNG INVOLVEMENT INCLUDE INCREASED RESPIRATION AND HEART RATES AS WELL AS A BLUISH DISCOLORATION OF THE SKIN. TARGET ORGANS: LIVER. LUNGS. AND KIDNEY.

PRIMARY ROUTE OF ENTRY: X<-- INHALATION  <-- INGESTION  X<-- ABSORPTION

EMERGENCY & FIRST AID PROCEDURES

INHALATION :

REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR If BREATHING BECOMES DIFFICULT.

EYE CONTACT:

RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

SKIN CONTACT:

WASH AFFECTED AREAS WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

INGESTION :

GIVE 3 TO 4 GLASSES OF WATER. BUT DO NOT INDUCE VOMITING.  IF VOMITING OCCURS. GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION. DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

NOTES TO PHYSICIAN :

INGESTION AND SUBSEQUENT VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL.

SECTION VI - TOXICITY INFORMATION

PRODUCT CONTAINS CHEMICAL LISTED AS CARCINOGEN OR POTENTIAL	CARCINOGEN BY:

IARC   <--YES   NTP   <--YES  OSHA  <--YES  ACGIH   <--YES	OTHER   <--YES

X<--NO    NTP  X<--NO   OSHA X<--NO   ACGIH  X<--NO	OTHER  X<--NO

MATERIAL SAFETY DATA SHEET:    FREE W/MOLY AEROSOL

(CONTINUED) - SECTION VI - TOXICITY INFORMATION

ALIPHATIC PETROLEUM DISTILLATE	SODIUM PETROLEUM SULFONATE

EYE-HMN:  470 PPM/1EM        3.	ORL-RAT LDSO:  6.O G/KG  4.

IHL-CAT LCLO:   10 G/M3/2.5H 3.

ORL-RAT LDSO:  5 G/KG        4.

LIGHT NAPHTHENIC HYDROTREATED DISTILLATE   ETHYL ACETATE

NO TOXICITY DATA AVAILABLE	IHL-RAT LC5O:   16OO PPM       3.

ORL-RAT LDSO:  562O MG/KG    3.

SKN IRRITATION RET: SLIGHT   4.

EYE IRRITATION RBT: SLIGHT   4. PROPANE

NO TOXICITY DATA AVAILABLE ISOBUTANE ORL-RAT LC5O:  57 PPH/15M    3.

SECTION VII - REACTIVITY DATA X <--STABLE   <--UNSTABLE ~  CONDITIONS TO AVOID

STABILITY

AVOID HEAT, HOT SURFACES. SPARKS AND OPEN FLAMES.

INCOMPATABILITY  (MATERIALS TO AVOID)  :

STRONG OXIDIZING AGENTS SUCH AS BLEACH  STRONG ACIDS OR BASES,

AND ALKALI METAL HYDROXIDES.

HAZARDOUS DECOMPOSITION PRODUCTS

OXIDES OF CARBON. SMOKE. FUMES. NITROGEN, AND SULFUR.

WILL NOT         MAY    "  CONDITIONS TO AVOID

HAZARDOUS   " X <--OCCUR	<--OCCUR
POLYMERIZATION-

N/ A	~

SECTION VIII - SPILL OR LEAK PROCEDURES STEPS TO BE TAKEN IF MATERIAL IS RELEASED OR SPILLED:

MATERIAL SAFETY DATA SHEET:    FREE W/MOLY AEROSOL	PAGE :  6

(CONTINUED) - SECTION VIII - SPILL OR LEAK PROCEDURES

DUE TO THE NATURE OF THE AEROSOL PACKAGING. A LARGE SPILL IS UNLIKELY. FOR A SMALL SPILL. ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. WEAR APPROPRIATE PROTECTIVE CLOTHING.

WASTE DISPOSAL METHOD(S):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL. STATE. AND LOCAL REGULATIONS. TYPICAL DISPOSAL IS TO WRAP THE EMPTY AEROSOL CONTAINER IN SERVERAL LAYERS OF NEWSPAPER AND DISPOSE OF IN THE TRASH.

NEUTRALIZING AGENT : NONE KNOWN.

SECTION IX - SPECIAL PROTECTION INFORMATION

REQUIRED VENTILATION :

LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE HEAVY MISTS OR VAPORS.

RESPIRATORY PROTECTION :

A NIOSH/MSHA APPROVED RESPIRATOR IN POORLY VENTILATED AREAS AND/OR FOR

EXPOSURE ABOVE THE ACGIH TLV OR OSHA PEL OR WHERE MISTING EXISTS.

GLOVE PROTECTION :

NEOPRENE RUBBER OR NITRILE GLOVES IF REPEATED OR

PROLONGED SKIN CONTACT IS LIKELY.

EYE PROTECTION :

SAFETY GLASSES WITH SIDE SHIELDS IF THE METHOD OF

USE PRESENTS THE LIKELIHOOD OF EYE CONTACT.

OTHER °ROTECTION :

WEAR GENERAL DUTY WORK CLOTHING AND SHOES.

MATERIAL SAFETY DATA SHEET:    FREE W/MOLY AEROSOL	PAGE

(CONTINUED) - SECTION IX - SPECIAL PROTECTION INFORMATION

SECTION X - STORAGE AND HANDLING INFORMATION

STORAGE TEMPERATURE         INDOOR        HEATED        REFRIGERATED    OUTDOOR MAX: 120 F.  MIN:  35 F.     X

PRECAUTIONS TO BE TAKEN IN HANDLING & STORING USE WITH CAUTION AROUND HEAT, SPARKS. PILOT LIGHTS, STATIC ELECTRICITY AND OPEN FLAME.

OTHER PRECAUTIONS

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE

LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL

DIRECTIONS.

SECTION XI - REGULATORY INFORMATION

CHEMICAL NAME	C.A.S NUMBER       UPPER % LIMIT

N/A

THOSE INGREDIENTS LISTED ABOVE ARE SUBJECT TO THE REPORTING REQUIRMENTS OF 313 OF TITLE III OF THE SUPERFUND AMENDMENTS AND REAUTHORIZATION ACT OF 1986 AND 4O CFR PART 372.

IF UE (USE EXEMPTION) APPEARS UNDER UPPER % LIMIT, END USERS ARE EXEMPT FROM NOTIFICATION BECAUSE THE PRODUCT IS USED AND LABELED FOR ROUTINE JANITORIAL WORK. OR THE PRODUCT IS USED AND LABELED FOR FACILITY GROUNDS MAINTENANCE (SUCH AS FERTILIZERS AND HERBICIDES). OR THE PRODUCT IS USED AND LABELED FOR MAINTAINING MOTOR VEHICLES.

THIS MSDS IS NOT SUITABLE FOR USERS IN THE STATE OF CALIFORNIA.

MATERIAL SAFETY DATA SHEET:    FREE W/MOUY AEROSOL

(CONTINUED) - SECTION XIII - REFERENCES

INJURY OR PROPERTY DAMAGE CAUSED BY THE USE, STORAGE OR DISPOSAL OF THE PRODUCT IN A MANNER NOT RECOMMENDED ON THE PRODUCT LABEL.  USERS ASSUME ALL RISKS ASSOCIATED WITH SUCH UNRECOMMENDED USE, STORAGE, OR DISPOSAL OF THE PRODUCT.

: : UJQ' •'