103018-01

MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 ' HUNTERSVILLE, -NORTH CAROLINA 28078

- MATERIAL SAFETY DATA SHEET -

THESE MATERIAL SAFETY DATA SHEETS ARE SENT -"TO YOU Iff COMPLIANCE WITH 29 CFR 1910.1200 WHICH BECAME EFFECTIVE NOVEMBER 25, 1985.

EMERGENCY PHONE:  (704) 875-6513 MSOS REFERENCES   * 13510  -•REVISION DATES     4/12/88

SECTION 1. - IDENTIFICATION

MANUFACTURER:'      MAINTENANCE SUPPLY SERVICE CORPORATION

POST OFFICE BOX 498 HUNTERSVILLE, NORTH 'CAROLINA 28073 .

PRODUCT:	ALKALINE SURFACTANTS

TRADE NA«ES        JET STREAM CONCENTRATE  ;

SECTION 2 - HAZARDOUS INGREDIENTS

ILM

TETRAPOTASSIUM PYROPHOSPATE	8.0	NOT ESTABLISHED

TRISOOIUM PHOSPHATE      .	8.0	NOT ESTABLISHED

OIPROPYLEME SLYCOL MONOMETHYL ETHER	2,4	1OO PPM

___ ________ £ECT£ON_3 -

SOILING POINT: 212 DEGREES F.

SOLUBILITY IN WATER: COMPLETE

SPECIFIC GRAVITY, (H20=l>: 1.21

PERCENT VOLATILE 8Y VOLUME: 75

P H : 13.5

APPEARANCE AND ODOR: CLEAR LIQUID WITH CHEMICAL

SECTION 4 - FIRE AND EXPLOSION HAZARDS

MOT FLAMMABLE	:

SECTION 5 - HEALTH HAZARD DATA

THRESHOLD LIMIT VALUE: NOT ESTABLISHED

PRIMARY. ROUTES OF ENTRY: SKIN ABSORPTION! IN6ESTIOJM

EFFECTS OF OVEREXPOSURE:

EYES: SEVERE IRRITANT SKIN: SEVERE IRRITANT IMGESTION: SEVERE GASTROINTESTINAL DISTRESS

MEDICAL CONDITIONS AGGRAVATED SY CVEREXPOSUREs PRIOR SKIS, GASTROINTESTINAL DISORDERS

EYES: FLUSH WITH WATER FOR 15 HINUTESI SEE A DOCTOR
IMMEDIATELY.	-

SKIN: FLUSH WITH MATER FOR 15 MINUTES.  IF IRRITATION PERSISTS, CONTACT A PHYSICIAN.

INGESTION: DRINX LARGE QUANTITIES OF MILK OR WATER,

00 NOT INDUCE VOMITING..  GO IMMEDIATELY 'TO EMERGENCY Of HOSPITAL.

CHEMICALS IN THIS PRODUCT ARE HOT CURRENTLY "LISTED AS CARCINOGENS,

ull    .J|J	j-  J -M   ||—	L     „, „,   _,„  -f	      	 ju-     a   a    .m.   mM ____	»•-.-«»—,™.-w, ..lt   „._-     -        -.       __.        ,   -_1—_   -_u_   '  L   .^ —^  ,i       n -i   -n-     L    _L   -j,- mrr ji-jmi -m  n—    L_t Tnr-.—tt •_;. ]i       r

.SECTION. 6 - REACTIVITY DATA

\

STABILITY: STABLE

HAZARDOUS POLY«£RIZATIOM: WILL NOT OCCUR

INCOMPATIBILITY: HIGH TEMPERATURE, ACIDS

SECTION  7 - SPILL OR LEAK PROCEDURES

SPILL OR LEAK PRECAUTIONS:

SMALL .SPILLS:   FLUSH INTO DRAIN WITH LARGE QUANTITIES OF

WATER*

LARGE SPILLS:   USE ABSORBENT MATERIAL TO SOAK UP MATERIAL.

DISPOSE OF HASTE AT APPROVED LANDFILL IN ACCORDANCE WITH STATE REGULATIONS.

WASTE DISPOSAL:   FOLLOW STATE AMD LOCAL REGULATIONS FOR DISPOSAL

OF LARGE- NOf^-RECLAI«A8Le SPILLS.

section 8 - special protection_

protective gloves: use when handliws concentrate

eye protection: use safety glasses or gogsles in handling

concentrate. other protective equipments safety aprojm is «,ecom«emoeo

handling concentrate,

section 9 - precautions: handling, storage and usage

precautions to be taken in handling aftio storage: avoid

temperature extremes in storage? store away froh acids? keep container lid sealed    not in use,

other precautions: if concentrate is spilled on clothes, wash -thoroughly before reuse,

section 10 - regulatory controls

department of transportation:

dot classification: corrosive

oot proper shipping name: compound cleaning liquid

other dot information: ha 1760