(O61922-O342<§> -UO125706-5942     ) j DATE OF ISSUE	! SUPERSEDES	|

I    O8/O2/94	|   _O4/°2/54_	'

SECTION I - GENERAL INFORMATION

CHEMICAL NAME & SYNONYMS	TRADE NAME & SYNONYMS

N/A _	SUPER REAM II/SAFETY KIT

CHEMICAL FAMILY	FORMULA

SOLVENT/ACID DRAIN OPENER	X<--MIXTURE

MANUFACTURERS  NAME:

CERTIFIED LABS, DIV. OF NCH CORP.

ADDRESS (NUMBER, STREET, CITY, STATE & ZIP CODE)

BOX 15217O

IRVING, TEXAS 75O15

PREPARED BY:	| PRODUCT CODE NUMBER ! EMERGENCY TELEPHONE NUMBER

B SANFORD/CHEMIST	5942	214-438-1381

SECTION II- HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS.

CHEMICAL NAME (INGREDIENTS) : SULFURIC ACID

HAZARD	>CORROSIVE    TLV	>1MG/M3 1.     PEL-->1MG/M3 2.

__STEL(TWA)* ->NOT EST.    ] CAS#-->7664-93-9   |	|

CHEMICAL NAME (INGREDIENTS) : ALIPHATIC PETROLEUM DISTILLATE

HAZARD	>COMB.        ! TLV	>NOT EST.1     PEL-->NOT EST. 2

STEL(TWA)* ->NOT EST.    | CAS#-->64742-47-8 |	|

CHEMICAL NAME (INGREDIENTS) : 1 ,2 DICHLOROBENZENE

HAZARD	>COMB .         TLV	>5OPPM C.1.    PEL-->5OPPMM C.2

STEL(TWA)* ->NOT EST.    | CAS#-->95-5O-1     [	!

CHEMICAL NAME (INGREDIENTS) : 1,4 DICHLOROBENZENE

HAZARD	>COMB/CARC.  ! TLV	>75PPM 1.      PEL-->75PPM 2.    |

STEL(TWA)* ->11OPPM        CAS#-->1O6-46-7

(CONTINUED)

SUPER REAM II/SAFETY KIT SECTION II- HAZARDOUS INGREDIENTS

O2

SECTION IIA - NON-HAZARDOUS INGREDIENTS

NON-HAZARDOUS INGREDIENT NAMES AND CAS NUMBERS ARE PROTECTED UNDER Nd TRADE SECRET REGISTRY #   4O9363-5O75P

SECTION III - PHYSICAL DATA

BOILING PT.(FAHRENHEIT)

VAPOR PRESSURE (MM HG). VAPOR DENSITY (AIR=1)

32O.OO

> 1

SPEC GRAVITY (H20=1 )

: 1 .68

color  jwater white/blue grn

| SOLVENT/SULFUR

odor

10O%

1O% < 1

15

PERCENT, VOLATILE BY VOLUME (%)

SOLUBILITY IN WATER VISCOSITY

CLARITY |CLEAR/TRANSLUCENT

r

EVAPORATION RATE (BU AC = 1 )

1

APPRECIABLE NON-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

FLASH POINT (METHOD USED) 12OF.    T.C.C.

FLAMMABLE LIMITS

1%

6%

EXTINGUISHING MEDIA <--FOAM

"ALCOHOL1 <--FOAM

X<--C02

DRY X<--CHEMICAL

WATER <--SPRAY

<--OTHER

SPECIAL FIRE FIGHTING PROCEDURES

USE NIOSH/MSHA APPROVED SELF-CONTAINED BREATHING APPARATUS WHERE THIS MATERIAL IS INVOLVED IN A FIRE.  WATER APPLIED DIRECTLY TO ACID CAN CAUSE SPATTERING AND EVOLUTION OF HEAT.

UNUSUAL FIRE & EXPLOSION HAZARDS

HIGHLY REACTIVE AND CAPABLE OF IGNITING FINELY DIVIDED COMBUSTIBLE MATERIALS AND CAN FORM OXIDES OF CARBON AND SULFUR UPON COMBUSTION.

NFPA HAZARD RATING ( O= INS IGNIF ICA.JT ; 1 =SL IGHT ; 2 = MODERATE ; 3=HIGH ; 4 = EXTREME ) : 3 <--HEALTH   2 <--FLAMMABILITY    1 <--REACTIVITY      <--SPECIAL

THRESHOLD LIMIT VALUE :

NOT ESTABLISHED FOR MIXTURE.   SEE ScCTiC.* II-.

EFFECTS OF OVEREXPOSURE

-	ACUTE -    (SHORT TERM EXPOSURE)
~~<>CONTAINS CONCENTRATED SULFURIC ACID. *POISON* CAUSES SEVERE BURNS.-"

OINHALATION: CAUSES SEVERE IRRITATION AND POSSIBLE BURNS TO NOSE, AND

RESPIRTORY TRACT. OEYES: CAUSES BURNS .<> -SKIN: CAUSES BURNS.-"INGESTION: CAUSES BURNS TO MOUTH, THROAT AND INTESTINAL TRACT.  COULD

CAUSE SEVERE INJURY OR DEATH."

-	CHRONIC -   (LONG TERM EXPOSURE)

CONTINUOUS INHALATION OF MIST OR SOLVENT VAPORS MAY RESULT IN DAMAGE TO LIVER AND KIDNEYS.   OTHER SKIN, EYES, AND INGESTION EFFECTS ARE THOSE SECONDARY TO TISSUE DESTRUCTION.

S»TARGET ORGANS: EYES, SKIN, RESPIRATORY TRACT, LIVER, KIDNEYS.®"

PRIMARY ROUTE OF ENTRY: X<-- INHALATION  <-- INGESTION   <-- ABSORPTION^

EMERGENCY & FIRST AID PROCEDURES

INHALATION :

REMOVE FROM AREA TO FRESH AIR.   IF NOT BREATHING, CLEAR AIRWAY AND START MOUTH-TO-MOUTH ARTIFICAL RESPIRATION.  GET IMMEDIATE MEDICAL ATTENTION.

EYE CONTACT:

IMMEDIATELY RINSE EYES WITH WATER.   REMOVE ANY CONTACT LENSES.  HOLD EYELIDS APART TO ENSURE RINSING OF THE ENTIRE SURFACE OF THE EYES AND LIDS WITH WATER.  CONTINE FLUSHING THE EYES WITH WATER FOR 15 MINUTES.  GET IMMEDIATE MEDICAL ATTENTION.

SKIN CONTACT:

WASH AFFECTED AREAS WITH LARGE AMOUNTS OF SOAP AND WATER FOR 15 MINUTES. REMOVE CONTAMINATED CLOTHING AND SHOES.  GET IMMEDIATE MEDICAL ATTENTION. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE.

INGESTION :

GIVE 3-4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING.   IF VOMITING OCCURS, GIVE FLUIDS AGAIN.  GET IMMEDIATE MEDICAL ATTENTION.   DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.   GET IMMEDIATE MEDICAL ATTENTION.

NOTES TO PHYSICIAN : N/A

SECTION VI - TOXICITY INFORMATION

PRODUCT CONTAINS CHEMICAL LISTED AS CARCINOGEN OR POTENTIAL CARCINOGEN BY:

IARC  X<--YES  NTP  X<--YES  OSHA    <--YES  ACGIH   <--YES  OTHER   <--YES
<--NO	<--NO	X<--NO	X<--NO	X<--NO

SUPER REAM II/SAFETY KIT
(CONTINUED)	SECTION VI - TOXICITY INFORMATION	PAGE : O4

SULFURIC ACID	1 , 2-D I CHLOROBENZENE

EYE-RBT: 1 . 3MG SEV  4.	IHL-RAT LCLO : 821PPM/7H 3.

UNK-MAN: 135MG/KG  4.	EYE-RBT:  1OOMG/3OS RNS MILD 3.

ORL-RAT LD5O: 214OMG/KG  4.	ORL-RAT LD5O: 5OOMG/KG  3.

INH-GPG LC5O: 8MG/M3  4.	IHL-GPG LCLO: 8OOPPM/24H  3.

ALIPHATIC PETROLEUM DISTILLATE	1 , 4-D ICHLOROBENZENE

UNK-MAN LDLO: 1176MG/KG  2.	EYE-HMN 8OPPM  3.

ORL-RAT LD5O: 28G/KG/ 2.	ORL-HMN TDLO : 3OOMG/KG:SYS 3.

ORL-RBT LD5O: 28G/KG 2.	ORL-RAT LD5O: 5OOMG/KG  3.

MMO-ASN: 2OOMG/L   3.

1 ,4-DICHLOROBENZENE HAS BEEN SHOWN TO CAUSE CANCER IN LAB ANIMALS, BUT THERE IS INADEQUATE EVIDENCE THAT IT MAY CAUSE CANCER IN HUMANS.

DETECTALBE AMOUNTS OF "CHEMICALS KNOWN TO THE STATE OF CALIFORNIA TO CAUSE BIRTH DEFECTS OR OTHER REPRODUCTIVE HARM" MAY BE PRESENT.

SECTION VII - REACTIVITY DATA

|X<--STABLE     <--UNSTABL !   CONDITIONS TO AVOID STAB ILITY      ------------------------- 1 -------------------------------------

AVOID ABNORMAL TEMPERATURES.

INCOMPATIBILITY (MATERIALS TO AVOID)

ORGANIC CHLORATES, CARBIDES, FULMINATES, PICRATES AMD STRONG

ALKLAIS.

HAZARDOUS DECOMPOSITION PRODUCTS

OXIDES OF SULFUR AND CARBON MAY BE FORMED UPON COMBUSTION.

WILL NOT         MAY   ! CONDITIONS TO AVOID

HAZARDOUS      X <--OCCUR       "°CCUR

polymerization

in/a

SECTION VIII - SPILL OR LEAK PROCEDURES

STEPS TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

WEAR PROTECTIVE CLOTHING. REMOVE SOURCES OF FLAMES, SPARK OR HEAT. PROVIDE ADEQUATE VENTILATION. ABSORB SPILLS WITH SODA ASH OR OTHER INERT MATERIAL, THEN PLACE IN A LABELED CONTAINER FOR PROPER DISPOSAL.   RINSE AREA THOROUGHLY WITH WATER TO REMOVEDANY_RESIDUAL MATERIAL.

WASTE DISPOSAL METHOD

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS.

NEUTRALIZING AGENT

ADD SLOWLY TO A SOLUTION OF SODA ASH AND SLAKED LIME WITH STIRRING.

SECTION IX - SPECIAL PROTECTION INFORMATIOPAGE : OS

REQUIRED VENTILATION

GOOD INDUSTRIAL HYGIENE PRACTICE DICTATES THAT THE WORK AREA SHOULD BE ISOLATED AND CONTAINED AND PROViD=D WITH 'ADEQUATE LOCAL EXHAUST VENTILATION OR OTHER CONTROLS TO MAINTAIN THE AIR CONCENTRATION OF SULFURIC ACID BELOW 1MG/M3 AS REQUIRED BY OSHA.

RESPIRATORY PROTECTION

NIOSH HAS CONCLUDED THAT ADHERENCE TO THE PRESENT FEDERAL STANDARD OF 1MG/M3 OF AIR,  IN CONJUNCTION WITH A STRONG PROGRAM OF WORK PRACTICES, WILL PROTECT THE WORKER FROM SULFURIC ACID EXPOSURE FOR UP TO 1O HOUR WORKDAY, 4O HOUR WORK WEEK OVER A WORKING LIFETIME.  THE IDLH LEVEL IS 8O MG/M3.   RECOMMENDED RESPIRATOR SELECTION.   FOR EXPOSURES:

- = 5OMG/M3 - GMAGHIEP/HIEPF/SAF/SCBAF

- = 1OOMG/M3 - SAF:PD,PP,CF
ESCAPE: GMAGHIEP/SCBA

PROTECTIVE GLOVES NEOPRENE RUBBER GLOVES.

EYE PROTECTION

GOGGLES OR FACE SHIELD.

OTHER PROTECTION

WEAR PROVIDED SAFETY KIT GEAR AT ALL TIMES WHEN

HANDLING AND USING THIS PRODUCT.

SECTION X - STORAGE AND HANDLING INFORMATION

STORAGE TEMPERATURE	!  INDOOR     !HEATED     [REFRIGERATED! OUTDOOR

1OO F.<--MAX   32 F.<--MIN   | X	|	|	|

PRECAUTIONS TO BE TAKEN IN HANDLING & STORING STORE IN ORIGINAL CONTAINER.   DO NOT GET IN EYES, ON SKIN OR ON CLOTHING.   DO NOT TAKE INTERNALLY. AVOID BREATHING MIST OR FUMES.   PROTECT AGAINST PHYSICAL DAMAGE AND WATER. *POISON* MAYBE FATAL OR CAUSE PERMANENT DAMAGE IF SWALLOWED. STORE  IN A COOL, DRY AREA.

OTHER PRECAUTIONS

READ THE ENTIRE LABEL BEFORE USING THIS PRODUCT.

KEEP OUT OF REACH OF CHILDREN.

NOTE:  WATER ON SULFURIC ACID WILL CAUSE

SPATTERING, FUMING AND EVOLUTION.

SECTION XI - REGULATORY INFORMATION

SUPER REAM II/SAFETY KIT
(CONTINUED)	SECTION XI - REGULATORY INFORMATION	PAGE : O6

| CHEMICAL NAME | C.A.S. NUMBER | UPPER % LIMIT |
| --- | --- | --- |
| SULFURIC ACID | 7664-93-9 | UE |
| 1 , 2-DICHLOROBENZENE | 95-5O- 1 | UE |
| 1 , 4-DICHLOROBENZENE | 1O6-46-7 | UE |

THOSE INGREDIENTS LISTED ABOVE ARE SUBJECT TO THE REPORTING REQUIREMENTS OF 313 OF TITLE III OF THE SUPERFUND AMENDMENTS AND REAUTHORIZATION ACT OF 1986 AND 4O CFR PART 372.

IF UE (USE EXEMPTION) APPEARS UNDER UPPER % LIMIT, END USERS ARE EXEMPT FROM NOTIFICATION BECAUSE THE PRODUCT IS USED AND LABELED FDR ROUTINE JANITORIAL WORK, OR THE PRODUCT IS USED AND LABELED FOR FACILITY GROUNDS MAINTENANCE (SUCH AS FERTILIZERS AND HERBICIDES), OR THE PRODUCT IS USED AND LABELED FOR MAINTAINING MOTOR VEHICLES.

CALIFORNIA PROPOSITION 65

WARNING: THIS PRODUCT CONTAINS THE FOLLOWING CHEMICAL(S) KNOWN TO THE STATE OF CALIFORNIA TO CAUSE (1) CANCER OR (2) BIRTH DEFECTS OR OTHER REPRODUCTIVE HARM: NONE

SECTION XII - TRANSPORTATION * (FOR FUTURE USE)

APPLICABLE REGULATIONS

<--49 CFR _  <--IMCO      <--TARIFF 6 D     <--IATA   <--MILITARY AIR (AFR 71-4

SHIPPING NAME

HAZARD CLASS	|  ID NUMBER | REPORT QTY

LABELS	| LIMITED QTY

UNIT CONTAINER

DOT  SPS CONTAINER	| NET EXPLOSIVE WT .

AEROSOL PROPELLANT(S)

SECTION XIII - REFERENCES

THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS

*SHORT TERM EXPOSURE LIMIT (TWA) LISTED AS FINAL RULE LIMITS PUBLISHED I FEDERAL REGISTER/VOL.54 NO.12,  1-19-89

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION, HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

CERTIFIED LABS, DIV. OF NCH CORP.         ASSUMES NO RESPONSIBILITY FOR PERSONAL INJURY OR PROPERTY DAMAGE CAUSED BY THE USE , STORAGE OR DISPOSAL OF THE PRODUCT IN A MANNER NOT RECOMMENDED ON THE PRODUCT LABEL. USERS ASSUME ALL RISKS ASSOCIATED WITH SUCH UNRECOMMENDED USE, STORAGE, OR DISPOSAL OF THE PRODUCT.