MATERIAL SAFETY DATA SHEET

EQUILON MSDS:   26412ET     01/04/99

URSA SUPER PLUS MOTOR OILS

TELEPHONE NUMBER:

24 HOUR EMERGENCY ASSISTANCE

EQUIVA SERVICES: 877-276-7283 CHEMTREC: 800-424-9300

NAME AND ADDRESS:

EQUILON ENTERPRISES LLC PRODUCT STEWARDSHIP P.O. BOX 674414 HOUSTON, TX  77267-4414

GENERAL MSDS ASSISTANCE 877-276-7285

LEGEND:

N.D.

- NOT DETERMINED

- LESS THAN

N.A. - NOT APPLICABLE >    - GREATER THAN

N.T- NOT TESTED

1.NAME

MATERIAL IDENTITY Product Code and Name:

26412 URSA SUPER PLUS MOTOR OILS Chemical Name and/or Family or Description:

Ursa Oils

This Material Safety Data Sheet may be used for the following products for Hazard Communications purposes only; .not intended to imply identical

oerformance/technical srscifications:

• s

- URSA SUPER PLUS	SAE	15W-40

- URSA SUPER PLUS	SAE	10W

- URSA SUPER PLUS	SAE	20-20W

- URSA SUPER PLUS	SAE	30

- URSA SUPER PLUS	SAE	40

- URSA SUPER PLUS	SAE	50
02120 URSA SUPER PLUS	SAE	10W-30

2. COMPOSITION/INFORMATION ON INGREDIENTS

THE CRITERIA FOR LISTING COMPONENTS IN THE COMPOSITION SECTION IS AS FOLLOWS: CARCINOGENS ARE LISTED WHEN PRESENT AT 0.1 % OR GREATER; COMPONENTS WHICH ARE OTHERWISE HAZARDOUS ACCORDING TO OSHA ARE LISTED WHEN PRESENT AT 1.0 % OR GREATER; NON-HAZARDOUS COMPONENTS ARE LISTED AT 3.0 % OR GREATER. THIS IS NOT INTENDED TO 3E A COMPLETE COMPOSITIONAL DISCLOSURE.  REFER TO SECTION 14 FOR APPLICABLE STATES' RIGHT TO KNOW AND OTHER REGULATORY INFORMATION.

Product and/or Component(s) Carcinogenic According to: OSHA    IARC    NTP-   OTHER    NONE

X

Composition: (Sequence Number and Chemical Name)

Seq. Chemical Name	.	CAS.Number      Range in %

01	# Solvent-dewaxed heavy paraffinic petroleum       64742-65-0 65.00-79.99

distillates '''.

02	# Hydrotreated heavy paraffinic petroleum        . 64742-54-7 10.00-19.99

distillates

03	# Solvent-refined heavy paraffinic petroleum       64741-88-4  3.00-9.99

distillates

04	# Solvent refined light paraffinic petroleum       64741-89-5  3.00-9.99

distillates  or

05	Hydrotreated middle distillate	64742-46-7  3.00-9.99

06	$ Modified polyolefin amide alkyleneamine	3.00-9.99

dispersant

07	* Zinc allcyl dithiophosphate	68649-42-3  1.00-2.99

PRODUCT IS NON-HAZARDOUS ACCORDING TO OSHA (1910.1200).

* COMPONENT IS HAZARDOUS ACCORDING TO OSHA.

& COMPONENT, BY DEFINITION, IS CONSIDERED HAZARDOUS ACCORDING TO OSHA

BECAUSE IT CARRIES THE PERMISSIBLE EXPOSURE LIMIT (PEL) FOR MINERAL OIL

MIST.

Exposure Limits referenced by Sequence Number in the Composition Section Sea. Limit

| 0 1 | 3 | mg/m3 | TWA-CSHA (MINERAL OIL MIST) |
| --- | --- | --- | --- |
| 01 | 5 | mg/m3 | TWA-ACGIH (MINERAL OIL MIST) |
| 01 | 10 | mg/m3 | STEL ACGIH (MINERAL OIL MIST) |
| 02 | 5 | mg/m3 | TWA-OSHA (MINERAL OIL MIST) |
| 02 | 5 | mg/m3 | TWA-ACGIH (MINERAL OIL MIST) |
| 02 | 10 | mg/m3 | STEL ACGIH (MINERAL OIL MIST) |
| 03 | 5 | mg/m3 | TWA-OSHA (MINERAL OIL MIST) * |
| 03 | 5 | mg/m3 | TWA-ACGIH (MINERAL OIL MIST) ' |
| 03 | 10 | mg/m3 | STEL ACGIH (MINERAL OIL MIST) |
| 04 | 5 | mg/m3 | TWA-OSHA (MINERAL OIL MIST) |
| 04 | 5 | mg/m3 | TWA-ACGIH (MINERAL OIL MIST) |
| 04 | 10 | mg/m3 | STEL ACGIH (MINERAL OIL MIST) |
| 3. | HAZARD | IDENTIF | I CAT ION |

EMERGENCY OVERVIEW Appearance:

Bright and clear liquid Odor:

Not determined

WARMING STATEMENT

WARNING !        USED GASOLINE MOTOR OIL IS A POSSIBLE SKIM CANCER HAZARD BASED ON ANIMAL DATA

KMIS	MFPA

Health:       1    Reactivity: 0      Health:       1    Reactivity: 0 Fiammability: 1    Special   : -      Flammability: 1    Special   : -

POTENTIAL HEALTH EFFECTS

EFFECTS CF O'/EREXPOSURE Acute:

o 9 nf 10

Eyes: May cause minimal irritation, experienced as temporary discomfort.

"

Skin:	.  '•

^-,       .Brief contact may cause slight irritation.  Prolonged contact, as with

clothing wetted with material, may cause more severe irritation and discomfort, seen as''local redness and swelling.

Other than the potential skin irritation effects noted above, acute (sho term) adverse effects are not expected from brief skin contact; see othe effects, below, and Section 11 for information regarding potential long term effects.

t;'        Inhalation:

Vapors or mist, in excess of permissible concentrations, or in unusually high concentrations generated from spraying, heating the material or as from exposure in poorly ventilated areas or confined spaces, may cause irritation of the nose and throat, headache, nausea, and drowsiness.

Ingestion:

If more than several mouthfuls are swallowed, abdominal discomfort, naus and diarrhea may occur.

Sensitization Properties: Unknown.

Chronic:

No adverse effects have been documented in humans as a result of chronic exposure.  Section 11 may contain applicable animal data.

Medical Conditions Aggravated by Exposure:

C        Because of its irritating properties, repeated skin contact may aggravat an existing dermatitis (skin condition). s~~

Other Remarks:

When overheated, product may release hydrogen sulfide (H2S) gas. H2S concentrations above permissible concentrations can cause irritation of eyes and respiratory tract, headache, dizziness, nausea, vomiting, diarrhea and pulmonary edema. At concentrations above 300 pprn, respirato oa r a i vs i s, c a us — ng unconsciousness ano ceatn, can occur.

4. FIRST AID MEASURES    7~ '  " " ~ '"  ~ ~ 	~ "" "" ~ ~ '" "	"

Eyes :

Flush eyes with plenty of water for several minutes.  Get medical attention if eye irritation persists.

Skin:

Wash skin with plenty of soap and water for several minutes.  Get medical attention if skin irritation develops or persists.

Ingestion:

If more than several mouthfuls of this material are swallowed, give two glasses of water (16 oz.).  Get medical attention.

Inhalation:

If irritation, headache, nausea, or drowsiness	occurs, remove to fresh air

Get medical attent-cr. __ kr=£.thifi9 becomes -	-'   .-*.:	,

irritation persists.

V      Other Instructions:

/r~~-     Remove and cry-clean.or launder clothing soaked or soiled with this material before reuse.  Dry cleaning of contaminated clothing may be more effective than normal laundering.  Inform individuals responsible for

cleaning of potential hazards associated with handling contaminated

clothing.	^,

5. FIRE-FIGHTING MEASURES~T!"

Ignition Temperature - AIT(degrees F):~""

Not determined. '. Flash Point (degrees F) :

> 400 (COG) Flammable -Limits (%) :

Lower:  Not determined.

Upper:  Not determined.

Recommended Fire Extinguishing Agents And Special Procedures:

Use water spray, dry chemical, foam, or carbon dioxide to extinguish flames.  Use water spray to cool fire-exposed containers.  Water or foam may cause frothing.

Unusual or Explosive Hazards:

Hydrogen sulfide (H2S) may be released if overheated.

Extinguishing Media Which Must Not Be Used: Not determined.

Special Protective Equipment for Firefighters:

Wear full protective clothing and positive pressure breathing apparatus.

6. ACCIDENTAL RELEASE MEASURES (Transportation Spills:CKEMTREC (300)424-9300)

Procedures in Case of Accidental Release, Breakage or Leakage:

Ventilate area.  Avoid breathing vapor.  Wear appropriate personal	~

protective equipment, including appropriate respiratory protection. Contain      j
spill if possible.  Wipe up or absorb on suitable material and shovel up.
Prevent entry into sewers and waterways.  Avoid contact with skin, eyes or  >
clothing.	'

If more than       840,265 pounds of product is spilled, then report spill according to SARA. 304 and/or CERCLA 102(a) requirements, unless product qualifies for the pecroieum exemption (CZRCLA Section 101(14)).

~. HANDLING AND STORAGE      ~  '  ~ ~

1

Precautions   to  be   Taken Tn Handling: Minimum   feasible   handling   temperatures   should  be  maintained.

Storage:

Periods of exposure to high temperatures should be minimized. Water contamination should be avoided.

3. EXPOSuRE CONTROLS/PERSONAL PROTECTION

Protective Equipment(Type) Eye/Face Protection:

Safety glasses, chemical type goggles, or face shield recommended to prevent eye contact.

Skin Protection:
'-:-•-?'•' ='~ ^'r^::\~      -•• -----	•'.--- rl^'lv with soap and wat^r.

Respiratory Protection:

Airborne concentrations should be keot to lowest levels possible.  If vapor, mist or dust is generated and the occuoational exoosure limit of the

r a ^ s -r u f i o

product, or any component of the product, is exceeded, use appropriate NIOSH or MSHA approved air purifying or air supplied respirator after •         determining the airborne concentration of the contaminant. Air supplied respirators should always be worn when airborne concentration of the

contaminant or oxygen content is unknown.

*

Ventilation:	''

Adequate  to meet   component   occupational  exposure   limits    (see   Section  2).

/

Exposure'Limit for Total Product:

None established for product; refer to Section 2 for component exposure limits.

9. PHYSICAL AND CHEMICAL PROPERTIES——

Appearance:

Bright and clear liquid Odor:

Not determined

Boiling Point (degrees F): Not determined.

Melting/Freezing point (degrees F) : Not applicable.

Specific Gravity (water=l): .88 - .91

pH of undiluted product: Not applicable.

i

Vapor Pressure: Not determined.

Viscosity: 45.1- 215 cSt at 40.0 C

VOC Content: Not determined.

V

Vapor Density (air=l):  j Not determined.

Solubility in Water (I): Not determined.

Other: None 10. STABILITY AND REACTIVITY

This Material Reacts Violently With:

(If Others is checked below, see comments for details) Air  Water   Heat   Strong Oxidizers   Others   None of These

X

Comments:

Under extreme temperatures or extended storage periods, hydrogen sulfide (H2S) gas may accumulate in the head-space of container.

Products Evolved When Subjected to Keat or Combustion:

Toxic levels of ccrbon monoxide, carbon dioxide, irritating aldehydes and ketones, and combustion products or compounds of sulfur (may include hydrogen sulfide).

Page 5 o\ 10

'Hazardous Polymerizations: DO NOT OCCUR

11. TOXICOLGGICAL INFORMATION

TOXICOLOGICAL INFORMATION(ANIMAL TOXICITY DATA) Median Lethal Dose,. Oral:

LD50 Believed to be > 5.00 g/kg (rat) practically non-toxic Inhalation:

Not determined. Dermal:

LD50 Believed to be > 2.00 g/kg (rabbit) practically non-toxic Irritation Index, Estimation of Irritation (Species) Skin:

(Draize) Believed to be > .50 - 3.00 /8.0 (rabbit) slightly irritating Eyes:

(Draize) Believed to be < 15.00 7110 (rabbit) no appreciable effect Sensitization:

Not determined. Other:

Used gasoline motor oils have been shown to cause skin cancer when repeatedly applied to mouse skin without any effort to remove the material between applications. There is no evidence of a causal relationship between skin cancer in humans and exposure to used motor oil.

12.	DISPOSAL CONSIDERATIONS   '~' ~" ~~  "     "  ~ ~~ ~ "" ~ ~ ~ "

Waste Disposal Methods"      — - — — ~
This product has been evaluated for RCRA characteristics and does not meet
the criteria of a hazardous waste if discarded in its purchased form.
Under RCRA, it is the responsibility of the user of the product to deter
mine at the time of disposal, whether the product meets RCRA criteria for
hazardous waste.  This is because product uses, transformations, mixtures,   >
processes, etc. may render the resulting materials hazardous.	'

Remarks None

13.	TRANSPORT INFORMATION			"	 "  " "      ' ~ 	 ' "

Transportation         ,

DOT:"	1

Proper Shipping Name: Not regulated

IMDG:

Proper Shipping Name: Met regulated

ICAO:

Proper Shipping Name Not regulated

TDG:

14. REGULATORY INFORMATION Federal Regulations:

SARA. Title III:

Section 302/304 Extremely Hazardous Substances
Seq. Chemical Name	. CAS Number     Range in %

None

Section 302/304 Extremely Hazardous Substances.(CONT)

Seq. TPQ        RQ

None

c

1

Section 311 Hazardous Categorization:

Acute   Chronic   Fire   Pressure	Reactive   N/A

X

Section 313 Toxic Chemical

Chemical Name	CAS Number     Concentration

Zinc alkyl dithiophosphate          '	68649-42-3  1-. 00-2. 99

CERCLA 102 (a)/DOT Hazardous Substances:	(+ indicates DOT Hazardous Substance)

Seq. Chemical Name	CAS Number     Range in %

01+  Naphthalene	,       91-20-3	0.01-0.09

02+  Cumene	98-82-3	0.01-0.09

03+  Styrene	100-42-5	0.01-0.09

04+  Ethylbenzene	100-41-4	0.01-0.09

CERCLA/DOT Hazardous	Substances (Sequence Numbers and RQ's):
Seq. RQ

01+	100

02+	5000

03+	1000

04+	1000

TSCA Inventory Status:

This product, or its components, are listed on or are exemct from the Tc:<ic Substance Control Act (TSCA) Chemical Substance Inventory.

Other:
None.	,

\

State Regulations: California Proposition 65:

The following detectable components of this product are substances,
or belong to classes of substances, known to the State of California
to cause cancer and/or reproductive toxicity.
Chemical Name	CAS Number

None International Regulations:

WHMIS Classification: Not controlled

Canada Inventory Status: Not determined.

EINECS Inventory Status: Not determined.

Australia Inventory Status: Not determined.

Japan Inventory Status: Not determined.

15. ENVIRONMENTAL INFORMATION

Aquatic Toxicity: \ Not determined.

Mobility: . Not determined.

Persistence and Biodegradability: Not determined.

Potential to Bioaccumulate : Not determined.

Remarks : None

,16. OTHER INFORMATION

None

THE INFORMATION CONTAINED HEREIN IS BELIEVED TO BE ACCURATE.  IT IS PROVIDED INDEPENDENTLY OF ANY SALE OF THE PRODUCT FOR PURPOSE OF HAZARD COMMUNICATION AS PART OF THE PRODUCT SAFETY PROGRAM.  IT IS NOT INTENDED TO CONSTITUTE PERFORMANCE INFORMATION CONCERNING THE PRODUCT.  NO EXPRESS WARRANTY, OR IMPLIED WARRANTY OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE IS MADE WITH RESPECT TO THE PRODUCT OR THE INFORMATION CONTAINED HEREIN.  DATA SHEETS ARE AVAILABLE FOR ALL PRODUCTS.  YOU ARE URGED TO OBTAIN DATA SHEETS FOR ALL PRODUCTS YOU BUY, PROCESS, USE OR DISTRIBUTE AND YOU ARE ENCOURAGED AND REQUESTED TO ADVISE THOSE WHO MAY COME IN CONTACT WITH SUCH PRODUCTS OF THE INFORMATION CONTAINED HEREIN.

TO DETERMINE APPLICABILITY OR EFFECT OF ANY LAW OR REGULATION WITH RESPECT TO THE PRODUCT, USER SHOULD CONSULT HIS LEGAL ADVISOR OR THE APPROPRIATE ' GOVERNMENT AGENCY.  WE WILL MOT UNDERTAKE TO FURNISH ADVICE ON SUCH MATTERS .

Data: 1999-01-04         New   X   'Revised, Suoersedes : 1998-12-11

'

Inquiries regarding MSDS should be directed to: Equiva Services LLC

KAREN G. HAYNES, Manager Product Stewardship P.O. Box 674414 Houston ,TX 77267-4414

FT PRODUCT LA3EL       "~ " "   ~" " "  ~ '  "" "'    Lacei Date: 1999-01-04

READ AMD UNDERSTAND MATERIAL SAFETY DATA SHEET BEFORE HANDLING OR DISPOSING OF PRODUCT.  THIS LABEL COMPLIES WITH THE REQUIREMENTS OF THE OSHA HAZARD COMMUNICATION STANDARD {29 CFR 1910.1200) FOR USE IN THE WORKPLACE.  THIS LABEL IS NOT INTENDED TO BE USED WITH PACKAGING INTENDED FOR SALE TO CONSUMERS AMD MAY NOT CONFORM WITH THE REQUIREMENTS OF THE CONSUMER PRODUCT SAFETY ACT OR OTHER RELATED REGULATORY REQUIREMENTS.

25412 URSA SUPER PLUS MOTOR OILS

WARMING !        USED GASOLINE MOTOR OIL IS A POSSIBLE SKIN CANCER HAZARD n P r^ 10

BASED ON ANIMAL DATA

PRECAUTIONARY MEASURES

-Continuous contact with used gasoline motor oils has caused
skin cancer in laboratory animals.
-Avoid contact with used motor oil.	:

-Avoid prolonged breathing of vapor, mist, or gas. -Clean oil-soaked clothing before reuse. -Wash thoroughly after handling.

FIRST AID

Eye Contact:

Flush eyes with plenty of water for several minutes.  Get medical

attention if eye irritation persists. Skin Contact:

Wash skin with plenty of soap and water for several minutes.  Get medical

attention if skin irritation develops or persists. Ingestion:

If more than several mouthfuls of this material are swallowed, give two

glasses of water (16 oz.).  Get medical attention. Inhalation:

If irritation, headache, nausea, or drowsiness occurs, remove to fresh ai

Get medical attention if breathing becomes difficult or respiratory

irritation persists. Note to Physician:

None

In case of fire, use water spray, dry chemical, foam or carbon dioxide. Water may cause frothing. Use water spray to cool fire-exposed containers

If mere than       840,265 pounds of product is spilled, then report spil according to SARA 304 and/or CERCLA 102(a) requirements, unless product qualifies for the petroleum exemption (CERCLA Section 101(14)).

Chemical Name	CAS Number

=    Sclvent-dewaxed heavy paraffinic petroleum       64742-55-0 65.00-79.5

distillates	,

S    Kydrotreated heavy paraffinic petroleum	64742-54-7 10.00-19.9

distillates fr    Solvent-refined heavy paraffinic petroleum       64741-33-4  3.00-9.99

distillates =    Solvent refined light paraffinic petroleum       64741-39-5  3.00-9.99

distillates  or

Kydrotreated middle distillate	64742-46-7  3.00-9.99

S    Modified polyclefin amide aikyler.eamine	3.00-9.99

dispersant
Zinc aikyl dithicphcsphate	63643-42-3  1.00-2.99

PRODUCT IS NOM-KAZARDOUS ACCORDING TO OSKA (1910.1200).

- COMPONENT IS HAZARDOUS ACCORDING TO OSHA.

8 COMPONENT, BY DEFINITION, IS CONSIDERED HAZARDOUS ACCORDING TO OSKA

BECAUSE IT CARRIES THE PERMISSIBLE EXPOSURE LIMIT (PEL) FOR MINERAL OIL

MIST.

Pennsylvania Special Hazardous Substance(s)    CAS Number      Range i

None

HMIS	NF?A

Health:       1    Reactivity: 0      Health:        1    Reactivity: 0

Page 9 of 10

Flammability:   1    .      Special

Transportation DOT:

Proper  Shipping  Name: Not  regulated      ,

Flammability:   1

Special

CAUTION:- Misuse of empty containers can be hazardous.  Empty containers can be hazardous if used to store toxic, flammable, or reactive materials.  Cutting or welding of empty containers might cause fire, explosion or toxic fumes from residues.  Do not pressurize or expose to open flame or heat.  Keep container closed and drum bungs in place. Name and Address:

Equilon Enterprises LLC P.O. Box 674414 Houston ,TX 77267-4414

TRANSPORTATION EMERGENCY:      (877)276-7283 CHEMTREC:      (300)424-9300

HEALTH EMERGENCY:

(377)276-7283