FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

MSDS #:    468.00

Revision Date:   March 14, 2001

Section 1 — Chemical Product and Company Identification

Magnesium

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Magnesium

CAS#: 7439-95-4

Section 3 — Hazards Identification

Silvery-white, odorless, metal, turnings or ribbon.

Flammable solid.

Substance not considered hazardous. However, not all health aspects of this substance have been

thoroughly investigated.

flinn at-a-glance

Health-0

Flammability-2 React! vity-2 Exposure-0 Storage-2

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Flammable solid.	NFPA code

Water reactive metal; avoid contact with acids or water.	H-0

Fire Fighting Instructions: Use Class D, Met-L-X, or dry sand as a fire extinguisher. Avoid water	F-l

contact, violent reaction with water. Firefighters should wear PPE and SCBA with full facepiece	R-l
operating in positive mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Remove all ignition sources and water. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #1. Store with metals and metal hydrides. Store in a Flinn Saf-Stor can.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

) 2002 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Magnesium

MSDS #:    468.00

Revision Date:   March 14,2001

^

Section 9 — Physical and Chemical Properties

Silvery-white, odorless, metal, granular, turnings or ribbon. Solubility: Insoluble in water. Soluble in acids. Formula: Mg Formula Weight: 24.31

Specific Gravity: 1.74 Melting Point: 651 C

Section 10 — Stability and Reactivity

Avoid contact with acids, acid chlorides, strong oxidizers, halogens, chlorinated solvents. Shelf life: Indefinite, if stored safely.

Section 11 —Toxicoloaical Information

Acute effects: Irritating dust. Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Magnesium Hazard Class: 4.1, Flammable solid UN Number: UNI869

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231-104-6), RCRA code D001.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemicals—Your Best Choice

flinn@flinnsci.com  www.flinnsci.com

P.O. Box 219   Batavia IL 60510 (800) 452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2