23 199§

952432	'-^MATERIAL SAFETY DATA SHEET*	Page 1 of 2

MEDICAL EMERGENCY ONLY, 24 HOUR SERVICE:  1-800-328-0026

Ecolab Inc.	Product Information:  1-612-293-2233

St. Paul MN 55102	Date of Issue:  December 14, 1988

- IDENTIFICATION /

- Product Name:  DIP IT (Institutional)

- Product Type:  Powdered Alkaline Oxygen Destainer

-I-+ Section 2 Provides SARA Section 313 Reporting Information + +

2.0	HAZARDOUS COMPONENTS /	TLV (mg/ra3)

%   PEL Other

- Sodium metasilicate   6834-92-0	25  None   UNK

- Sodium perborate 7632-04-4	20  None    *

-'(Anhydrous sodium tetraborate = 1 mg/m3)

This product contains no other component considered hazardous according to the criteria of 29 CFR 1910.1200.

- PHYSICAL DATA~	'	   ~""

- Appearance and Odor:  White granular powder - no specific odor.

- Solubility in Water:  Complete

- pH:  1% = 11.0-12.0

470~FIRE AND EXPLOSION DATA  /	 ~	"~~

- Special Fire Hazards:  None

- Fire Fighting Methods:  Product does not support combustion.

- REACTIVITY DATA /        	  	"

- Stability:  Stable under normal conditions of handling.

- Conditions to Avoid:  Do not mix with anything but water.

- SPILL OR LEAK PROCEDUR"ES~7USE PROPER PROTECTIVE EQUIPMENT

- Cleanup:  Pick up without raising dust.  Flush area with water.

- Waste Disposal:  Consult state and local authorities for
restrictions on disposal of chemical waste.

UNK = Unknown at this time PEL = Permissible Exposure Limit TLV = Threshold Limit Value STEL = Short Term Exposure Level C = Ceiling Limit, Not To Be Exceeded

M30 Product:  DIP IT (Institutional)	Page 2 of 2 ^~

Ecolab Inc.	952432

MEDICAL EMERGENCY ONLY, 24 HOUR SERVICE:  1-800-328-0026

- HEALTH HAZARD DATA /	DANGER

- Effects of Overexposure:

Skin and Eyes:  CAUSES SEVERE CHEMICAL BURNS.  Eye contact may cause blindness.  Harmful contact may not cause immediate pain.

++ Immediate water flushing is vital in case of eye contact. ++ If Swallowed:  HARMFUL OR FATAL.  Causes chemical burns of mouth, throat and stomach.

If Inhaled;  Damages airways and lungs, depending upon amount and duration of exposure.  Effects can vary from slight irritation to bronchitis or pneumonia.  People with asthma or other lung problems may be more susceptible.

- FIRST AID /	~~~~~~  ""~       ________  —_

- Eyes:  Immediately flush with plenty of cool running water.  Remove
contact lenses.  Continue flushing for at least 15 minutes, holding
eyelids apart to ensure rinsing of the entire eye.  CALL A
PHYSICIAN IMMEDIATELY.

- Skin:  Immediately flush skin with plenty of cool running water for
at least 15 minutes while removing contaminated clothing and shoes.
Wash clothing before reuse.

- If Swallowed:  Rinse mouth at once; then drink 1 or two large
glasses of water or milk,  DO NOT induce vomiting.  NEVER give
anything "by mouth to an unconscious person.

- If Inhaled:  Immediately move to fresh air.

CALL A POISON CONTROL CENTER OR PHYSICIAN IMMEDIATELY

- SPECIAL PROTECTION INFORMATION""/  ~~      '"         ~~~~

- Respiratory:  Avoid breathing dusts or mists of this product.

- Skin:  Rubber gloves - protective cuff or gauntlet type preferred.

- Eyes:  Splashproof glasses, goggles or face shield.

- ADDITIONAL INFORMATION/PRECAUTIONS /~~

- DOT Class:  Not DOT Regulated.

- Store in a cool, dry place.

KEEP OUT OF REACH OF CHILDREN

The above information is believed to be correct with respect to the formula used to manufacture the product.  As data, standards and regulations change, and conditions of use and handling are beyond our control, NO WARRANTY, EXPRESS OR IMPLIED, IS MADE AS TO THE COMPLETENESS OR CONTINUING ACCURACY OF THIS INFORMATION.