«-

WHITE LlGHTHim

Material Safety Data	Sheet

Document Code: POLY-RoofRepair/WL	Date of Preparation

Version: 01	February 13, 2001

Section 1 - Product and Company Identification

PRODUCT NAME, NUMBERS, •& COLORS	HMIS CODES

Polyurethane Roof Repair	Health         2*

WL09118  Black	Flammability    2

Reactivity     0

MANUFACTURER'S NAME	EMERGENCY TF.I.KPHONF, NO.

THE SHERWIN-WILLIAMS COMPANY	(216) 566-2917

101 Prospect Avenue N.W.	INFORMATION TELEPHONE NO.

Cleveland, OH  44115	(216) 566-2902

Section 2 - Composition/Information on Ingredients

-

| % WT . CAS No . Ingredient Name |  |  |
| --- | --- | --- |
| 3 108-88-3 Toluene |  |  |
| ACGIH TLV . TWA ' | 50 . | ppm- (S.kin) |
| -...'. ...V.. - . : • • osha pel twa - ' | 100 | ppm (Skin). |
| •"'.'. ••-.'.- • •;• •'-' . '.v.'v ,••' ••'.'-. .••"' • ' .', OSHA • •. P.E'ij . - . STE'Lr . | .150 ;. | ,ppm .-(Skin) ... :. ."..'•;•:.'. '"•;. ...-/: .-.-:•. -:.--x-:: |
| . ;.;'; 20 '•••''•; '• :\:"471-3'4/:i'/.:-//'.v';CaiGium^Carb(onate | ':••!- '' f.V-.."J |  |
| ACGIH TLV TWA | 10 | mg/m3 as Dust |
| OSHA PEL TWA | 15 | mg/m3 as Total Dust |
| OSHA PEL TWA | 5 | mg/m3 as Respirable Fraction |
| <5 1333-86-4 Carbon Black |  |  |
| ACGIH TLV TWA | 3.5 | mg/m3 |
| OSHA PEL TWA | 3.5 | mg/ |

Section 3 - Hazards Identification

ROUTES OF EXPOSURE

Exposure may be by INHALATION and/or SKIN or EYE contact, depending on conditions of use. To minimize exposure, follow recommendations for proper use, ventilation, and personal protective equipment. EFFECTS OF OVEREXPOSURE

'••;. ' :,'' ' ..•Irritation of ..^yes,skin, and upper respirato.ry system.. .:'•-. In.- a confined area-; vapors in high concentration may cause headache, nausea or dizziness. SIGNS AND SYMPTOMS OF OVEREXPOSURE

Redness and itching or burning sensation may indicate eye or excessive skin exposure. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE

May cause allergic skin reaction or sensitization in susceptible persons. This effect may be delayed several hours after exposure. CANCER INFORMATION

FOR COMPLETE DISCUSSION OF TOXICOLOGY DATA REFER TO SECTION 11.

Section 4 - First Aid Measures

If INHALED: If any breathing problems occur during use, LEAVE THE AREA and get fresh air. If problems remain or occur later, IMMEDIATELY get medical attention.

If on SKIN:    Wash affected area thoroughly with soap and water.  Remove contaminated clothing and launder before re-use.

EYES:    Flush eyes with large amounts of water for 15 minutes.

Get medical attention. If SWALLOWED:  Do not induce vomiting.  Get medical attention immediately.

Section 5 - Fire Fighting Measures

FLASH POINT	LEL      UEL

148 °F	1.0      7.0

FLAMMABILITY CLASSIFICATION

Combustible, Flash above 99 and below 200 °F EXTINGUISHING MEDIA

Carbon Dioxide, Dry Chemical, Foam UNUSUAL FIRE AND EXPLOSION HAZARDS

Keep containers tightly closed.  Isolate from heat, electrical equipment, sparks, .and open ;.f;l.a.me .  Closed;, containers', may explode when exposed .to extreme heat.  Application to hot surfaces requires special precautions.  During emergency conditions overexposure to decomposition products may cause a health hazard. Symptoms may not be immediately apparent. Obtain medical attention. SPECIAL FIRE FIGHTING PROCEDURES

Full protective equipment including self-contained breathing apparatus should be used. Water spray may be ineffective.  If water is used, fog nozzles are preferable.  Water may be used to cool closed containers to prevent pressure build-up and possible autoignition or explosion when exposed to extreme heat.

Section 6 - Accidental Release Measures

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

Remove all sources of ignition.  Ventilate the area.  Cover spill with absorbent material.  Deactivate spilled material with a 10% ammonium hydroxide solution (household ammonia).  After 10 minutes, collect in open containers and add more ammonia.  Cover loosely.  Wash spill area with soap and water.

Section 7 - Handling and Storage

DOL STORAGE CATEGORY

2

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE

Contents are COMBUSTIBLE.  Keep away from heat and open flame.  Consult NFPA Code. Use approved Bonding and Grounding procedures.  Keep container closed when not in use.  Transfer only to approved containers with complete and appropriate labeling. Do not take internally.  Keep out of the reach of children.

Section 8 - Exposure Controls/Personal Protection

PRECAUTIONS TO BE TAKEN IN USE

NO PERSON SHOULD USE THIS PRODUCT, OR BE IN THE AREA WHERE THIS PRODUCT

ARE BEING USED, IF THEY HAVE EVER HAD A REACTION TO ISOCYANATES.

Use only with adequate ventilation.  Avoid breathing Vapor and spray mist.

Avoid contact with skin and eyes. .Wash hands after .using,

.•'.. —.'This .product m^^rvoonfeain,.TOat^bi^ls>-'claS;S;4.Eie<:i •as.VTOJi^saiicfe :pa:rtic;viiat;es\;(ii:stedi;: ' -"as'-•Dust"- in; Section 2)'•' which' may' ':bev.'pie'sent' at hazardous 'levels" only'•' during"'

sanding or abrading of the dried film.  If no specific dusts are listed

in Section 2, the applicable limits for nuisance dusts are ACGIH TLV 10 mg/m3

(total dust), 3 mg/m3 (respirable fraction), OSHA PEL 15 mg/m3 (total dust),

5 mg/m3 (respirable fraction) .

VENTILATION

Local exhaust preferable.  General exhaust acceptable if the exposure to

materials in Section 2 is maintained below applicable exposure limits.  Refer to

OSHA Standards 1910.94, 1910.107, 1910.108.

. RESPIRATORY PROTECTION

••'••'... '.'•-. '.'.   .Tf. personal ex|xps.ure cannot, be, controlled. below •'• applicable limits by   •.':;:.•:..; ventilation, wear a properly fitted organic vapor /particulate respirator approved by NIOSH/MSHA for protection against materials in Section 2.

When sanding or abrading the dried film, wear a dust/mist respirator approved by NIOSH/MSHA for dust which may be generated from this product, underlying paint, or the abrasive. PROTECTIVE GLOVES

To avoid contact with skin, wear chemical resistant, impervious gloves which are recommended by glove supplier for protection against materials in Section 2. EYE PROTECTION

Wear safety spectacles with unperf orated sldeshields.

Section 9 - Physical and Chemical Properties

PRODUCT WEIGHT	13.3 Ib/gal	EVAPORATION RATE	Slower than Ether

SPECIFIC GRAVITY	1.60	VAPOR DENSITY	Heavier than Air

BOILING RANGE	222-238 °F	MELTING POINT	N.Av.

VOLATILE VOLUME	<5 %	SOLUBILITY IN WATER	N.Av.

VOC	0.4 Ib   47 gm

Section 10 - Stability and Reactivity

STABILITY

Stable CONDITIONS TO AVOID

Avoid contact with amines, strong bases and active surface materials. INCOMPATIBILITY

rNone known. HAZARDOUS DECOMPOSITION PRODUCTS

By fire: Carbon Dioxide, Carbon Monoxide HAZARDOUS POLYMERIZATION Will not occur

Section 11 - lexicological Information

''Carbon Black is 'Classified by IARC: as possibly carcinogenic to humans (group 2B) based on experimental animal data, however, there is insufficient evidence in humans for its carcinogenicity .

Prolonged overexposure to solvent ingredients in Section 2 may cause adverse effects to the liver, urinary, cardiovascular, and reproductive systems.

Persons sensitive to isocyanates will experience increased allergic reaction on repeated exposure .

Reports have associated repeated and prolonged overexposure to solvents with Permanent brain and nervous system damage. TOXICOLOGY DATA

CAS No.       Ingredient Name

108-88-3      Toluene

:.-..;,.  .     . .. '•.  . .'..  .L;G50.   RAT. . . . . 4HR..	4000:   ppm      .;•> . v-V.-.:>.;:
••'•'"•.'.•':'. ;-V.        ::'^-D'50 '. : RAT' • ':: '.''•'  • • , ' : "636 :    mg/kg
471-34-1      Calcium Carbonate

LC50   RAT    4HR	Not Available

LD50   RAT	6450   mg/kg
1333-86-4      Carbon Black

LC50   RAT     4HR	Not Available

LD50   RAT	>15400 mg/kg

Section 12 - Ecological Information

ECOTOXICQLpGlCAL;IN;FORMATIQN;;    .' \: •'•.: •-. \-V;:'.: ;/::;{;'';•.: >•/ ,'-/_. ;;-:-:y:?;;:.;V: -!:•.• ":••:*. •'::•.• ±:~- -.' :'•'.:•':''•. '••.•. '•.'•.-.•• y': v-":-';-;V-'::': ?-'^:\''s2.i?,  :/'•'

Section 13 - Disposal Considerations

WASTE DISPOSAL METHOD

Waste from this product is not hazardous as defined under the Resource Conservation and Recovery Act (RCRA) 40 CFR 261.

Incinerate in approved facility.  Do not incinerate closed container.  Dispose of in accordance with Federal, State, and Local regulations regarding pollution.

Section 14 - Transport Information

DOT PROPER SHIPPING DESCRIPTION: Compounds, Caulking or Glaziers, NOI IATA/IMDG SHIPPING DESCRIPTION: Compounds, Caulking or Glaziers, NOI

Section 15 - Regulatory Information

SARA 313 (40 CFR 372. 65C) SUPPLIER NOTIFICATION

CAS No.     CHEMICAL/COMPOUND	% by WT   % Element

108-88-3      Toluene.	3

CALIFORNIA PROPOSITION 65

WARNING:  This product contains chemicals known to the State of California to cause cancer and birth defects or other reproductive harm. TSCA CERTIFICATION

All chemicals in this product are listed, or are exempt from listing, on the TSCA Inventory.

Section 16 - Other Information

CANADIAN DISTRIBUTOR:  Sherwin-Williams Canada

180 Brunei Rd. Mississauga, ON  L4Z ITS

NOTE:  This product has been classified in accordance with the hazard criteria of the CPR and the MSDS contains all of the information required by the CPR.

The above information pertains to this product as currently formulated, and is based on the information available at this time.  Addition of reducers or other additives to this product may substantially alter the composition and hazards of the product.  Since conditions of use are outside our control, we make no warranties, express or implied, and assume no liability in connection with any use of this information.