FLinn scientific inc.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS#:    71200

Revision Date:   November 25, 2002

FLinn

"Your Safer Source for Science Supplies Material Safety Data Sheet (MSDS)

Sodium Borate, Tetra

MSDS#:    712.00

Revision Date:   November 25, 2002

Section! — Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

Sodium Borate, Tetra

Flinn Scientific, Inc. P.O.Box219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

White, odorless crystals or powder.

Solubility: Soluble in water and glycerol; not alcohol.

Formula: Na2B4O7 1OH2O

Formula Weight: 381.42

Specific Gravity: 1.73

Melting Point: between 75 & 320 C

Section 2 —Composition. Information on Ingredients

Sodium Borate, Tetra

Synonym: Borax, sodium pyroborate

CAS#:  1303-96^

Section 10 — Stability and Reactivity

Shelf Life: Material efflorescent. Store in a Flinn Chem-Saf bag.

Section 3 — Hazards Identification

White, odorless crystals or powder.

Slightly toxic by ingestion, inhalation, and skin absorption. Irritating to body tissues. Avoid all body

tissue contact,

flinn at-a-glance Health-1

Flammability-Q React! vity-0 Exposure-1 Storage-1

0 is io\v hazard, 3 is high hazard

Section 11 —Toxjcological Information

Acute effects: Irritant Chronic effects: N.A. Target organs: Central nervous system, kidneys

N.A. = Not available, not all health aspects of this subsiar

Section 12 — Ecological Information

ORL-RAT LD50: 2660 mg/kg IHL-RATLC50:NA. SKN-RBT LD50: >2 gm/kg

c been fully investigated.

Section.4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. After vomiting, give mixture of 2 Tbs. of activated

charcoal mixed with one cup of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

NFPA code

Non flammable solid.

None Established

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A * Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (215-540-4),

Section 6 — Accidental Release Measures

Section 16 — Other Information

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site

after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Consult your copy of the Fltnn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

Flinn Suggested Chemical Storage Pattern: Inorganic #8. Store with borates, chromates, manganates and permanganates. Store in a cool dry place. Deliquescent, store in Flinn Chem-Saf Bag.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use ventilation to keep airborne concentrations below exposure limits Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 5 mg/m3 (Ba) (ACGIH)

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn MSDS Prevent Chemical Accidents

ninn@flmnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510 (800)452-1261  Fax (846) 452-1436