PAGE 1 OF 3

x^.

ZEP MANUFACTURING COMPANY FIRST IN MAINTENANCE PRODUCTS

MATERIAL SAFETY DATA SHEET

DISPOSAL INFORMATION

ZEP BRAKE SOL

O4/24/86

DATE       : SUPERSEDES:

05OO

PRODUCT NUMBER:

SECTION   I-  -EMERGENCY

CONTACTS

P. 0. BOX 2015 ATLANTA, GEORGIA 30301 TELEPHONE (404)352-1680 BETWEEN 8: OOA. M. -5: OOP. M. (EASTERN TIME ZONE)

ZEP MANUFACTURING COMPANY NON-OFFICE HOURS, WEEKENDS/ AND HOLIDAYS:AREA CODE 404

435-2973, 996-O899, 252-1587, 351-2952, 445-9226

LOCAL POISON CONTROL CENTER

TRANSPORTATION EMERGENCY

CHEMTREC: TOLL FREE 1-8OO-424-93OO ALL CALLS RECORDED DISTRICT OF COLUMBIA (202)483-7616 ALL CALLS RECORDED

SECTION   II-HAZARDOUS

INGREDIENTS

DESIGNATIONS

•** LIGHT ALIPHATIC NAPHTHA ** solvent naphtha

(petroleum), medium aliphatics; formerly: light

aromatic naphtha; CAS#64742-88-7; RTECS# NONE;

OSHA PEL SOOppm

#* 1,1,1-TRICHLOROETHANE ** methyl chloroform;

chlorothene; CAS# 71-55-6; RTECS* KJ2975OOO

OSHA PEL 350ppm

TLV EFFECTS % IN (PPM) (SEE REVERSE) PROD. 100 IRR CBL 30-40

350   IRR	60-70

SPECIAL NOTE: ADVERSE HEALTH EFFECTS WOULD NOT BE EXPECTED UNDER RECOMMENDED CONDITIONS OF USE SO LONG AS PRESCRIBED SAFETY PRECAUTIONS ARE PRACTICED.

SECTION   III

HEALTH

HAZARD

DATA

ACUTE EFFECTS OF OVEREXPOSURE:

INHALATION OF VAPOR CAN PRODUCE CENTRAL NERVOUS SYSTEM DEPRESSION CHARACTERIZED

DIZZINESS, HEADACHE, NAUSEA, CARDIAC AND/OR RESPIRATORY DEPRESSION, STUPOR, UNCONSCIOUSNESS AND DEATH, IN EXTREME CASES. EXPOSURE TO HIGH CONCENTRATIONS OF VAPOR BY DIRECT CONTACT OR INHALATION CAN BE IRRITATING TO MUCOUS MEMBRANES, SUCH AS EYES AND UPPER RESPIRATORY TRACT. SEVERE EYE EXPOSURE TO LIQUID CAN CAUSE REVERSIBLE EYE DAMAGE. SKIN CONTACT MAY CAUSE A BURNING SENSATION AND REDDENING OF THE SKIN.  INTRODUCTION OF SOLVENT TO THE LUNGS, AS IN ASPIRATION OF VOMITUS FLUIDS, MAY CAUSE CHEMICAL PNEUMONIA. EXPOSURE TO THIS PRODUCT MAY AGGRAVATE EXISTING RESPIRATORY AND CARDIAC CONDITIONS.

SECTION   III   -HEALTHHAZARDDATA (CONTINUED)

CHRONIC EFFECTS OF OVEREXPOSURE:

REPEATED OR PROLONGED CONTACT BY INHALATION OR SKIN ABSORPTION MAY PRODUCE LIVER OR KIDNEY DAMAGE OR DAMAGE TO THE CENTRAL NERVOUS SYSTEM (characterized by tingling or numbness in the extremities* blurred vision or confusion). SKIN WHICH IS DEFATTED BY REPEATED EXPOSURE TO SOLVENTS, IS MORE SUSEPTIBLE TO IRRITATION, INFECTION, AND DERMATITIS.

EXPOSURE TO SOME INGREDIENTS IN THIS PRODUCT CAN AGGRAVATE EXISTING LIVER DISEASE OR HEART RHYTHM DISORDERS. NONE OF THE HAZARDOUS INGREDIENTS ARE LISTED AS CARCINOGENS BY IARC, NTP, & OSHA

EST'D PEL/TLV: NOT ESTABLISHED     PRIMARY ROUTES OF ENTRY: INH, SKIN. HMIS CODES:  HEALTH 1;FLAM. 2; REACT.  1;PERS. PROTECT. X  ; CHRONIC HAZ. YES

FIRST AID PROCEDURES:

SKIN  : WASH CONTAMINATED SKIN THROUGHLY WITH SOAP OR A MILD DETERGENT. APPLY A SKIN CREAM WITH LANOLIN. GET MEDICAL ATTENTION IF IRRITATION PERSISTS.

EYES  : IMMEDIATELY FLUSH EYES WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES, 0^ CASIONALLY LIFTING UPPER AND LOWER LIDS. GET MEDICAL ATTENTION AT ONCE

INHALE: MOVE EXPOSED PERSON TO FRESH AIR AT ONCE.  IF BREATHING HAS STOPPED, PERFORM ARTIFICIAL RESPIRATION. GET MEDICAL ATTENTION IMMEDIATELY.

INGEST: IF SWALLOWED, DO NOT INDUCE VOMITING.   IF VOMITING OCCURS, KEEP HEAD BELOW HIP LEVEL.  GET EMERGENCY MEDICAL ATTENTION IMMEDIATELY.

SECTION   IV   -SPEC   IAL

PROTECTION

INFORMATION

PROTECTIVE CLOTHING

EYE PROTECTION

RESPIRATORY PROTECTION:

VENTILATION

WEAR VITON GLOVES OR USE GLOVES WITH DEMONSTRATED RESISTANCE TO THE INGREDIENTS IN THIS PRODUCT. USE TIGHT-FITTING, SPLASH-PROOF SAFETY GOGGLES. CONTACT LENSES SHOULD NOT BE WORN WHEN HANDLING THIS MATERIAL. IF VENTILATION IS INADEQUATE, WEAR A PROPERLY FITTING MSA OR OSHA-APPROVED RESPIRATOR.

VENTILATION SHOULD BE EQUIVALENT TO OUTDOORS. USE EXHAUST FANS AND OPEN WINDOWS IN ENCLOSED SPACES.

SECTION   V-PHYSICAL

DATA

BOILING POINT (F) VAPOR PRESSURE(MMHG) VAPOR DENSITY(AIR=1) SOLUBILITY IN WATER

APPEARANCE AND ODOR

RANGE 170-387F SPECIFIC GRAVITY
APP. SIMM      PERCENT VOLATILE BY VOLUME
N/D	EVAPORATION RATECN-BUOAC

= 1)

NEGLIGIBLE     PH(CONCENTRATE)	: N/A

PH(USE DILUTION OF N/A	): N/A

CLEAR, COLORLESS LIQUID WITH CHLORINATED SOLVENT ODOR

0. 93 10O7. 0. 2

SECTION   VI

FIRE

AND

EXPLOSION

DATA

(TCC    )

FLASH POINT(F) (METHOD USED): INITIAL 16OF

FLAMMABLE LIMITS     LEL 1. O7.   UEL 15.0%

EXTINGUISHING MEDIA  : DRY CHEMICAL, FOAM, C02.

SPECIAL FIRE FIGHTING: FIRE EXPOSED DRUMS SHOULD BE COOLED WITH STREAM OF WATER.

UNUSUAL FIRE HAZARDS : CONCENTRATED VAPOR MAY IGNITE IF EXPOSED TO SPARK.

SECTION   VII-REACTIVITY

DATA

STABILITY

INCOMPATIBILITY(AVOID) POLYMERIZATION HAZARDOUS DECOMPOSITION

STABLE

STRONG ALKALIES, OXIDIZERS, AND ACTIVE METALS.

WILL NOT OCCUR.

CARBON DIOXIDE, CARBON MONOXIDE/ HYDROGEN CHLORIDE,

SMALL AMOUNTS OF PHOSGENE & CHLORINE GAS.

AND

SECTION VIII -SPILL

AND

DISPOSAL

PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED:

IMMEDIATELY ELIMINATE ALL FLAME, IGNITION AND HIGH-HEAT SOURCES.  ABSORB SPILL ON INERT ABSORBENT MATERIAL (eg ZEP-O-ZORB).  PICK UP AND PLACE RESIDUE IN A CLEAN, D. 0. T. SPECIFICATION CONTAINER FOR DISPOSAL.  WASH AREA THOROUGHLY WITH A DETERGENT SOLUTION AND RINSE WELL WITH WATER.

WASTE DISPOSAL METHOD:

-LIQUIDS CANNOT BE SENT TO LANDFILLS UNLESS SOLIDIFIED.  UNUSABLE PRODUCT AND COLLECTED, SPENT MATERIAL MAY REQUIRE DISPOSAL AS A HAZARDOUS WASTE AT A PERMITTED TREATMENT/STORAGE/DISPOSAL FACILITY.   IN MOST STATES HAZARDOUS WASTE IN TOTAL AMOUNTS OF 220 LBS. OR LESS PER MONTH MAY BE DISPOSED OF IN A CHEMICAL OR IJilQUSTRIAL WASTE LANDFILL.  CONSULT LOCAL, STATE, AND FEDERAL AGENCIES FOR F   3ER DISPOSAL METHOD IN YOUR AREA.

RCRA HAZ. WASTE NOS. : D001, F002 (SEE ABOVE)

SECTION   IX   -SPECIAL        PRECAUTIONS

PRECAUTIONS TO BE TAKEN WHEN HANDLING AND STORING:

COMBUSTIBLE! STORE AND USE AWAY FROM HEAT, SPARKS, OPEN FLAME, OR ANY SOURCE OF

IGNITION.

CLOSE CONTAINER TIGHTLY WHEN NOT IN USE. STORE AWAY FROM SUN AND HEAT.

CLOTHING OR SHOES WHICH BECOME CONTAMINATED WITH SUBSTANCE SHOULD BE REMOVED

PROMPTLY AND NOT REWORN UNTIL THOROUGHLY CLEANED.

VAPORS ARE HEAVIER THAN AIR AND WILL ACCUMULATE AT LOW POINTS. VENTILATION

SHOULD INCLUDE FLOOR LEVEL EXHAUSTING.

KEEP PRODUCT AWAY FROM SKIN AND EYES.

DO NOT BREATHE SPRAY MISTS OR VAPORS.

KEEP OUT OF THE REACH OF CHILDREN.

SECTION   X-TRANSPORTATION        DATA DOT   PROPER   SHIPPING   NAME

dl. HAZARD CLASS: N/A

DOT I. D. NUMBER : N/A      DOT LABEL/PLACARD: NONE

EPA TSCA CHEMICAL INVENTORY - ALL INGREDIENTS ARE LISTED

EPA CWA 40CFR PART 117 SUBSTANCE(RQ IN A SINGLE CONTAINER): NONE