942771	-MATERIAL SAFETY DATA SHEET*	Page 1 of 2

MEDICAL EMERGENCY ONLY, 24 HOUR SERVICE:  1-800-328-0026

Ecolab Inc.	Product Information:  1-612-293-2233

St. Paul MN 55102	Date of Issue:  April 12, 1989

- IDENTIFICATION /

- Product Name:  LIME-A-WAY

- Product Type:  Aqueous Acidic'Cleaner/Delimer

++ Section 2 Provides SARA Section 313 Reporting Information ++

— .«•.••• — — — — — •.» — «.— — • — — .•—...•• — —— — — — • — •»••.— •»••»• — •• — *» — — — — — — — —•—•• — -•• — •• — — — .•-. — — —• —

2.0	HAZARDOUS COMPONENTS /	Air Limits (mg/m3)

%    PEL  Other

2.1	Phosphoric acid   7664-38-2	30    1     1

STEL = 3

This product contains no other component considered hazardous according to the criteria of 29 CFR 1910.1200.

- PHYSICAL DATA~7""

- Appearance and Odor:  Clear green liquid; mild organic odor

- Solubility in Water:  Complete

- PH:  100% = 1.0-1.5; 1.0% = 2.0-3.0

- Boiling Point:  212 deg F	Specific Gravity:  1.20-1.30

4.Q FIRE AND EXPLOSION DATA  /	~~"

- Special Fire Hazards:  None

- Fire Fighting Methods:  Product does not support combustion.

- REACTIVITY DATA~7~

- Stability:  Stable under normal conditions of handling.

- Conditions to Avoid:  Mix only with water.  Can react violently
with alkaline material or metals.  Do not mix with chlorinated
detergents or sanitizers - will cause hazardous vapors.

6.Q SPI-LL OR LEAK PROCEDURES /     USE PROPER PROTECTIVE EQUIPMENT

- Cleanup:  Rinse small amounts to drain where possible.  Dike or dam
large spills; pump to containers or soak up on inert absorbent.
Flush residue to sanitary sewer; rinse area thoroughly.

- Waste Disposal:  Consult state and local authorities for
restrictions on disposal of chemical waste.  Unused product as a
waste is Corrosive (D002) by RCRA criteria.

UNK = Unknown at this time PEL = Permissible Exposure Limit TLV = Threshold Limit Value STEL = Short Term Exposure Level C = Ceiling Limit, Not To Be Exceeded

M43

Product:  LIME-A-WAY	Page 2 of 2

Ecolab Inc.	942771

MEDICAL EMERGENCY ONLY, 24 HOUR SERVICE:  1-800-328-0026

HEALTH HAZARD DATA /	DANGER

Effects of Overexposure:

Skin and Eyes:  Can cause severe irritation, possible chemical

burns.

If Swallowed:  Harmful.  Can cause chemical burns of mouth, throat

and stomach.

If Inhaled:  Vapors cause irritation, including a burning taste,

sneezing, coughing and difficulty breathing.  People with asthma or

other lung problems may be more susceptible.

FIRST AID /		     ~

Eyes:  Immediately flush with plenty of cool running water.  Remove

contact lenses.  Continue flushing for at least 15 minutes, holding

eyelids apart to ensure rinsing of entire eye.  CALL A PHYSICIAN

IMMEDIATELY.

Skin:  Immediately flush skin with plenty of cool running water for

at least 15 minutes while removing contaminated clothing and shoes.

Wash clothing before reuse.

If Swallowed:  Rinse mouth at once; then drink 1 or 2 large glasses

of water or milk.  DO NOT induce vomiting.  NEVER give anything by

mouth to an unconscious person.

If Inhal-ed:  Immediately move to fresh air.

CALL A POISON CONTROL CENTER OR PHYSICIAN IMMEDIATELY

SPECIAL PROTECTION INFORMATION /

Respiratory:  Avoid breathing dusts or mists of this product. Skin:  Rubber gloves - protective cuff or gauntlet type preferred. Eyes:  Splashproof glasses, goggles or face shield. Other:  Avoid contact with use solutions.  These may also be hazardous.

3 ADDITIONAL INFORMATION/PRECAUTIONS /     ~~~   ~~      __— 1 DOT Class:  Corrosive Material UN 1760

KEEP OUT OF REACH OF CHILDREN

above information is believed to be correct with respect to the
nula used to manufacture the product.  As data, standards and
illations change, and conditions of use and handling are beyond our
trol, NO WARRANTY, EXPRESS OR IMPLIED, IS MADE AS TO THE
PLETENESS OR CONTINUING ACCURACY OF THIS INFORMATION.	^

^