FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"	MSDS #.    721 Oo

Material Safety Data Sheet (MSDS)	Revision Date:    March i, 2004

Section 1 — Chemical Product and Company Identification

Sodium Chloride

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition, Information on Ingredients

Sodium Chloride Synonym: table salt

CAS#: 7647-14-5

Section 3 — Hazards Identification

White, odorless, crystals.	fl.nn Ar-A-GLANCE

Very slightly toxic by ingestion. Dust may cause minor irritation to mucous membranes upon inhalation.

Flammability-0

Reactivity-0

Exposure-0

Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash with soap and water.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non combustible.	NFpA code

When heated to decomposition, emits toxic fumes of Cl and Na2O.	None

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and	Established
SCBA with full facepiece operated in positive mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and phosphates. Store in a cool dry place. Keep container tightly closed.

Section 8 — Exposure Controls, Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

© 2005 Flinn Scientific, Inc. All Rights Reserved.	PAGE 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"	msds#:   72100

*m   a.     •   i e\   f  *     n   *     «i        i /.•/tr-vnv	Revision Date:   March 1,2004

Material Safety Data Sheet (MSDS)

Sodium Chloride

Section 9 — Physical and Chemical Properties

White crystalline powder. Odorless.	Specific Gravity: 2.165

Solubility: Soluble in water and glycerol; slightly in alcohol.	Melting Point: 801 C

Formula: NaCl	Boiling Point: 1413 C
Formula Weight: 58.44

Section 10 — Stability and Reactivity

Reacts violently with bromine trifluoride and lithium. Avoid contact with strong oxidizers, acids, bromine. Shelf life: Fair, somewhat hygroscopic.

Section 11 —lexicological Information

Acute effects: Irritant	ORL-RAT LD50: 3000 mg/kg

Chronic effects: N.A.	IHL-RAT LC50: N.A.

Target organs: N.A.	SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231-598-3).

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Questions on Chemical Disposal or Storage?—Call Flinn

flinn@flinnsci.com   www.flinnsci.com P.O. Box 219        BatavialL 60510