MATERIAL SAFETY DATA SHEET: VAN-AWAY AEROSOL

Page: 1

(000000-000000-

-5069

DATE   OF   ISSUE 7/02/2031

SUPERSEDES 6/01/2001

SECTION   I   -   GENERAL   INFORMATION

Chemical Name   fc  Synonyms N/A

Trade Name   &   Synonyms VAN-AWAY   AEROSOL

Chemical Family: XYLENE  EMULSION

Formula Mixture  --> X

Manufacturer' s   Name:

MANTEK,    DIVISION   OF   NCH   CORP.

Address:

BOX  152170 IRVING,

TEXAS   75015

Prepared By:

L  Boynton/Chemist

Product  Code Number 5069

Emergency Phone Number 800-424-9300

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical Name (Ingredients)

PROPANE

N-BUTANE

OIL SOLUBLE SODIUM SULFONATE

OCTYLPHENOXYPOLYETHOXYETHANOL

SODIUM NITRITE

PINE OIL

XYLENE

ETHYL BENZENE

NONYLPHENOL POLYETHOXYLATE

$ OIL MIST VALUE

$$ IRR/FLAM/CARC

Hazard

ASPHYXIANT

ASPHYXIANT

IRRITANT

IRRITANT

IRR/TOX

IRRITANT

IRR/FLAM

$$

IRRITANT

TLV

1000 PPM 1 800 PPM  1 5 MG/M3$ 1 N/E   1. N/E   1. 5 MG/M3$ 1 100 PPM  1 N/E   1. N/E   1.

PEL

1000 PPM 2 800 PPM  2 5 MG/M3$ 2 N/E   2. N/E   2. 5 MG/M3$ 2 100 PPM  2 N/E   2. N/E   2.

STEL

N/E

N/E

N/E

N/E

N/E

N/E

N/E

N/E

N/E

CAS »

74-98-6

106-97-6

68608-26-4

9036-19-5

7632-00-0

8002-09-3

1330-20-7

100-41-4

9016-45-9

SECTION Ila - NON-HAZARDOUS INGREDIENTS (NON-HAZARDOUS INGREDIENT NAMES AND CAS NUMBERS ARE PROTECTED UNDER NJ TRADE)

Secret Registry #: 409363-5010P

SECTION III - PHYSICAL DATA

Boiling Point (F) :	IBP 210	Specific Gravity	(H20=l) :	0.925

Vapor Pressure (MM HG) :	12	Color:	OFF-WHITE TO TAN

Vapor Density (Air=l) :	0.6	Odor:	XYLENE

PH 8 100» :	8.8	Clarity:	OPAQUE

% Volatile by Volume:	93	Evaporation Rate	(BU A/C=1) :	1.0

H2O Solubility:	APPRECIABLE	Viscosity:	NON-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

100°F    / SETAFLASH	XYLENE	1.1	7.0

Extinguishing Media

X <—Foam         X <—Alcohol Foam        X <—CO2       X <--Dry chemical        X <—Water Spray      <—Other

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF

THE SURROUNDING FIRE.

Unusual Fire and Explosion Hazards:

USE WATER SPRAY TO COOL FIRE EXPOSED CONTAINERS TO PREVENT BURSTING. FLAME EXTENSION: 36" BURNBACK: 2 - 3".

Aerosol Level (NFPA 30B):    1

NFPA 704 Hazard Rating       (0=Insignif icant l=slight  2=Moderate  3=High  4=Extreme) 2  <—Health      3  <—Flammability  0  <—Instability      <--Special

SECTION V - HEALTH HAZARD DATA

Threshold Limit Value: f^ "T  ESTABLISHED   FOR MIXTURE.      SEE   SECTION   II.

.fects of Overexposure:

-Acute (Short Term Exposure)

SKIN CONTACT: CAUSES SEVERE IRRITATION SEEN AS ITCHING AND REDNESS. PROLONGED OR REPEATED CONTACT MAY CAUSE BURNING, DRYNESS AND CRACKING OF THE SKIN. EYE CONTACT: CAUSES SEVERE IRRITATION SEEN AS TEARING, SWELLING, STINGING, REDNESS AND A BURNING SENSATION-

SECTION V - HEALTH HAZARD DATA-- tCgntinued)

INHALATION: CAUSES RESPIRATORY IRRITATION SEEN AS COUGHING AND SNEE2ING. MAY CAUSE DIFFICULTY IN BREATHING, NOSE AND THROAT IRRITATION, PULMONARY IRRITATION, HEADACHE, DIZZINESS, DROWSINESS, UNCONSCIOUSNESS, ANESTHESIA AND OTHER CENTRAL NERVOUS SYSTEM EFFECTS.  INHALATION OF LARGER AMOUNTS MAY CAUSE SYSTEMIC EFFECTS AS NITRITES ARE ABSORBED BY LUNG TISSUE RESULTING IN EFFECTS SIMILAR TO INGESTION.

INGESTION: CAUSES IRRITATION WITH POSSIBLE NAUSEA, VOMITING AND DIARRHEA. MAY CAUSE IRRITATION TO THE MOUTH, ESOPHAGUS AND STOMACH, CYANOSIS, A DROP IN BLOOD PRESSURE, PULMONARY INJURY AND AN ALLERGIC REACTION.

-Chronic (Long Term Exposure)

target organs: central and peripheral nervous system, circulatory system, liver, lungs and kidneys.

medical conditions aggravated by exposure are pre-existing kidney diseases, respiratory and skin conditions such as asthma, emphysema, and

dermatitis.

chronic exposure can cause peripheral nerve numbness, chronic inhalation of solvents like xylene have caused permanent central and peripheral nervous

system damage resulting in decreased learning ability. a condition known as "painter's syndrome" can occur causing a loss of sensation in the arms

and hands (peripheral neuropathy) . under certain conditions, nitrites may react with secondary amines to form carcinogenic nitrosamines. studies with

rabbits indicate that sustained, occluded skin contact with undiluted surfactant may result in the development of inflammatory changes in the lung.

overexposure to this material has been suggested as a cause of mild, reversible liver and kidney effects in laboratory animals.

Primary Routes of Entry:     X <--Inhalation       <--Ingestion        <--Absorption

Emergency and First Aid Procedures:

-Inhalation:

REMOVE FROM THE AREA TO FRESH AIR. IF NOT BREATHING, CLEAR THE AIRWAY AND START MOUTH TO MOUTH ARTIFICIAL RESPIRATION. GET IMMEDIATE MEDICAL ATTENTION.

-Eye Contact:

IMMEDIATELY RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING FOR AT LEAST 15 MINUTES. HOLD THE EYELIDS APART TO ENSURE RINSING OF THE ENTIRE SURFACE OF THE EYES AND LIDS WITH WATER. GET IMMEDIATE MEDICAL ATTENTION.

-Skin Contact:

WASH AFFECTED AREAS WITH LARGE AMOUNTS OF SOAP AND WATER FOR 15 MINUTES- REMOVE CONTAMINATED CLOTHING AND SHOES. GET IMMEDIATE MEDICAL ATTENTION. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE.

-Ingestion:

GIVE   3   TO   4   GLASSES   OF   WATER,    BUT   DO   NOT    INDUCE   VOMITING.     IF   VOMITING   OCCURS,    GIVE   FLUIDS   AGAIN.    GET    IMMEDIATE   MEDICAL   ATTENTION.    DO   NOT   GIVE ANYTHING   BY  MOUTH   TO   AN   UNCONSCIOUS   OR   CONVULSING   PERSON.

-Notes   to  Physician: THERE   IS   NO   SPECIFIC   ANTIDOTE.    TRFAT   THE   PATIENT   SYMPTOMAT ICALLY.

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

1ARC—> Yes         NTP—> No	OSHA—> No	ACGIH—> No	OTHER—> No

PROPANE NO TOXICITY DATA AVAILABLE

N-BUTANE

IHL-RAT LC50: 658 G/M3/4H 3. IHL-MUS LC"0: 630 ".'M2/4H 3

OIL SOLUBLE SODIUM SULPONATE NO TOXICITY DATA AVAILABLE

OCTYLPHENOXYPOLYETHOXYETHANOL

ORL-RAT LD50: 4190 MG/KG	3,

SKN-MOUSE TDLo: 600 MG/KG/4W-I	3,

EYE-RBT-SDT: 1% SEVERE	3,

SODIUM NITRITE

ORL-HMN TDLo: 14 MG/KG	3.

ORL-HMN LDLo: 71 MG/KG	3.

ORL-RAT LD50: 180 MG/KG	3.

IHL-RAT LC50: 5500 UG/M3/4H	3.

EYE-RBT-SDT: 500 MG/24H MILD	3.

PINE OIL

ORL-HMN(MAN) TDLo: 4700 MG/KG	3.

ORL-RAT LD50: 3200 MG/KG	3.

SKN-RBT LD50: 5 GM/KG	3.

SKN-RBT-SDT: 500 MG/24H SEVERE	3.

XYLENE

ORL-RBT LD50: 4300 MG/KG IHL-RBT LC50: 5000 PPM/4H IHL-HMN LCLo: 1000 PPM/6H SKN-RBT LD50: >1700 MG/KG 3. SKN-RBT SDT: 87 MG MILD 3. EYE-RBT SDT: 5 MG/24H SEVERE 3.

ETHYL BENZENE

ORL-HMN TCLo: 100 PPM/8H 3. ORL-RBT LD50: 3500 MG/KG 3. SKN-RAT LD50: 12800 MG/KG 3.

SKN-RBT OPEN IRRITATION TEST: 15 MG/24H MILD 3. EYE-RBT SDT: 500 MG SEVERE 3.

THE IARC HAS EVALUATED ETHYL BENZENE AND CLASSIFIED IT AS A POSSIBLE HUMAN CARCINOGEN (GROUP 2B) BASED ON SUFFICIENT EVIDENCE FOR CARCINOGENICITY IN EXPERIMENTAL ANIMALS, BUT INADEQUATE EVIDENCE FOR CANCER IN EXPOSED HUMANS .

NONYLPHENOL POLYETHOXYLATE ORL-RAT LD50: 4 GM/KG   3.

SECTION VII - REACTIVITY DATA

Incompatibility (Materials to Avoid):

STRONG OXIDISING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE;  REDUCING AGENTS SUCH AS SODIUM THIOSULFATE, ACIDS, BASES,

AMMONIUM AND HYDROXYL COMPOUNDS, COMBUSTIBLES, ORGANICS, HALOGEN AND MOLTEN SULFUR.

Hazardous Decomposition Products:

OXIDES OF CARBON, NITROGEN AND SULFUR; DEGRADED ALCOHOLS, HYDROCARBONS, HYDROGEN, SODIUM, FUMES AND SMOKE.

Hazardous Polymerisation:	<--May Occur      X <--Will Not Occur

Conditions to Avoid:

N/A

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:

DUE TO THE NATURE OF THE AEROSOL PACKAGING, A LARGE SPILL IS UNLIKELY. FOR A SMALL SPILL, ELIMINATE ALL SOURCES OF IGNITION AND WEAR APPROPRIATE

PROTECTIVE CLOTHING. ABSORB WITH AN INERT MATERIAL. TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS. TYPICAL DISPOSAL IS TO WRAP THE EMPTY AEROSOL CONTAINER IN SERVERAL LAYERS

OF NEWSPAPER AND DISPOSE OF IN THE TRASH. AEROSOL RECYCLING PROGRAMS ARE AVAILABLE IN MANY AREAS. DO NOT PUNCTURE OR INCINERATE THIS CONTAINER.

Neutralizing Agent: N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE MISTS OR VAPORS.

Respiratory Protection:

A NIOSH/MSHA APPROVED RESPIRATOR IN POORLY VENTILATED AREAS AND/OR FOR EXPOSURE ABOVE THE ACGIH TLV OR OSHA PEL OR WHERE MISTING EXISTS -

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES SHOULD BE WORN.

Eye Protection:

CHEMICAL GOGGLES SHOULD BE WORN.

Other Protection:

wear protective clothing when handling.

r

SECTION  X   -   STORAGE  AND   HANDLING   INFORMATION

Borage Temperature:    Indoor s--> X~ ~	~ OufctOo"r3=>

-inimuin Temperature: 35°F    Maximum Temperature: 120°F

Precautions to be Taken in Handling and Storing:

USE WITH CAUTION AROUND HEAT, SPARKS, PILOT LIGHTS, STATIC ELECTRICITY AND OPEN FLAME.

Other Precautions :

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT.  FOLLOW THE LABEL DIRECTIONS.

SECTION XI - REGULATORY INFORMATION

i cal Name	CAS Nnmher	Upper % Limit

XYLENE	1330-20-7	50

ETHYL BENZENE	100-41-4	10

SODIUM NITRITE	7632-00-0	5

Those ingredients listed above	are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthori nation Act of
1986 and 40 CFR part 372.

Please call 1-800-527-9919 for	additional information if you are a California customer.
This MSDS is not intended for users in the state of California.

SECTION XII - REFERENCES

1. THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND BIOLOGICAL EXPOSURE INDICES, ACGIH, 2001.

2 . OSHA PEL .

3. REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFODisc, 2001.

ALL COMPONENTS IN THIS PRODUCT CAN BE FOUND IN THE CURRENT TSCA INVENTORY.

IRR: IRRITANT , FLAM/FLAMM: FLAMMABLE, COMB: COMBUSTIBLE, CORR: CORROSIVE  CARC: CARCINOGENIC , TOX: TOXIC, N/A:NOT APPLICABLE, N/E: NOT ESTABLISHED,

COC: CLEVELAND OPEN CUP, PMCC: PENSKY-MART IN CLOSED CUP, TCC: TAGLIABUE CLOSED CUP, LELlLOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT, NFPA: NATIONAL

FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL TOXICOLOGY PROGRAM, OSHA: OCCUPATIONAL SAFETY & HEALTH

ADMINISTRATION, ACGIH: AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS , TLV: THRESHOLD LIMIT VALUE, PEL: PERMISSIBLE EXPOSURE LIMIT,

STEL: SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE, SEV:SEVERE, MUT :MUTAGENIC, AS PHYX: ASPHYXIANT , PNOS : PARTICULATES (INSOLUBLE) NOT OTHERWISE

SPECIFIED, SDT: STANDARD DRAISE TEST, ORL: ORAL, IHL: INHALATION, HMN: HUMAN

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

TTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product a manner not recommended on the product label.  Users assume all risks associated with such unrecommended use, storage, or disposal of the .oduct.