FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS#:    600.10

Revision Date:   March 12, 2001

Section 1 — Chemical Product and Company Identification

o-Phosphoric Acid Solution

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

o-Phosphoric Acid (7664-38-2) <1% and Water (7714-18-5) >99% Synonym: orthophosphoric acid solution

CAS#: None established

Section 3 — Hazards Identification

Colorless, odorless liquid.

Slightly toxic by ingestion and inhalation; body tissue irritant.

flinn at-a-glance

Health-1 Flammabiliry-0 Reactivity-0 Exposure-1 Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

'

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give no more than 1-2 cups of water for dilution. Do not induce vomiting. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable, non combustible liquid.

NFPA code

None established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Remove all ignition sources and ventilate area. Contain spill with sand and absorbent material, neutralize with sodium bicarbonate or calcium hydroxide and deposit in sealed bag or container. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #9. Store with acids.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

^

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

o-Phosphoric Acid Solution

MSDS#:    600.10

Revision Date:   March 12, 2001

Section 9 — Physical and Chemical Properties

Colorless, odorless liquid. Formula: H3PO4 Formula Weight: 98.00

Section 10 — Stability and Reactivity

Shelf life: Good.

Section 11 —lexicological Information

Acute effects: Harmful liquid, irritant Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: 1530 mg/kg as o-Phosphoric acid

IHL-RAT LC50: N.A.

SKN-RBT LD50: 2740 mg/kg as o-Phosphoric acid

N.A. = Not available, not all health aspects of this substance have been folly investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26b is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

Not listed.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Questions on Chemical Disposal or Storage?—Call Flinn

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   BatavialL 60510

(800) 452-1261 Fax (866) 452-1436