MATERIAL   SAFETY   DATA   SHEET:   OPTI-GRO   IRON  AIDE	Pa9e:   *

(OOO'OOO-OOCOOO-	-0410	)	DATE   OF   ISSUE	SUPERSEDES

5/28/1999	12/10/1998

SECTION   I   -   GENERAL   INFORMATION	'

Chemical   Name   £  Synonyms	Trade  Name   &   Synonyms

r

N/A	OPTI-GRO   IRON AIDE

ical Family:	Formula Mixture —> X

OUS IRON SOLUTION

Manufacturer's Name:

MANTEK, DIVISION OF NCH CORP.

Address:

BOX 152170

IRVING,         TEXAS 75015

Prepared By:	Product Code Number	Emergency Phone Number

B MALKEY/CHEMIST	0410	800-424-9300

SECTION II - HAZARDOUS INGREDIENTS
	THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical Name (Ingredients)	Hazard	TLV	PEL	STEL	CAS »

FERROUS SULFATE	CORROSIVE	1 MG/M3$	1    1 MG/M3S	2    NOT EST.	7782-63-0

SODIUM DIMETHYL NAPHTHALENE SULFONATE	IRRITANT	NOT EST.	1    NOT EST.	2    NOT EST.	26264-58-4
$MEASURED AS IRON SALTS, SOLUBLE, AS IRON

SECTION   Ila   -   NON-HAZARDOUS   INGREDIENTS
	(NON-HAZARDOUS   INGREDIENT  NAMES   AND   CAS   NUMBERS   ARE   PROTECTED   UNDER NJ   TRADE)

Secret  Registry   #:    409363-5008P
	SECTION   III   -   PHYSICAL  DATA

Boiling Point (F) :	210	Specific Gravity (H20=l) :	1.245

Vapor Pressure (MM HG) :	15	Color:	DARK BROWN

""•v-r r>=!-ity_iiir=lX:_ -	" ?-		 Oder: .-		--MUSIS

, 1004 :	2.4	Clarity:	OPAQUE

% Volatile by Volume:	95	Evaporation Rate (BU A/C=1) :	<0.10

H2O Solubility:	COMPLETE	Viscosity:	NON-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

NON-FLAM / N/A	N/A	N/A	N/A

Extinguishing Media

X <—Foam         X <—Alcohol Foam       X <—CO2       X <—Dry Chemical        X <—Water Spray      <—Other

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF

THE SURROUNDING FIRE.

Unusual Fire and Explosion Hazards:

USE WATER SPRAY TO COOL FIRE EXPOSED CONTAINERS TO PREVENT BURSTING.

Aerosol Level (NFPA 30B) :    N/A

NFPA 704 Hazard Rating       <0=Insignifleant l=Slight  2=Moderate  3=High  4=Extreme> 2  <—Health      1  <—Flammability  0  <--Instability      <—Special

SECTION V - HEALTH HAZARD DATA

Threshold Limit Value: NOT ESTABLISHED FOR MIXTURE.  SEE SECTION II.

Effects of Overexposure:

-Acute (Short Term Exposure)

INHALATION: MIST CAUSES IRRITATION TO THE RESPIRATORY TRACT.  MAY CAUSE  COUGHING.  EYES: CAUSES SEVERE IRRITATION. MAY CAUSE REDDENING, TEARING AND CONJUNCTIVITIS.  SKIN CONTACT: CAUSES SEVERE IRRITATION SEEN AS REDNESS, ITCHING AND A  BURNING SENSATION.  INGESTION: MAY CAUSE IRRITATION TO THE MOUTH t THROAT, HEARTBURN, NAUSEA,  VOMITING, CONSTIPATION (OR DIARRHEA).  INGESTION OF LARGE AMOUNTS MAY  CAUSE SEVERE HEMORRHAGIC GASTRITIS WITH ABQOMINAL PAIN, RETCHING, VIOLENT  DIARRHEA AND VOMITING.  THE CIRCULATORY SYSTEM MAY ALSO BE EFFECTED WITH  SYMPTOMS OF SHOCK, RAPID, WEAK OR NO ",, SEVERE HYPOTENSION AND  PULMONARY CHANGES WITH DYSPNEA AND EMPHYSEMA MAY OCCUR.

^^-•*"	-Chronic (Long Term Exposure)

NO HUMAN CHRONIC EFFECTS KNOWN.  MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE: MAY AGGRAVATE  PREEXISTING LIVER AND CIRCULATORY DISORDERS. TARGET ORGANS: LIVER S. CIRCULATORY SYSTEM.  THERE IS NO PRIMARY ROUTE OF ENTRY INTO THE BODY.  THE PRIMARY ROUTE OF EXPOSURE IS SKIN AND EYE CONTACT.

MATERIAL SAFETY DATA SHEET: OPTI-GRO IRON AIDE         Page: 2 SECTION V - HEALTH HAZARD DATA (Continued)

<--Inhalation	<--Ingestion	<--Absorption

Emergency  an-! First Aid  Procedures:

-Inhalation:	^_

REMOVE   FROM THE  AREA  TO   FRESH AIR.    SEEK  MEDICAL  ATTENTION   IF  RESPIRATORY   IRRITATION  DEVELOPS   OR   IF   BREATHING  BECOMES   DIFFICULT.

-Eye Contac t:

IMMEDIATELY RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING FOR AT LEAST 15 MINUTES. HOLD THE EYELIDS APART TO ENSURE	^_

RINSING OF THE ENTIRE SURFACE OF THE EYES AND LIDS WITH WATER. GET IMMEDIATE MEDICAL ATTENTION.

-Skin Contact:

WASH AFFECTED AREAS WITH LARGE AMOUNTS OF SOAP AND WATER FOR 15 MINUTES. REMOVE CONTAMINATED CLOTHING AND SHOES. GET IMMEDIATE MEDICAL ATTENTION. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE.

-Ingestion:

IMMEDIATELY GIVE 3 TO 4 GLASSES OF WATER, AND INDUCE VOMITING BY PLACING A FINGER OR BLUNT OBJECT ON THE BACK OF THE PERSON'S TONGUE. GIVE FLUIDS UNTIL VOMITUS IS CLEAR. GET IMMEDIATE MEDICAL ATTENTION. DO NOT INDUCE VOMITING OR GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

-Notes to Physician:

GASTRIC LAVAGE MAY BE INDICATED USING DEFEROXAMINE, 2 GRAMS IN 1 LITER OF WATER WHICH CONTAINS SODIUM BICARBONATE 20 GRAMS/LITER.  LEAVE 10 GRAMS OF DEFEROXAMINE IN 50 ML OF 5% SODIUM BICARBONATE IN THE STOMACH.  TREAT SYMPTOMATICALLY AND SUPPORTIVELY.

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

LARC—> No         NTP—> No	OSHA—> No	ACGIH—> No	OTHER—> No

FERROUS SULFATE

ORL-RAT LD50: 1520 MG/KG    3.

ORL-RAT LDLO: 1389 MG/KG    4.

IVN-RBT LDLO: 99 MG/KG      4. SODIUM DIMETHYL NAPHTHALENE SULFONATE

ORL-RAT LD50: 5.62 G/KG     4.

PRIMARY DERMAL TOXICITY: 0.63 NON IRRITATING.       3.

PRIMARY IRRITATION INDEX: 1.21 MILD IRRITANT.       3.

IN A SUB ACUTE 90 DAY FEEDING STUDY DOGS WERE FED LEVELS OF 500, 1000,

1500 AND 5000 PPM.  NO SIGNIFICANT EFECTS WERE NOTED ON GROWTH,

BLOOD UREA NITROGEN, GLUCOSE, BLOOD ENZYME VALUES OR OTHER TESTS       3.

AMES MUTAGENIC ASSAY: NO MUTAGENIC ACTIVITY DETECTED.	3.

ACUTE INHALATION STUDIES-RAT: NO MORTALATIES AFTER 19.1 MG/1 HOUR.     3.

SECTION VII - REACTIVITY DATA

Stability:         X <—Stable      <—Unstable Conditions to Avoid: NONl? KNOWN.

Incompatibility (Materials to Avoid):

OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND HYDROGEN PEROXIDE; ALKALIES.

Hazardous Decomposition Products: OXIDES OF SULFUR AND CARBON.

Hazardous Polymerization:	<--May Occur      X <—Will Not Occur

Conditions to Avoid: NONE KNOWN.

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:

DIKE AND CONTAIN SPILL IF SAFE TO DO SO. ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. WEAR

APPROPRIATE PROTECTIVE CLOTHING.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS.

Neutralizing Agent:

LIME OR SODIUM BICARBONATE.

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

GENERAL VENTILATION IS NORMALLY ADEQUATE.

Respxratory Protection:

TYPICAL USE OF THIS PRODUCT DOES NOT REQUIRE THE USE OF A RESPIRATOR.  IF PRODUCT IS MISTED, USE NIOSH APPROVED RESPIRATOR FOR SPRAY MIST.

Glove Protection:

NEOPRENE OR VITON GLOVES IF REPEATED OR PROLONGED SKIN CONTACT IS EXPECTED.

Eye Pro tec tion:

CHEMICAL GOGGLES AND A FACE SHIELD SHOULD BE WORN.

Other Protection:

RUBBER APRON IF NECESSARY TO PREVENT SKIN CONTACT.

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:	Indoors--> X	Outdoors-->	Heated-->	Refrigerated-->

MATERIAL SAFETY DATA SHEET: OPTI-GRO IRON AIDE         Pa.ge: 3 " _ [ __ SECTION X - STORAGE AND HANDLING INFORMATION (Continued)

Minimum Temperature: 35°F   Maximum Temperature: 120°F

Precautions to be Taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE.

Other Precautions :

OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS.

SECTION XI - REGULATORY INFORMATION

CAS Number	Upper % Linu. t

Those ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthonzation Act of 1986 and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. This MSDS is not intended for users in the state of California.

_ SECTION XII - REFERENCES _

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS
AND BIOLOGICAL EXPOSURE INDICES, ACGIH, 1998.

- OSHA PEL.   3. VENDOR'S MSDS

4. SAX'S DANGEROUS PROPERTIES OF INDUSTRIAL MATERIALS, EIGHTH EDITION,

RICHARD J. LEWIS, SR. ALL COMPONENTS IN THIS PRODUCT CAN BE FOUND IN THE CURRENT TSCA INVENTORY.

IRR: IRRITANT, FLAM/FLAMM: FLAMMABLE, COMB: COMBUST IBLE, CORR: CORROSIVE CARC:CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC: CLEVELAND OPEN CUP, PMCC: PENSKY-MART IN CLOSED CUP, TCC: TAGLIABUE CLOSEDCUP, LEL: LOWER EXPLOSION LIMIT, DEL:UPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL TOXICOLOGY PROGRAM, OSHA: OCCUPATIONAL SAFETY £ HEALTH ADMINISTRATION, ACHGIH:AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV: THRESHOLD LIMIT VALUE, PEL: PERMISSIBLE EXPOSURE LEVEL, STEL: SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD: MODERATE, SEV:SEVERE, MUT:MUTAGENIC, ASPHYX: ASPHYXIANT, PNOC: PARTICULATES NOT OTHERWISE CLASSIFIED

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

MAHTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product f   manner noir"recomnKindea on trie produce label.  user's assume a~ii risKS associateor~witn such unrecommended use, storage," "OTT^OIspo^gd: or the

let.