Buckeye International, Inc.

Material Safety Data Sheet

24 Hour Medical Emergency Telephone Number: 1-800-303-0441. Outside North America call: 651-632-8956.

24 Hour Transportation Emergency Telephone Number:   1-800-535-5053. Outside North America call: 352-323-3500.

2700 Wagner Place Maryland Heights, MO 63043 314/291-1900

| N. F. P. A. 4 = Extreme 3 = High 2 = Moderate 1 = Slight 0 = Insignificant | ^-T HEALTH | 2 |
| --- | --- | --- |
|  | £&*- FIRE | 0 |
|  | 2*f_ REACTIVITY | 0 |

| SECTION I - IDENTIFICATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PRODUCT NAME |  | RAM |  |  | DATE PREPARED April 1, 2005 |
| CHEMICAL FAMILY |  | Heavy Duty Cleaner Degreaser, Water Based |  |  | CODE 5014 |
| PROPER D.O.T. SHIPPING NAME |  | Compound, Cleaning, Liquid |  |  |  |
| D.O.T. HAZARD CLASSIFICATION |  | None |  |  |  |
| SECTION II - INGREDIENTS AND IDENTITY INFORMATION |  |  |  |  |  |
| % By WGT | MATERIAL |  |  | PEL T.L.V. | C.A.S. NO. |
| 2.0 | Sodium Metasilicate Anhydrous |  |  | NE NE | 6834-92-0 |
| >85.0 | Soft Water |  |  | NE NE | 7732-18-5 |
| <7.0 | Linear Primary Alcohol Ethoxylate |  |  | NE NE | 34398-01-1 |
| <3.0 | Sodium Iminodisuccinate |  |  | NE NE | 144538-83-0 |
| <3.0 | Coloring and additives less than |  | 1% | NA NA | NA |
| ITEMS MARKED * ARE SARA TITLE III SEC 313 REPORTABLES. ALL INGREDIENTS ARE ON TSCA INVENTORY. |  |  |  |  |  |
| SECTION III - PHYSICAL DATA |  |  |  |  |  |
| BOILING POINT °F |  | 212°F |  | pH (CONC.) | 12.6 ±0.2 |
| SOLUBILITY IN WATER |  | Infinite |  | pH (USE DILUTION) 1:10 | 11.8±0.2 |
| % VOLATILE BY WEIGHT |  | 91.0 |  | EVAPORATION RATE (Water=l) | 1.0 |
| SPECIFIC GRAVITY |  | 1.03 |  | PRODUCT FORM | Liquid |
| APPEARANCE AND ODOR |  | Mild, No Perfume Added, Clear Green Solution |  |  |  |
| SECTION IV - FIRE AND EXPLOSION DATA |  |  |  |  |  |
| FLASH POINT (Test Method) Tag Closed Cup: None! |  |  |  | FLAMMABLE LIMITS |  |
| EXTINGUISHING MEDIA |  | NA |  | UPPER LIMIT: NA | LOWER LIMIT: NA |
| SPECIAL FIRE FIGHTING PROCEDURES ¦ |  |  | None |  |  |
| UNUSUAL FIRE AND EXPLOSION HAZARDS ¦ Oxides of carbon, silicon, and nitrogen. |  |  |  |  |  |
| SECTION V - REACTIVITY DATA |  |  |  |  |  |
| STABILITY |  | Stable |  |  |  |
| CONDITIONS TO AVOID |  | None determined. |  |  |  |
| INCOMPATIBILITY |  | Do not mix with chlorine bleach. |  |  |  |
| HAZARDOUS DECOMPOSITION PRODUCTS None known. |  |  |  |  |  |
| HAZARDOUS POLYMERIZATION |  |  | Will not occur. |  |  |

CODE:  5014

SECTION VI - HEALTH HAZARD DATA

ROUTE(S) OF ENTRY:	INHALATION?   No	SKIN?   Yes	INGESTION?    No

HEALTH HAZARDS (Acute and Chronic)

Will cause severe eye and slight skin irritation.

Not toxic by ingestion.

:ARCINOGENICITY:	NTP?   No	IARC MONOGRAPHS?     No	OSHA REGULATED?    No

SIGNS AND SYMPTOMS OF OVEREXPOSURE

For Eyes:   Redness or burning sensation.

For Skin:   Redness of skin or a warming sensation.

MEDICAL CONDITIONS

GENERALLY AGGRAVATED BY EXPOSURE

Dermatitis

EMERGENCY AND FIRST AID PROCEDURES

For Eyes: Flush with cool water for 15 minutes. If irritation occurs and persists, see physician.
For Skin:   Flush with water for 15 minutes. If irritation occurs and persists, see physician.
For Ingestion:   Give two large glasses of water.   DO NOT induce vomiting. Call physician.
Never give anything by mouth to an unconscious person.

SECTION VII - SPILL OR LEAK PROCEDURES

SPILL RESPONSE

Pick up with mop, wet/dry vac or absorbent material. Rinse area with clear water and allow floor to dry before allowing traffic.

WASTE DISPOSAL

METHOD	Flush to sanitary sewer or send to sanitary landfill, following local, state and federal laws.

SECTION VIII - SPECIAL PROTECTION INFORMATION

EYE PROTECTION              When eye contact may occur,               VENTILATION
wear safety glasses or chemical splash goggles.	Normal room ventilation.

SKIN	Rubber gloves or other	RESPIRATORY

PROTECTION	impervious gloves.*	PROTECTION	None required.

OTHER

PROTECTION	Footwear that keeps feet dry is suggested.

Follow good personal hygiene practices.
	* Needed for clean up of spill or for possible prolonged skin contact.

SECTION IX - SPECIAL PRECAUTIONS

PRECAUTIONS IN	KEEP OUT OF REACH OF CHILDREN!

HANDLING AND	Store at room temperature.

STORAGE	Keep container closed when not in use.

OTHER	This product is not regulated under CERCLA.   This product is not regulated under

PRECAUTIONS	RCRA when product label and MSDS spill directions are followed. Not reportable

NA = Not Applicable	under SARA Title III, Section 311/312.

NE = Not Established	Product produced under the following U.S. patent: 6,716,804.

PREPARED BY:         Mark Gindling, Director of Research