Material Safety Data Sheet

May be used to comply with OSHA's Hazard Communication Standard, 29 CFR 1910.1200.   Standard must be consulted for specific requirements.

HEALTH FLAMMABILITY

HMIS8

0

REACTIVITY	0

PERSONAL PROTECTION	None

Identity (As Used on Label and List) RID O GRIME

No1e:    Blank spaces are nor permitted.   If any item is not applicable, or no information is available, the space must be marked to indicate that.

SECTION I

| SOLD BY: Continental Research Corporation | Emergency Telephone Number 1-800-325-4869 |
| --- | --- |
| Address (Number, Street, City, State, and ZIP Code) Box 15204 | Telephone Number for Information 1-314-426-0410 |
| St. Louis, MO 63110 | Date Prepared October 29, 2001 |
|  | Signature of Preparer (Optional) REGULATORY DEPT. |

SECTION II - Hazardous Ingredients/Identity Information

Hazardous Components	(Specific

Chemical identity; Common Name(s))

Isoparaffin

9-Octadecenoic acid

Clay

Pumice

ACGIH TLV

OSHA PEL

CAS No.

68551-17-7        5 mg/m3(mist)        5 rng/m3(mist)

Not established      Not established

112-80-1

1 5 mg/m3(dust)        10 mg/m3(dust)

1317-65-3

60676-86-0        1 5 mppcf (dust)     Not established

OTHER LIMITS RECOMMENDED

%{0pt.)

None of the compounds in this product are currently listed for reporting under SARA Title III Sec. 313.   Reference 40 CFR Part
372.						~~

Any substance listed as hazardous by the States of California, Florida, Illinois, Michigan, New Jersey, Ohio, Pennsylvania or Texas is described above if known present in regulated concentrations.

SECTION III   Physical/Chemical Characteristics

| Boiling Point (initial) | 212° F | Specific Gravity (H2O = 1) @70° F | 0.95 |
| --- | --- | --- | --- |
| Vapor Pressure (mm Hg @ 70° F) | No Data | Melting Point | 140°F |
| Vapor Density (AIR = 1) | Greater than one (1) | Evaporation Rate (Butyl Acetate = 1) | No Data |
| Solubility in Water | Completely dispersible | pH | 8.3 + 0.5 |

Appearance and Odor -

Opaque orange ringing gel with a citrus scent.

SECTION IV - Fire and Explosion Hazard Data

| Flash Point (Method Used) - None to boiling point (PMCC | - est.) | Flammable Limits | No Data | LEL No Data | UEL No Data |
| --- | --- | --- | --- | --- | --- |

Extinguishing Media - Not usually necessary as this product does not readily support combustion.   Use carbon dioxide, dry

chemical, foam or water fog if required.

Special Fire Fighting Procedures - Keep containers cool and vapors down with water spray.   Prevent runoff from entering sewers and public waterways.   Wear SCBA in chemical fires. Unusual Fire and Explosion Hazards None known.

SECTION V - Reactivity Data

| Stability | Unstable |  | Conditions to Avoid - | Temperatures under | 40°F |
| --- | --- | --- | --- | --- | --- |
| Stable |  | X |  |  |  |

Incompatibility (Materials to Avoid) -

Strong acids, bases, oxidizers, and selected amines.

Hazardous Decomposition or Byproducts -

Carbon dioxide and monoxide, hydrogen bromide, nitrous oxides and ammoniacal compounds.

| Hazardous Polymerization | May Occur |  | Conditions to Avoid - | None known |
| --- | --- | --- | --- | --- |
|  | Will Not Occur | X |  |  |

SECTION VI - Health Hazard Data

| Route(s) of | Entry | Eyes? | Yes | Inhalation? | No | Skin? | No | Ingestion? | No |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Health Hazards (Acute and Chronic) - Warning: May cause eye irritation (pumice).   May be harmful if swallowed.   Ingestion may irritate gastrointestinal tract or cause CNS effects.

IARC Monographs?

OSHA Regulated?

No

Carcinogenicity:

None known

NTP?

No

No

Signs and Symptoms of Exposure - Redness, tearing, burning, scratching in eyes.   Irritation of throat or stomach, followed by

diarrhea.   Drowsiness, dizziness, loss of coordination.

Medical Conditions Generally Aggravated by Exposure

Pre-existing skin disorders   (i.e. - dermatitis).

Emergency and First Aid Procedures:

Eyes - Flush with plenty of water for at least 1 5 minutes lifting eyelids to insure complete removal.   Get immediate medical

attention.   Ingestion - Rinse out mouth.   Drink plenty of water.   Call a physician or poison control center immediately.   Never

give anything by mouth to an unconscious person. Inhalation - Get to fresh air.   If breathing has stopped, qualified personnel

should administer artificial respiration.   Skin - Wipe off or rinse off with water.

SECTION VII - Precautions for Safe Handling and Use

Steps to be Taken in Case Material is Released or Spilled -

Caution - slip hazard.   Wipe up or scoop product for reuse, or put in suitable container for disposal.   Rinse residuals with water.

Waste Disposal Method -

Consult local, state, and federal regulations.   Triple rinse empty container then offer for recycling, reuse or reconditioning.

Precautions to be Taken in Handling and Storing - Avoid contact with eyes.   Do not swallow.   Keep from freezing.   Store in a

cool, dry location.   Replace lid when not in use or use follow plate.   Wash or wipe skin after use.	f

Other Precautions -   Follow label directions carefully.   Keep out of reach of children.   Keep container tightly sealed when not in

use, use follow plate or dispenser.   Do not contaminate water, food or feed by use or storage.

SECTION VIII - Control Measures

Respiratory Protection (Specify Type) -

Not usually necessary.   Use with adequate ventilation.   Use NIOSH/MSHA approved respirator if PELs or TLVs are exceeded.

| Ventilation | Local Exhaust | Not necessary |  | Special | None |
| --- | --- | --- | --- | --- | --- |
|  | Mechanical (General) | Yes |  | Other | None |
| Protective Gloves - Not necessary |  |  | Eye Protection - Only if splattering possible. |  |  |

Other Protective Clothing or Equipment -

Not usually necessary.

Work/Hygienic Practices -

Normal.   Wipe or wash hands after use and before eating, drinking, smoking, using restrooms, etc.

While tne information and recommendations set forth herein are believed to be accurate as of the date hereon Continental Research Corporation makes no warranty with respect i disclaims all liability from reliance thereon.