FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

MSDS #:    437.00

Material Safety Data Sheet (MSDS)	Revision Date: March 9,2001

Section 1 — Chemical Product and Company Identification

Lead Nitrate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Lead Nitrate

Synonym: lead (II) nitrate

CAS#:  10099-74-8

Section 3 — Hazards Identification

White crystals. Slight nitric acid odor.	flinn Ar-A-GLANCE

Moderately toxic by ingestion or inhalation. Toxic as lead dust or lead fume. Possible carcinogen.	Health-2

Severe eye, skin and mucous membrane irritant. Avoid all body contact.	Flammability-0

Chronic exposure to inorganic lead via inhalation or ingestion can result in accumulation in and	Reactivity-3

damage to the soft tissues and bones.	Exposure-2

Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. After vomiting, give mixture of 2 Tbs. of activated

charcoal mixed with one cup of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable solid.	NFPA code

Strong oxidizer, dangerous fire risk in contact with organic materials.	None Established

When heated to decomposition, emits toxic fumes of Pb and NOx.

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE

and SCBA with full facepiece operated in positive pressure mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #3. Store with amides, nitrates and nitrites. Store in a cool dry place. Use and dispense in a hood.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 0.15 mg/m3 (Pb) (ACGIH)

) 2002 Flinn Scientific, Inc. All Rights Reserved.	page 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Lead Nitrate

MSDS #:     437.00

Revision Date:   March 9, 2001

Section 9 — Physical and Chemical Properties

White crystals. Slight nitric acid odor.

Solubility: Soluble in water and alcohol. Decomposes at 470 C

Formula: Pb(NO3)2

Formula Weight: 331.21

Specific Gravity: 4.53 Melting Point: 470 C (dec.)

Section 10 — Stability and Reactivity

Avoid contact with strong reducers, finely powdered metals. Shelf Life: Indefinite.

Section 11 — lexicological Information

Acute effects: Harmful dust, irritant

Chronic effects: Reproductive hazard

Target organs: Blood, kidneys, nerves, male/female reproductive

system

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #27f is one option.

Section 14 — Transport Information

Shipping Name: Lead nitrate Hazard Class: 5.1, Oxidizer, poison UN Number: UN1469

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (233-245-9), RCRA code D001, D008.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Improve Student Lab Results-Use Flinn Chemicals

flinn@flinnsci.com  www.flinnsci.com

P.O. Box 219   BatavialL 60510

(800) 452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2