FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

MSDS #:    94.00

Material Safety Data Sheet (MSDS)	Revision Date: November 25,2002

r r^^^

Section 1 — Chemical Product and Company Identification

Barium Chloride

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Barium Chloride

CAS#: 10326-27-9

Section 3 — Hazards Identification

White odorless powder.	flinn Ar-A-GLANCE

Highly toxic by ingestion and inhalation. All soluble barium compounds are poisonous if swallowed	Health-3

and cause nausea, vomiting, stomach pains and diarrhea.	Flammability-0

Reactivity-0 Exposure-3 Storage-1

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Induce vomiting. After vomiting, give a mixture of 1 Tbs. magnesium sulfate in one cup of water. Call a physician or

poison control at once.

Section 5 — Fire Fighting Measures

Non-flammable solid.	NFPA code

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE         None Established and SCBA with full facepiece operated in positive pressure mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and

phosphates. Store in a Flinn Chem-Saf Bag.

Store in a cool dry place. Use and dispense in a hood.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 0.5 mg/m3 (Ba) (OSHA, ACGIH)

' 2002 Flinn Scientific, Inc. All Rights Reserved.	page 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Barium Chloride

MSDS #:    94.00

Revision Date:   November 25, 2002

Section 9 — Physical and Chemical Properties

White odorless powder.

Solubility: Soluble in water. (39.3 grams/100 ml).

Formula: BaC12 2H2O Formula Weight: 244.28 Specific Gravity: 3.1

Section 10 — Stability and Reactivity

Avoid contact with strong oxidizers. Shelf life: Indefinite.

Section 11 —Toxicoloqical Information

Acute effects: highly toxic, irritant, stomach pains, vomiting, diarrhea

Chronic effects: N.A.

Target organs: heart, nerves, kidneys, GI system, bone marrow, spleen,

liver

N.A. = Not available, not all health aspects of this substance have been fully investigated.

ORL-RAT LD50: 118 mg/kg IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #27h is one option.

Section 14 — Transport Information

Shipping Name: Barium Compounds, n.o.s. Hazard Class: 6.1, keep away from foods. UN Number: UN 15 64

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (233-788-1), RCRA D005.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Need a Chemical Fast?-Order from Flinn

flinn@flinnsci.com  www.flinnsci.com

P.O. Box 219   BatavialL 60510 (800) 452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2