PAGE 1 OF 2

MATERIAL SAFETY DATA SHEET	U.S. DEPARTMENT OF LABOR

COMPLIES WITH USDL SAFETY AND HEALTH REGULATIONS,  (29 CFR 19*0.1200)

SECTION I :  MANUFACTURED FOR

SEVEN OAKS SUPPLY CO

BOX 215

OAKBORO NC  28129

EMERGENCIES :  (7O4)-485-4117

PRODUCT IDENTIFICATION

'BIG^'G"        SUPER   GRIME   &   GREASE   GET! INDUSTRIAL   CLEANER

MGR:B/3O/8«-4

SECTION   II   :

HAZARDOUS INGREDIENTS ETHYLENE GYLCOL MONOETHYL ETHER ETHYLENE GLYCOL NONOBUTYL ETHER INERTS

7.

- 8

- 5
93. 7

TLV,  (IN PPM)

50

25

CONSTITUENTS CONTAINED IT

BASIC RAW MATERIALS, WHICH

MAY INCLUDE WATER.  TLV IS

EXPECTED TO EXCEED 1000 PPM.

SECTION III : PHYSICAL DATA

CLEAR LIQUID

BOILING POINT : 2O5 DEGREES FAHRENHEIT

26. 6 MM HG

VAPOR PRESSURE,  (@ 25 DEGREES CENTIGRADE)

VAPOR DENSITY, (AIR=1) : 2.6

SOLUBILITY IN WATER : COMPLETE

SPECIFIC GRAVITY, (H20=l) : 1.05

PERCENT VOLATILE BY VOLUME : 767.

EVAPORATION RATE,  (WATER=1) : > 1

P H : 13. 1

APPEARANCE   AND   ODOR   :    BLUE-GREEN,    CLEAN   SMELLING,

SECTION IV : FIRE AND EXPLOSION DATA

FLASH POINT,(METHOD USED) : NO FLASH @ BOIL,  (C.C.)

FLAMMABLE LIMITS,  (A.I. 65O DEGREES FAHRENHEIT) : LEL: 2. O, UEL:11.1

EXTINGUISHING MEDIA : 1) DRY CHEMICAL;  2) FOAM;  3) CO2

SPECIAL FIREFIGHTING PROCEDURES : NONE

UNUSUAL FIRE AND EXPLOSION HAZARDS : NONE

SUPER GRIME & GREASE GETTER
MATERIAL SAFETY DATA SHEET     BIG "G"	PAGE 2 OF 2

SECTION V : HEALTH HAZARD DATA	*

THRESHOLD LIMIT VALUE : HAS NOT BEEN ESTABLISHED.  SEE SECTION II. EFFECTS OF OVEREXPOSURE : DIZZINESS; REDDENING OF SKIN; ITCHING OF EYES RECOMMENDED EXPOSURE FOR ETHYLENE GLYCOL MONOETHYL ETHER IS 5 PPM-TWA, "SKIN NOTATION", ACGIH, 1982.  STUDIES IN EXPERIMENTAL ANIMALS HAVE PRODUCED DAMAGE TO THE SPERM-PRODUCING CELLS OF THE TESTICLE WHEN MALES WERE EXPOSED & BIRTH DEFECTS WHEN PREGNANT FEMALES WERE EXPOSED.  NO ADVERSE EFFECTS IN HUMANS HAVE BEEN REPORTED FROM USE OF THIS MATERIAL EMERGENCY FIRST AID PROCEDURES:

SKIN: WASH THOROUGHLY AFTER ANY EXPOSURE.

EYES: FLUSH WITH WATER, IF IRRITATION PERSISTS, CALL PHYSICIAN. INGESTION: CAN CAUSE GASTROINTESTINAL IRRITATION, NAUSEA, VOMITING, AND DIARRHEA.

DRINK LARGE VOLUMES OF MILK OR OTHER LIQUID, CALL PHYSICIAN IMMEDIATELY.

SECTION VI : REACTIVITY DATA

STABILITY DATA : STABLE

INCOMPATIBILITY : AVOID MIXING WITH ACIDS. HAZARDOUS DECOMPOSITION PRODUCTS : NONE HAZARDOUS POLYMERIZATION : WILL NOT OCCUR

SECTION VII : SPILL OR-LEAK PROCEDURES

SMALL SPILLS : WIPE UP PRODUCT, RINSE SOILED AREA WITH WATER DOWN

THE DRAIN. LARGE SPILLS : STOP LEAK AT THE SOURCE AND COLLECT INTO A SUITABLE

CONTAINER, THEN TREAT AS A SMALL SPILL.

FOLLOW STATE AND LOCAL REGULATIONS FOR DISPOSAL OF LARGE NON-RECLAIMABLE SPILLS.

SECTION VIII : SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION : NONE REQUIRED VENTILATION : MECHANICAL,  (GENERAL) PROTECTIVE GLOVES : FOR PROLONGED. USE EYE PROTECTION : SAFETY GLASSES OTHER PROTECTIVE EQUIPMENT : NONE REQUIRED

SECTION IX : SPECIAL PRECAUTIONS

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE :

ALWAYS STORE UNUSED PORTION IN ORIGINAL CONTAINER WITH CAP SECURE.

AVOID TEMPERATURE EXTREMES. OTHER PRECAUTIONS :

AS WITH ALL CHEMICALS, USE WITH DUE CARE ACCORDING TO LABEL

INSTRUCTIONS AND PRECAUTIONS.