From: Judy Stryjewshi National Chemical Labs (FaxRush) To; Klaclys Helms Sevan OeKs Supply Co

MATERIAL SAFETY DATA SHEET

(SIMILAR TO OSHA FORM 174)

N.A.=*Jol Applicable Not EsLcwoL E

SEgJION 1-MAHUFACTVSf K/PJ^TRJPUT-QR IN^QRMAIIO.bL

Product Name: Cyclone Intensive Ceramic Tile/Grout Cleaner	Product #;   2516

Supplisr's Name: National Chemical Laboratories o' PA, Inc.

Supplier's Address: 401 M. 10th Street • Philadelphia, PA 19123	Emergency Phone #:CH£M-TeL (800)-255-3924

HWUS Hazard Code:  Heslth:2   Flarpmability:Q   Reactivity:!}   Other;Q	Chemical Name; Alkaline Detergent

Chemical Family: Detergent Blend	Formula: N-A. Blended Product

O.O.T. Shipping Name: CORROSIVE LIQUID n,e>-s. (contains potassium hydroxide)

Hazard Clasjs: 8	_	_________	^D.OT, ID#:^JM1760

SECTION 2-HAZARDOUS INGREDIENTS

OHl-MICAL NAME

Polyethylene Mono (Nonylphenyl) Ether Glycols Potassium Hydroxide 2-Butoxy Ethanol Dodecyi-Benzenesulfonic Acid, Sodium Salt

CAS HO,

9016-45-9

1310-58-3

111-76-2

25155-30-0

% Concern

PROP, PROP, PROP. PROP,

Not Est. 2 mg/m" 25 ppm Not Est.

| SECTION 3-PHYSICAL DATA |  | SECTION 4-FIRE AND EXPLOSION DATA |
| --- | --- | --- |
| Boiling Point (F): 212° Vapor Pressure: -To Water Vapor Density; = To Water Solubility In Water: Complete Appearance: Clear, Thin, Green Liquid Odor: Buttery Citrus | Specific Gravity: 1.044 Evaporation Rate: = To Waier | Flash Point (F): None to boiling (TOC) Flammable Limits: N.A, Unusual Fire & Explosion Hazards; None |

SECTION 5-HEALTH HAZARD DATA

Routc(s) of Entry: lnhalacion:Ho   SkirrYes ' lnge!;tion:Yes	Threshold Limit Value: Not Est,

Effects of Overexposure: Causes skin and eye iritation. Harmful if swallowed. May cause severe rnucosal damage and gastric

distress. May cause systemic intoxication.

Emergency & FirstAid Procedures: ForEyes:   rlusn with water for at least fifteen minutes. Get medical aid- ForSkin: Thoroughly

wash with water and apply emollient creme.  If irritation persists get medical aid.   If Ingested: Do not induce vomiting.  Give milk, egg

white, gelatin, or waisr. Get immediate medical aid.

Conditions Aggravated by Exposure:   May aggravate existing skin conditions.

Listed Carcinogen, Mutagen, Teratogen (Ingred ent, Source, Amount}: None

| SECTION 6-REACTIV1TY DATA | SECTION 7-SPILL OR LE^K PROCEDURES |
| --- | --- |
| Stability: Stable Conditions to Avoid: None Incompatibility: Acids and Oxides Hazardous Dacornposition Products: Carbon Oiiidizere snci Amines l-Izzardous Polymerization: Will not occur. | Steps to be taken if material is released or spilled: Recontsiherize by mopping, we! vacuum, or suitable absorbent. Rinse remaining material to drains. Be cautious of slippery floors. Waste disposal method: Follow local, state, and federal regulations. |
| SECTION 8-SPECIAL PROTECTION INFORMATION | SECTION 9-SPECIAL PRECAUTIONS |
| Respiratory Protection: Generally not Required, Vgntilation: Local Protective Gloves: Rubber or latex. Eye Protection: Goggles or safety glasses with s. de shields Other Protective Equipment: Rubber boots should be worn if working in standing solution. | Precautions to be taken in handling and storage: Store between 40° and 120" F, Other Precautions: Keep out of reach of children. For sale to, use and storage by servioe personnel only. |

The information contained in this WISDS was obtained Trim curront and reliable sourc«. however the d?ta la provided without any warranty, expressed
or implied, regarding ita correctness or accuracy. Since the conditions or handling, storage* and disposal of This product arc beyond the control of this
company, it Is not responsible for loss, injury and expense arising ouiofthe product's improper use. No warranty, expressed or inferred, regarding thfr
product describee! in this MSOS shall bft created or infer-eri by any statement in this MSDS.  Various government agencies may have specific raguls-
lions regarding rhe transportation, handling, storage, uses or disposal of this product which may not be covered by this MSDS. The user is responsible
for Ml compliance.

OA.TE PREPA.RED-10/05/00   By; S-C.