FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS#:      285.00

Revision Date:      September 20, 2004

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Copper(ll) Sulfate

MSOS #:     285.00

Revision Date:   September 20, 2004

Section 9 — Physical and Chemical Properties

Section 1 — Chemical Product and Company Identification

Copper(II) Sulfate

Flinn Scientific, Inc. P.O.Box2i9 Batavia, 1L 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Blue crystalline powder, granules or larger crystals. Odorless. Solubility: Soluble in water and methanol; slightly in alcohol. Formula: CuSO4 5H20 Formula Weight: 249.69

Section 10 — Stability and Reactivity

Specific Gravity: 2.28

flinn Ar-A-glance

Section 2 — Composition, Information on Ingredients^

Copper(II) sulfate, penlahydrate

Synonym: Cupric sulfale, blue vitiol, chalcanthite

CAS#:  7758-99-8

Section 3 — Hazards Identification

ORL-RAT LD50: 300 mg/Kg

IHL-RATLC50:N.A.

SKN-RBTLD50:N.A.

Blue crystalline powder, granules or larger crystals. Odorless.

Skin and respiratory irritant; moderately toxic by ingestion and inhalation.

Health-2 Flammability-0 Reaclivity-0 Expos ure-2 Storage-1

0 is lov. hazard 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

tnhalalion: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. After vomiting, give mixture of 2 Tbs. of activated charcoal mixed

\\ith one cup of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable, non combustible solid.	NFPA code
However, sulfur trioxide can be produced ai temperatures above 653 C.	None

Fire Fighting Instructions: Use triciass, dry chemical fire extinguisher. Firefighters should wear PPE and	Established

SCBA with full facepiece operated in positive pressure mode.

Avoid contact with finely powdered metals, heal. Will corrode steel. Shelf Life: Fair, slowly effloresces in air.

Section 11 —Toxicolggical Information

Acute effects: Toxic, severe eye irritant, gastrointestinal disturbances Chronic effects: Possible mutagen Target organs: Liver, kidneys, blood

N.A. - Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not vet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26ais one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

Section 6 — Accidental Release Measures

^jct.uyn o — y-Mnuciiuti rvcujaac ntcnautca

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

TSCA-listed, EINECS-listed <231-847-6).

Section 16 — Other Information

Consult your cop;1 of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Ffinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites. thiosulfates and phosphates. Efflorescent. Protect from air, store in a Flinn Chem-Saf bag. Use and dispense in a hood.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8 — Exposure Controls, Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apro:

Flinn Chemical Packaging Prevents Accidents

ninn@ninnsci.com   www.fiinnsci.com P.O. Box 219       Batavia IL 60510 (800)452-1261   pax (866) 452-1436

© 2004 Fiinn Scientific, Inc. AH Rights Reserved.

page 1 of 2

© 2004 Fiinn Scientific, Inc. All Rights Reserve

page 2 of 2