Southern  Coatings P.O.  Box   160 Sumter,   SC     29150

EMERGENCY    PHONE   NO.       803-775-6351 INFORMATICS   PHONE   NO.   803-775-6351

H . M .  I  . S .

HEALTH	2

F LAMMAB I L I TY        2
REACTIVITY	0

These   ratings   should   be   used   only as   part   of   fully   implemented   H.M.I.S.   program.

MATERIAL    SAFETY     DATA    SHEET

SECTION

DATE   OF   PREPARATION     2/03/89

TRADE    NAME	POLY      EPOXY      CATALYST

MANUFACTURER   CODE   I.D.	714-5435

4 J - 3

- 2

SECTION  II  -  HAZARDOUS  INGREDIENTS

INGREDIENT

2-PROPOXYETHANOL

AROMATIC HYDROCARBONS

CAS    NO.

2807-30-9 64742-95-6

MFR

PPM    MG/CU.M.

25 NONE    ESTABLISHED

ALLOWABLE EXPOSURE   LEVEL

MPPCF

SKIN

SKIN

SARA	VP

313   mm   HG   6 20   DEG.C

SKIN  =  SKIN  ABSORPTION  MUST  BE  CONSIDERED  AS  A  ROUTE  OF  EXPOSURE

C-CEILING=  ALLOW.  EXPOSURE  LEVEL  SHOULD  NOT  BE  EXCEEDED  FOR  ANY  TIME  PERIOD

MFR  =  MANUFACTURER  RECOMMENDED  EXPOSURE  LIMIT

STEL  =  SHORT  TERM  EXPOSURE  LIMIT

X-SARA  313  =  CHEMICAL  IS  SUBJECT  TO  REPORTING  REQUIREMENTS  OF  SECTION  313

OF  TITLE  III  OF  S.A.R.A.  40  CFR  PART  372

SECTION III - HEALTH INFORMATION

EFFECTS   OF   SHORT   TERM   OVEREXPOSURE SWALLOWING

May  cause  gastrointestinal INHALATION

irritation.

May cause nose or  throat  irritation.  High  concentrations  may  cause acute ^~^  central  nervous  system depression  characterized  by  headaches,  dizziness, nauseaandconfusion.

May cause eye  irritation. SKIN

May  cause  defatting  and  irritation EFFECTS OF REPEATED OVEREXPOSURE

of      the     skin.

Reports  have associated prolonged and  repeated occupational  overexposure to solvents  with  permanent  brain and  nervous  system damage.  Intentional misuse by deliberately concentrating  and  inhaling  the contents  may be

SIGNIFICANT   LABORATORY   DATA' WITH   POSSIBLE   RELEVANCE   TO   HUMAN   HEALTH. None    currently     known

SECTION  IV  -  FIRST  AID  AND  EMERGENCY PROCEDURES

SWALLOWING

If  swallowed  do  not  induce  vomiting.  Call  poison  control  center,  hospital emergency room or  physician  immediately. INHALATION

Remove to fresh  air  immediately.    If  breathing  has  stopped,  give  artificial  respiration.   Keep warm and quiet.   Get  medical  attention  immediately.

EYE

Flush with  large amounts  of  water,  lifting  upper  and  lower  lids  occasionally.   Continue  for  at  least  15  minutes.   Get  medical  attention.

SKIN

Remove contaminated clothing.  Wash  affected area  with  soap and water. Obtain medical  attention  if  irritation persists. NOTES TO PHYSICIAN

Any  treatment  that  might  be  required  for  overexposure  should  be directed  at  the  control  of  symptoms  and  the  clinical  conditions.

SECTION V - PHYSICAL DATA

BOILING   RANGE	301         DEG.F.      TO     345         DEG.F.

VAPOR   DENSITY	Heavier    than    air.	%    VOLATILE    BY    VOLUME	6 3

EVAPORATION   RATE   Slower    than    ether.        VOC     4.66   Ib/gal    less v\ater  & NPRS*       559   g/l   less    water C A L C U L A T E D
WEIGHT   LB./GAL.      8.3	VOC     2.81     Ib/gal    solids	337   g/l   solids	CALCULATED

Negligibly     Photochemical   ly     Reactive     Materials

SECTION  VI  -  FIRE AND  EXPLOSION  DATA

NFPA   FLAMMABILITY    CLASSIFICATION

COMBUSTIBLE  LIQUID

CLASS  II

| CABARRUS 714-5435 | COUNTY SCHOOL |  |  | s |  |  | PAGE DATE | 3 2/03 | /89 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| •::• ::::"-::.•;;:•.::.::- :. .'::.. | .SECTION | XI | - | OTHER | INFORMATION; | (CONTINUED) |  |  |  |

THE  INFORMATION  CONTAINED  HEREIN  IS  BASED  ON  DATA  CONSIDERED  TO  BE ACCURATE.  WHILE  THE  INFORMATION  IS  BELIEVED  TO  BE  RELIABLE,  NO  WARRANTY  IS EXPRESSED  OR  IMPLIED  REGARDING  THE  ACCURACY  OF  THIS  DATA  OR  THE  RESULTS  TO f—•  BE  OBTAINED  FROM  THE  USE  THEREOF.  SINCE  THE  USE  OF  THIS  INFORMATION  AND  THE CONDITIONS    AND  USE  OF  THIS  PRODUCT  ARE  CONTROLLED  BY  THE  USER,  IT  IS  THE USER'S  OBLIGATION  TO  DETERMINE  THE  CONDITIONS  OF  SAFE  USE  OF  THE  PRODUCT.

CABARRUS COUNTY SCHOOLS MAINTANCE DEPT. P. O. BOX 388 CONCORD, N. C.