MATERIAL SAFETY DATA SHEET

PAI NUMBER: 86208, C6208

PRODUCT NAME: AERO-CHEM TEF-GEL LUBRICANT

TRADE NAME OR CHEMICAL NAME:

NFPA RATING:	HMIS RATING:

HEALTH:   2	HEALTH: 2	*'*

FLAMMABILITY: 4	FLAMMABELITY: 4

REACTIVITY: 0	REACTIVITY: 0

SPECLAL:	PERSONAL PROT:

******SECTION I****** MANUFACTURED FOR PARTS ASSOCIATES INC.

DISTRIBUTORS NAME:	PARTS ASSOCIATES INC.

DISTRIBUTOR'S ADDRESS:	12420 PLAZA DR.

CITY, STATE, ZIP:	PARMA, OH 44130

TELEPHONE NUMBER:	216-433-7700

EMERGENCY TELEPHONE:	(800) 255-3924 CHEM-TEL, INC.

DATE PREPARED: 04/03/92

***SECTION H: HAZARDOUS INGREDIENTS/IDENTITY INFO*** HAZARDOUS COMPONENTS.

INGREDIENTS

CAS#	OSHAPEL      ACGIHTLV      STEL     %(OPTIONAL)

HEXANE	50	50	43

110-54-3

PARAFFINIC PETROLEUM     5MG/M3         5MG/M3       10MG/M3        18 DISTILLATES 64742-65-0

*

ALIPHATIC HYDROCARBON	5MG/M3	2

MIXTURE 64742-63-8

PROPANE/ISOBUTANE/         1000         1000      ASPHYXIANT       25 N-BUTANE 74-98-6

*ZINC COMPOUND	5MG/M3	0.1-0.5

CAS UNKNOWN

2-BUTOXYETHANOL	25 (SKIN)   25 (SKIN)        .       0.1-0.5

111-76-2

PETROLATUM	NE	NE	NE	8

8009-03-8

NON-HAZARDOUS	3

^

*ALL CHEMICAL COMPOUNDS MARKED WITH AN ASTERISK (*) ARE TOXIC CHEMICALS SUBJECT TO THE REPORTING REQUIREMENTS OF SECTION 313 OF TITLE Id OF THE SUPER-FUND AMENDMENTS AND REAUTHORIZATION ACT (SARA) OF 1986 AND 40 CFR PART 372. YOU MUST NOTIFY EACH PERSON TO WHOM THIS MIXTURE OR TRADE NAME PRODUCT IS SOLD. THIS STATEMENT MUST REMAIN A PART OF THIS MATERIAL SAFETY DATA SHEET.

***SECTION HI: PHYSICAL/CHEMICAL CHARACTERISTICS***

•

BOILING POINT: -40F TO >500F

SPECIFIC GRAVITY (H20 = 1); 0.7

VAPOR PRESSURE (PSIG @ 70F): ND

MELTING POINT: LIQUID NA

VAPOR DENSITY (AIR = 1) (HEAVIER THAN AIR): 4.0

% VOLATILES BY ORGANIC BY WEIGHT:

EVAPORATION RATE (BUTYL ACETATE =1): >1

WEIGHT PER GALLON:

SOLUBILITY IN WATER: NIL

APPEARANCE AND COLOR: CLEAR SOLVENT

***SECTION IV: FIRE AND EXPLOSION HAZARD DATA***

FLASH POINT (METHOD USED): -40F TCC FLAMMABLE LIMITS: LEL: 1.8   UEL: 12.0 FLAMMDBILITY CLASSIFICATION: OSHA: DOT:

EXTINGUISHING MEDLA: USE WATER FOG, DRY CHEMICAL OR CARBON DIOXIDE.

SPECIAL FIRE FIGHTING PROCEDURES: AEROSOL CANS MAY RUPTURE WHEN HEATED.

UNUSUAL FIRE AND EXPLOSION HAZARDS: HEATED CANS MAY BURST.

***SECTION V: HEALTH HAZARD*** ROUTES OF ENTRY: INHALATION: YES SKIN: YES INGESTION: YES

HEALTH HAZARDS (ACUTE AND CHRONIC): MAY CAUSE DIZZINESS OR NARCOSIS IN HIGH VAPOR CONCENTRATIONS. WILL CAUSE DEFATTING OF SKIN. EFFECTS ARE REVERSIBLE LONG TERM EXPOSURE (YEARS) TO HIGH CONCENTRATIONS OF VAPOR MAY CAUSE LUNG, LIVER, OR KIDNEY DAMAGE. THE SOLVENTS LISTED HAVE BEEN REPORTED TO AFFECT THE CENTRAI^NERVOUS SYSTEM. EYE AND SKIN IRRITANT. ASPIRATION HAZARD IF SWALLOWED. MAY IRRITATE NASAL AND RESPIRATORY PASSAGES. SWALLOWING MAY CAUSE IRRITATION OF MOUTH, ESOGHAGUS, AND GASTROINTESTINAL SYSTEM, AND MAY BE FATAL. HEXANE MAY DAMAGE PERIPHERAL NERVE TISSUE. 2-BUTOXETHANOL MAY CAUSE CORNEAL INJURY AND BLOOD ABNORMALITIES AND MAY BE ABSORBED THROUGH THE SKIN WITH TOXIC EFFECTS.

CARCINOGENICITY: PRESENTLY NOT ON ANY LIST.

NTP:

LARC MONOGRAPHS:

OSHA REGULATED:

SIGNS AND SYMPTOMS OF EXPOSURE: ...INHALATION - DIFFICULTY IN BREATHING. ...SKIN - REDNESS. ...INGESTION - VOMITING.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE: HEART DISEASE; RESPIRATORY DISORDERS.

EMERGENCY AND FIRST AID PROCEDURES: GIVE OXYGEN - DO NOT INDUCE VOMITING - GASTRIC LAVAGE - WASH EYES AND SKIN WITH WATER.

***SECTION VI: REACTIVITY DATA***

STABILITY: STABLE

CONDTIONS TO AVOID: HIGH TEMPERATURES

INCOMPATIBILITY (MATERIALS TO AVOID): INCOMPATIBLE WITH STRONG

OXDDTZERS, ALKALI METALS, INTERHALOGEN COMPOUNDS.

HAZARDOUS DECOMPOSITION PRODUCTS: IN FIRE, WELL DEOMPOSE TO THE

FOLLOWING: CARBON DIOXIDE, WATER, HYDROGEN FLUORIDE AND PARFLUORO

HYDROCARBONS, SULFUR COMPOUNDS, CALCIUM OXIDE, ALDEHYDES, AND

KETONES.

HAZARDOUS POLYMERZATION: WILL NOT OCCUR

CONDITIONS TO AVOID: NONE

***SECTION VH: SPDLL OR LEAK PROCEDURES***

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED: USE ABSORBANT SWEEPING COMPOUND TO SOAK UP MATERIAL. PUT INTO CONTAINER. DISPOSE AS HAZARDOUS WASTE.

WASTE DISPOSAL METHOD: DISPOSE AS HAZARDOUS WASTE IN ACCORDANCE WITH EPA RCRA

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORING: KEEP AWAY FROM HEAT, SPARKS, OR OPEN FLAME. STORE AT TEMPERATURES BELOW 120F.

OTHER PRECAUTIONS: WHEN SPRAYING MORE THAN ONE HALF CAN CONTINUOUSLY OR MORE THAN ONE CAN CONSECUTIVELY, USE NIOSH APPROVED RESPIRATOR.

***SECTION Vffl: SPECIAL PROTECTION INFORMATION***

RESPIRATORY PROTECTION (SPECIFY TYPE): SELF-CONTAINED BREATHING APPARATUS IF ABOVE TLV LIMIT EXCEEDING.

VENTILATION: LOCAL EXHAUST: YES

PROTECTIVE GLOVES: NOT REQUIRED IF SPRAYING

EYE PROTECTION: WEAR EYE PROTECTION

OTHER PROTECTIVE CLOTHING OR EQUIPMENT: LONG SLEEVES AND LONG PANTS

WORK/HYGENIC PRACTICES: DO NOT SMOKE WHILE USING. WASH HANDS AFTER USE.

***SECTION DC: DISCLAIMER ***

THIS INFORMATION IS BELIEVED TO BE ACCURATE AND REPRESENTS THE BEST INFORMATION CURRENTLY AVAILABLE TO US. HOWEVER, WE MAKE NO WARRANTY

OF MERCHANT ABILITY OR ANY OTHER WARRANTY, EXPRESSED OR IMPLIED, WITH RESPECT TO SUCH INFORMATION, AND WE ASSUME NO LIABILITY RESULTING FROM ITS USE. USERS SHOULD MAKE THEIR OWN INVESTIGATION TO DETERMINE THE SUIT ABILITY OF THE INFORMATION FOR THEIR PARTICULAR PURPOSES.