punn scientific me.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS#:     803.00

Revision Date:    March 12, 2001

punn scientific inc.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Tannic Acid

MSDS#:     803.00

Revision Date:   March 12, 2001

Section 1 —Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

Tannic Acid

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Pale yellow powder. Slight cucumber odor. Solubility: Water and alcohol. Formula: C76H52O46 Formula Weight: 1701.18

Melting Point: decomposes at 210 C

Section 2— Composition. Information on Ingredients

Tannic Acid

CAS#:   1401-55-4

Section 10 — Stability and Reactivity

Avoid contact with strong oxidtzers, and strong

Shelf Life: Indefinite, substance colors on exposure to air.

Section 3 — Hazards Identification

ORL-RAT LD50: 2260 r IHL-RAT LC50: N.A. SKN-RBTLD50:N.A.

Pale yellow powder. Slight cucumber odor.

Siightiy toxic by ingestion. Body tissue irritant and possible allergen. Avoid all body tissue contact.

Combustible solid.

flinn at-a-glance

Health-1 Flammability-l Reactivity-0 Exposure-t Storage-Q

0 is low hazard, 3 is high hazard

Section 11 —Toxicoloqical Information

Acute effects: Irritant, nausea, abdominal pain Chronic effects: N.A. Target organs: Liver,kidneys

N.A. = Not available, not all health aspects of ihis substance have been fully investigated.

Section 12 — Ecological Information

Section 4 — First Aid Measures

Call a physician, seek medical attention for funher treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Combustible solid.	NFPA code

Flash Point: 390 F; Autoignition Temperature: 980 F	H-0

Fire Fighting Instructions: Use tnclass, dry chemical fire extinguisher. Firefighters should wear PPE         F-l
and SCBA with full facepiece operated in positive pressure mode.	R-0

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number; N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (215-753-2).

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Organic HI. Store with acids, anhydrides and peracids. Store in a cool, dry place. Use and dispense in a hood.

Section 16 — Other Information

Consult your copy of the Fiinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Fhnn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing, Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations below exposure limits.

Need a Chemical Fast?--Order from Flinn

flinn®fiinnsci.com   www.dinnsci.com

P.O. 8ox2I9   Batavia IL 60510 (800)452-1261  Fax (866) 452-1436

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

O 2002 Flinn Scientific. Inc. All Rights Reserved.