jul l o iaau

942854	^MATERIAL SAFETY DATA SHEET*	Page 1 of 2 -

• r

MEDICAL EMERGENCY ONLY, 24 HOUR SERVICE:  1-800-328-0026

ECOLAB INC  Ecolab Center        Product Information:  1-612-293-2233
St. Paul MN 55102	Date of Issue:  February 12, 1990

- IDENTIFICATION /

- Product Name:  SUPER TRUMP

- Product Type:  High Alkaline Liquid Automatic Dishmachine Detergent

++ Section 2 Provides SARA Section 313 Reporting Information + +

2.0	HAZARDOUS COMPONENTS /	Air Limits (mg/m3)

1   TWA Other

2.1	Sodium hydroxide   (caustic  soda)   1310-73-2	25         2  C         2  C

This   product   contains  no   other  component   considered  hazardous according  to   the   criteria  of  29   CFR  1910.1200.

- PHYSICAL  DATA"T~'	~       "     ~	~~~	—

- Appearance:     Clear yellow  liquid

- Solubility   in Water:      Complete

.3.3  PH:      1002  =   13.0-13.5;   0.22  =   11.5-12.5
3.4-'Boiling  Point:     >212  deg F	Specific  Gravity:     1.25-1.30

4.0	-FIRE AND "EXPLOSION DATA  /	'

- Special Fire Hazards:  None

- Fire Fighting Methods:  Product does not support combustion.

5.0 REACTIVITY     ~         ^	~

- Stability:  Stable under normal conditions of hardling.

- Conditions to Avoid:  Do not mix with anything but water.  Reacts
violently with acids.  Reacts with soft metals such as aluminum and
zinc.

- SPILL OR LEAK PROCEDURES 7     USE PROPER PROTECTIVE EQUIPMENT  ~

- Cleanup:  Rinse small amounts to drain where possible.  Dike or dam
large spills; pump to containers or soak up on inert absorbent.
Flush residue to sanitary sewer; rinse area thoroughly.

- Waste Disposal:  Unused product" as a waste is Corrosive (D002) by
RCRA criteria.  Consult state and local authorities for
restrictions on disposal of chemical waste.

UNK = Unknown at this time     PEL = Permissible Exposure Limit TWA = OSHA 8 Hour Average      STEL = 15 Minute Average C = Ceiling Limit, Not To Be Exceeded

Product:  SUPER TRUMP	Page 2 of 2

ECOLAB INC   Ecolab Center	942854

MEDICAL EMERGENCY ONLY, 24 HOUR SERVICE:  1-800-32.8-0026

- HEALTH HAZARD DATA /	DANGER

- Effects of Overexposure:

Skin and Eyes: CAUSES SEVERE CHEMICAL BURNS. Eye contact may cause blindness. Harmful skin contact may not cause immediate pain.

++ Immediate water flushing is vital in case of eye contact. ++ If Swallowed:  HARMFUL OR FATAL.  Causes chemical burns of mouth, throat and stomach.

- FIRST AID /~~~~~~

- Eyes;  Immediately flush with plenty of cool running water.  Remove
contact lenses.  Continue flushing for at least 15 minutes, holding
eyelids apart to ensure rinsing of the entire eye.

CALL A PHYSICIAN IMMEDIATELY.

- Skin:  Immediately flush skin with plenty of cool running water for
at least 15 minutes while removing contaminated clothing and shoes.
Wash clothing before reuse.

- If Swallowed:  Rinse mouth at once; then drink 1 or two large

. glasses of water or milk.  DO NOT induce vomiting.  NEVER give anything by mouth to an unconscious person.

CALL^A POISON CONTROL CENTER OR PHYSICIAN IMMEDIATELY

- SPECIAL PROTECTION INFORMATION""?     		'

- Eyes:  Splashproof glasses, goggles or face shield.

- Skin:  Rubber gloves - protective cuff or gauntlet type preferred.
9-3 Other:  Avoid contact with use solutions.  These may also be

hazardous.

- ADDITIONAL INFORMATION/PRECAUTIONS /	:         ~"~

- DOT Class:  Alkaline Liquid N.O.S. (Sodium Hydroxide), NA1719 /

Corrosive

10.2	KEEP FROM FREEZING.

KEEP OUT OF REACH OF CHILDREN

The above information is believed to be correct with respect to the formula used to manufacture the product.  As data, standards and regulations change, and conditions of use and handling are beyond our control, NO WARRANTY, EXPRESS OR IMPLIED, IS MADE AS TO THE COMPLETENESS OR CONTINUING ACCURACY OF THIS INFORMATION.

W9