Revised

November 1980

MATERIAL SAFETY DATA SHEET

(Approved by U.S. Department of Labor "Essentially Similar" to Form LSB-OOS-4)

NPVLA6-70

879-70

Section I

MANUFACTURER'S NAME

Hysan Corporation

STREET ADDRESS

919 West  38th Street

CITY, STATE. AND Z\P CODE

Chicago,   IL    60609

EMERGENCY TELEPHONE NO.       312/376-8900

CHEMICAL NAME AND SYNONYMS

N.A.

TRADE NAME

Hot  Rod/Mr.Mac

CHEMICAL FAMILY

Acidic Drain Line Maintainer

FORMULA

N.A.

NAZARDOUS INGREDIENT^

PAINTS, PRESERVATIVES, & SOLVENTS

PIGMENTS

tlv (Units)

SOLVENTS

TLV (Unlti)

CATALYST

ADDITIVES

-IICLE

OTHERS

HAZARDOUS MIXTURES OF OTHER LIQUIDS, SOLIDS, OR GASES

tlv

(Units)

Sulfuric Acid

C.A.S.  No.   7664-93-9

1 mg

Section III-PHYSICAL DATA

(ON  CONCENTRATE)"

BOILING POINT CF.)

IBP  540 F

SPECIFIC GRAVITY (H,O=1)

1.83

VAPOR PRESSURE (mm Hg.)

0.001 @ 20 C

PERCENT  VOLATILE BY VOLUME (%)

N.A.

VAPOR DENSITY (AIR=1)

SOLUBILITY IN WATER

3.4

Miscible

EVAPORATION  RATE

.

Under 1.0

APPEARANCE AND ODOR

Clear, red liquid having a pungent odor.

Section IV - FIRE AND EXPLOSION HAZARD DATA

FLASH POINT (METHOD USED)	o

EXTINGUISHING MEDIA

None

Dry Chemical

FLAMMABLE LIMITS N.A.

•SPECIAL FIRE FIGHTING PROCEDURES

Water applied directly to sulfuric acid causes evolution of heat and splattering.

UNUSUAL FIRE AND EXPLOSION HAZARDS	Highly reactive & capable of igniting finely divided combustible.

Revised  November 1986

879-70

SECTION V - Health Hazard Data

THRESHOLD LIMIT VALUE

1 mg/m3

EFFECTS_,OF OVEREKPOSURE	#

Danger – Poison – Causes severe burns.  Sulfuric acid mist severly irritates the eyes, respiratory tract, and skin.  Concentrated sulfuric acid destroys tissue due to its severe dehydrating action.

EMERGENCY AND FIRST AID PROCEDURES

Eyes – Wash eyes at once with large amounts of water lifting lower and upper lids occasionally.  Call a physician.  Skin – Flush with water.  Call a physician.  Ingestion – If conscious give large amount of water at once.  DO NOT INDUCE VOMITING. Call a physician.

SECTION VI - Reactivity Data

STABILITY

UNSTABLE   STABLE  X

CONDITIONS TO AVOID

u

Avoid heating to abnormal temperatures.

INCOMPATABILITY:  Avoid contact with organic chlorates, carbides, fulminates, picrates and strong alkalis.

HAZARDOUS DECOMPOSITION PRODUCTS

Oxides of carbon and nitrogen formed upon combustion

formed upon  combustion.

HAZARDOUS POLYMERIZATION

MAY OCCUR

WILL NOT OCCUR X

CONDITIONS TO AVOID

NONE

SECTION VII - SPILL or LEAK PROCEDURES

•teps TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED        _	-

Wear protective equipment and clothing before entering area.  Ventilate area.  Absorb in an absorbent, dry sand or earth or dilute with water.

WASTE DISPOSAL METHOD

Place used absorbent in sealed containers and dispose of in a secured sanitary landfill.  Or, dilute with water and neutralize, then flush to sewer.

| SECTION VIII – Special Protection Information |  |  |
| --- | --- | --- |
| RESPIRATORY PROTECTION (Specify type) None Required |  |  |
| VENTILATION | LOCAL EXHAUST Hood or Fan | SPECIAL rf N.A. |
|  | MECHANICAL (General) Maintain Adequate Ventilation | OTHER N.A. . |
| PROTECTIVE GLOVES EYE PRO! Rubber or Polyethylene Gloves |  | ECTION Use Face Shield |
| OTHER PROTECTIVE EQUIPMENT Impervioius clothing or apron |  |  |
|  |  | - |

Section IX - SPECIAL PRECAUTIONS

PRECAUTIONS TO BE TAKEN  IN HANDLING  AND  STORING

Store in a dry, cool area.  Keep out of reach or children.

OTHER PRECAUTIONS  READ ENTIRE LABEL BEFORE USING.