MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page 001

Date Prepared: 01/14/02 Date Printed:  03/26/02 MSDS No: 503.0177148-008.005 fROIL WINDSHIELD DEICER 12/11.5 OZ

1-.   CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Material Identity

Product Name: PYROIL WINDSHIELD DEICER 12/11.5 OZ

General or Generic ID: SOLVENT BLEND

Company	Telephone Numbers

The Valvoline Company	Emergency:      1-800-274-5263

P.O. Box 14000

Lexington, KY  40512	Information:    1-859-357-7206

2.    COMPOSITION/INFORMATION ON INGREDIENTS

Ingredient(s)	CAS Number     % (by weight)

METHYL ALCOHOL	67-56-1       62.0

ETHYLENE GLYCOL	107-21-1         6.0

CARBON DIOXIDE	124-38-9     0.0-  8.0

;/	•-	_				       			|

'	~      	 '    '	~		"   "	"

3.    HAZARDS IDENTIFICATION Potential Health Effects

Eye

Can cause eye irritation.

Skin

May cause mild skin irritation.  Prolonged or repeated contact may dry and crack the skin.  Although rare, skin contact with ethylene glycol may cause allergic skin reaction (delayed skin rash which may be followed by blistering, scaling and other skin effects). Passage through the skin may add to toxic effects from breathing or swallowing.

Swallowing

Swallowing this material may be harmful.

Continued On Next Page

MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page 002

Date Prepared: 01/14/02 Date Printed:  03/26/02 MSDS No: 503.0177148-008.005 WINDSHIELD DEICER 12/11.5 OZ

Inhalation

Breathing of vapor or mist is possible.  Breathing small amounts of this material during normal handling is not likely to cause harmful effects.  Breathing large amounts may be harmful.

Symptoms of Exposure

stomach or intestinal upset (nausea, vomiting, diarrhea), irritation (nose, throat, airways), central nervous system excitation (giddiness, liveliness, light-headed feeling) followed by central nervous system depression (dizziness, drowsiness, weakness, fatigue, nausea, headache, unconsciousness) and other central nervous system effects, involuntary eye movement, visual impairment (including blindness), and death.

Target Organ Effects

OvQrexposure to this material (or its components) has been suggested as a cause of the following effects in laboratory animals, and may aggravate preexisting disorders of these organs in humans:  liver abnormalities, spleen damage, nervous system damage, eye damage, lung damage, brain damage, kidney damage, visual impairment, kidney damage, liver damage.

velopmental Information

Ethylene glycol has caused birth defects in animal studies at high oral doses.

Cancer Information No data

Other Health Effects No data

Primary Houte(s) of Entry

Inhalation, Skin absorption, Skin contact.

4.    FIRST AID MEASURES

Eyes

If symptoms develop, immediately move individual away from exposure and into fresh air.  Flush eyes gently with water for at least 15 minutes while holding eyelids apart; seek immediate medical attention.

Continued On Next Page

MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page 003

Date Prepared: 01/14/02
^	Date Printed:  03/26/02

MSDS Nor 503.0177148-008.005 WINDSHIELD DEICER 12/11.5 OZ

Skin

Remove contaminated clothing.  Wash exposed area with soap and water.  If symptoms persist, seek medical attention.  Launder clothing before reuse.

Swallowing

Seek medical attention.  If individual is drowsy or unconscious, do not give anything by mouth; place individual on the left side with the head down.  Contact a physician, medical facility, or poison control center for advice about whether to induce vomiting. If possible, do not leave individual unattended.

Inhalation

If symptoms develop, immediately move individual away from exposure and into fresh air.  Seek immediate medical attention; keep person warm and quiet.  If person is not breathing, begin artificial respiration.  If breathing is difficult, administer oxygen.

Note to Physicians

This product contains methanol which can cause intoxication and central nervous system depression.  Methanol is metabolized to formic acid and formaldehyde.  These metabolites can cause metabolic acidosis, visual disturbances and blindness.  Since metabolism is required for these toxic symptoms, their onset may be delayed from 6 to 30 hours following ingestion.  Ethanol competes for the same metabolic pathway and has been used to prevent methanol metabolism.  Ethanol administration is indicated in symptomatic patients or at blood methanol concentrations above 20 ug/dl.  Methanol is effectively removed by hemodialysis.  This product contains ethylene glycol.  Ethanol decreases the metabolism of ethylene glycol to toxic metabolites.  Ethanol should be administered as soon as possible in cases of severe poisoning since the elimination half-life of ethylene glycol is 3 hours.  If medical care will be delayed several hours, give the patient three to four 1-ounce oral "shots" of 86-proof or higher whiskey before or during transport to the hospital.  Fomepizole (4-methylpyrazole} is an effective antagonist of alcohol dehydrogenase, and as such, may be used as an antidote in the treatment of ethylene glycol poisoning.  Hemodialysis effectively removes ethylene glycol and its metabolites from the body. Effects of acute ethylene glycol poisoning appear in three fairly distinct stages.  The initial stage occurs shortly after exposure, lasts 6-12 hours, and is characterized by central nervous system effects (transient exhilaration, nausea, vomiting, and in severe cases, coma, convulsions, and possible death.  The second stage

Continued On Next Page

MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page 003

Date Prepared: 01/14/02
^	Date Printed:  03/26/02

MSDS Nor 503.0177148-008.005 WINDSHIELD DEICER 12/11.5 OZ

Skin

Remove contaminated clothing.  Wash exposed area with soap and water.  If symptoms persist, seek medical attention.  Launder clothing before reuse.

Swallowing

Seek medical attention.  If individual is drowsy or unconscious, do not give anything by mouth; place individual on the left side with the head down.  Contact a physician, medical facility, or poison control center for advice about whether to induce vomiting. If possible, do not leave individual unattended.

Inhalation

If symptoms develop, immediately move individual away from exposure and into fresh air.  Seek immediate medical attention; keep person warm and quiet.  If person is not breathing, begin artificial respiration.  If breathing is difficult, administer oxygen.

Note to Physicians

This product contains methanol which can cause intoxication and central nervous system depression.  Methanol is metabolized to formic acid and formaldehyde.  These metabolites can cause metabolic acidosis, visual disturbances and blindness.  Since metabolism is required for these toxic symptoms, their onset may be delayed from 6 to 30 hours following ingestion.  Ethanol competes for the same metabolic pathway and has been used to prevent methanol metabolism.  Ethanol administration is indicated in symptomatic patients or at blood methanol concentrations above 20 ug/dl.  Methanol is effectively removed by hemodialysis.  This product contains ethylene glycol.  Ethanol decreases the metabolism of ethylene glycol to toxic metabolites.  Ethanol should be administered as soon as possible in cases of severe poisoning since the elimination half-life of ethylene glycol is 3 hours.  If medical care will be delayed several hours, give the patient three to four 1-ounce oral "shots" of 86-proof or higher whiskey before or during transport to the hospital.  Fomepizole (4-methylpyrazole} is an effective antagonist of alcohol dehydrogenase, and as such, may be used as an antidote in the treatment of ethylene glycol poisoning.  Hemodialysis effectively removes ethylene glycol and its metabolites from the body. Effects of acute ethylene glycol poisoning appear in three fairly distinct stages.  The initial stage occurs shortly after exposure, lasts 6-12 hours, and is characterized by central nervous system effects (transient exhilaration, nausea, vomiting, and in severe cases, coma, convulsions, and possible death.  The second stage

Continued On Next Page

MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page	005

Date	Prepared: 01/14/02

Date	Printed:  03/26/02

MSDS	No: 503.0177148-008.005

jrSTROIL WINDSHIELD DEICER 12/11.5 OZ

Fire Fighting Instructions

Wear a self—contained breathing apparatus with a full facepiece operated in the positive pressure demand mode with appropriate turn—out gear and chemical resistant personal protective equipment-  Refer to the personal protective equipment section of this MSDS.

NFPA Rating

Health - 1, Flammability - 3, Reactivity - 0

6.    ACCIDENTAL RELEASE MEASURES

Small Spill

Absorb liquid on vermiculite, floor absorbent, or other absorbent material and transfer to hood.

Large Spill

Eliminate all ignition sources (flares, flames including pilot lights, electrical sparks). Persons not wearing protective

s~*.   equipment should be excluded from area of spill until clean—up has been completed. Stop spill at source. Prevent from entering drains, sewers, streams or other bodies of water.  Prevent from spreading.  If runoff occurs, notify authorities as required. Pump or vacuum transfer spilled product to clean containers for recovery.  Absorb unrecoverable product. Transfer contaminated absorbent, soil and other materials to containers for disposal. Prevent run-off to sewers, streams or other bodies of water. If run-off occurs, notify proper authorities as required, that a spill has occured.

7.    HANDLING AND STORAGE

Handling

Containers of this material may be hazardous when emptied.  Since emptied containers retain product residues (vapor, liquid, and/or solid), all hazard precautions given in the data sheet must be observed.

Continued On Next Page

MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page 006

Date Prepared: 01/14/02
^	Date Printed:  03/26/02

USDS No: 503.0177148-008.005 PYROIL WINDSHIELD DEICER 12/11.5 OZ

Storage

Not applicable

8.    EXPOSURE CONTROLS/PERSONAL PROTECTION

Eye Protection

Not required under normal conditions of use.

Skin Protection

Not required under normal conditions of use.. Other protective equipment: not required under normal conditions of use..

Respiratory Protections

Not required under normal conditions of use.

Engineering Controls

Not required under normal conditions of use.

Exposure Guidelines -Component

METHYL ALCOHOL (67-56-1) OSHA VPEL 200.000 ppm - TWA ((Skin)) OSHA VPEL 250.000 ppm - STEL ((Skin)) ACGIH TLV 200.000 ppm - TWA ((Skin)) ACGIH TLV 250.000 ppm - STEL ((Skin))

ETHYLENE GLYCOL (107-21-1)

OSHA VPEL 50.000 ppm - Ceiling

ACGIH TLV 100.000 mg/m3 - Ceiling as an aerosol

CARBON DIOXIDE (124-38-9) OSHA VPEL 10000.000 ppm - TWA OSHA VPEL 30000.000 ppm - STEL ACGIH TLV 5000.000 ppm - TWA ACGIH TLV 30000.000 ppm - STEL

Continued On Next Page

MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page 007

Date Prepared: 01/14/02 Date Printed: 03/26/02 MSDS No: 503.0177148-008.005

*"YROIL WINDSHIELD DEICER 12/11.5 OZ

9 . .   PHYSICAL AND CHEMICAL PROPERTIES

Boiling Point

(for component) 147.0 F (63.8 C) § 760 mmHg

Vapor Pressure

(for component) 97.680 mmHg @ 68.00 F

Specific Vapor Density

>  1.000        § AIR=1

Specific Gravity

.875 - .895 8 77.00 F

Liquid Density

7.160 Ibs/gal § 77.00 F .885 kg/1 ? 25.00 C

Percent Volatiles (Including Water) 85.0       - 100.0      %

>*-'->litile Organic Compounds (VOC) (Maximum) 606.000 g/1 5.060 Ibs/gal

Evaporation Rate

FASTER THAN ETHYL ETHER

Appearance CLEAR

State

LIQUID

Physical Form No data

Color

COLORLESS

Continued On Next Page

MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page	008

Date	Prepared: 01/14/02

^_^	Date	Printed:  03/26/02

MSDS	No: 503.0177148-008.005
PYROIL WINDSHIELD DEICER 12/11.5 OZ

Odor

No data

pH

Not applicable

Flame Propagation <  18.000 IN

10.   STABILITY AND REACTIVITY

Hazardous Polymerization

Product will not undergo hazardous polymerization.

Hazardous Decomposition

May form:  carbon dioxide and carbon monoxide.

Chemical Stability

Stable.

.icompatibility

Avoid contact with:  strong oxidizing agents.

11.  TOXICOLOGICAL INFORMATION No data

12.  ECOLOGICAL INFORMATION No data

13.  DISPOSAL CONSIDERATION

Waste Management Information

Destroy by liquid incineration.  Dispose of in accordance with all applicable local, state and federal regulations.

Continued On Next Page

MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page	009

Date	Prepared: 01/14/02

Date	Printed:  03/26/02

MSDS	No: 503.0177148-008.005

PYROIL WINDSHIELD DEICER 12/11.5 OZ

14.  TRANSPORT INFORMATION

DOT Information - 49 CFR 172.101 DOT Description:

AEROSOLS,2.1,UN 1950

Container/Mode:

CASES/SURFACE - NO EXCEPTIONS

NOS Component: None

RQ (Reportable Quantity) - 49 CFR 172.101 Product Quantity (Ibs) Component

8071         METHANOL

15.  REGULATORY INFORMATION

~~ Federal Regulations

TSCA (Toxic Substances Control Act) Status

TSCA (UNITED STATES) The intentional ingredients of this product are listed.

CERCLA RQ - 40 CFR 302.4

Component	Component

METHYL ALCOHOL	5000

ETHYLENE GLYCOL	5000

SARA 302 Components - 40 CFR 355 Appendix A None

Section 311/312 Hazard Class - 40 CFR 370.2

Immediate(X)   Delayed(X)   Fire(X)   Reactivef )    Sudden Release of Pressure( )

Continued On Next Page

MATERIAL SAFETY DATA SHEET

The Valvoline Company	Page	010

Date	Prepared: 01/14/02

S~^	Date	Printed:  03/26/02

MSDS	No: 503.0177148-008.00!
PYROIL WINDSHIELD DEICER 12/11.5 OZ

SARA 313 Components - 40 CFR 372.65

Section 313 Component(s)	,         CAS Number

METHANOL	67-56-1

ETHYLENE GLYCOL	107-21-1

International Regulations Inventory Status

Not determined

/

State and Local Regulations

California Proposition 65 None

New Jersey RTK Label	Information

METHYL ALCOHOL	67-56-1

ETHYLENE GLYCOL	107-21-1

CARBON DIOXIDE	124-38-9

Pennsylvania RTK Label	Information

METHANOL	67-56-1

1,2-ETHANEDIOL	107-21-1

CARBON DIOXIDE	124-38-9

16.  OTHER INFORMATION

The information accumulated herein is believed to be accurate but is not warranted to be whether originating with the company or not. Recipients are advised to confirm in advance of need that the information is current, applicable, and suitable to their circumstances.

Last Page URGE:ASHLAND INC WTR     EASYWTR