230 10 00

MATERIAL SAFETY DATA SHEET

Section 1 — PRODUCT AND COMPANY IDENTIFICATION

PRODUCT NUMBER 230

PRODUCT NAME

MINWAX* WOOD FINISH*, Early American MANUFACTURER'S NAME

MINWAX Company

10 Mountainview Road

Upper Saddle River, NJ 07458 DATE OF PREPARATION

17-AUG-06

HMIS CODES Health

Flammability Reactivity

EMERGENCY TELEPHONE NO. (216) 566-2917

INFORMATION TELEPHONE NO. (800) 523-9299

2 2

0

Section 2 — COMPOSITION/INFORMATION ON INGREDIENTS
% by WT      CAS No.  INGREDIENT	UNITS	VAPOR PRESSURE

55

2 mm 1 mm

64742-88-7

Mineral Spirits

ACGIH TLV    100  ppm

OSHA PEL    100  ppm Mineral Spirits (Odorless)

5    64741-65-7

ACGIH TLV    100  ppm

OSHA PEL    100  ppm Heavy Naphthenic Petroleum Oil

8     64742-52-5

ACGIH TLV     5 mg/m3 as Mist

OSHA PEL     5 mg/m3 as Mist Highly refined Naphthenic Oil

64742-53-6

ACGIH TLV  "  5 mg/m3 as Mist

OSHA PEL     5 mg/m3 as Mist

Section 3 — HAZARDS IDENTIFICATION

ROUTES OF EXPOSURE

INHALATION of vapor or spray mist.

EYE or SKIN contact with the product, vapor or spray mist. EFFECTS OF OVEREXPOSURE

EYES:  Irritation.

SKIN:  Prolonged or repeated exposure may cause irritation.

INHALATION:  Irritation of the upper respiratory system.

May cause nervous system depression.  Extreme overexposure may result in unconsciousness and possibly death. SIGNS AND SYMPTOMS OF OVEREXPOSURE

Headache, dizziness, nausea, and loss of coordination are indications of excessive exposure to vapors or spray mists.

Redness and itching or burning sensation may indicate eye or excessive skin exposure. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE

None generally recognized. CANCER INFORMATION

For complete discussion of toxicology data refer to Section 11.

Continued on page 2

230	page 2

Section 4 — FIRST AID MEASURES

EYES:  Flush eyes with large amounts of water for 15 minutes.

Get medical attention. SKIN:  Wash affected area thoroughly with soap and water.

Remove contaminated clothing and launder before re-use. INHALATION:  If affected, remove from exposure.  Restore breathing.

Keep warm and quiet. INGESTION:  Do not induce vomiting.

Get medical attention immediately.

Section 5 — FIRE FIGHTING MEASURES

FLASH POINT	LEL      UEL

105 F PMCC	1.0      7.0

FLAMMABILITY CLASSIFICATION

Combustible, Flash above 99 and below 200 F EXTINGUISHING MEDIA

Carbon Dioxide, Dry Chemical, Foam UNUSUAL FIRE AND EXPLOSION HAZARDS

Closed containers may explode when exposed to extreme heat.

Application to hot surfaces requires special precautions.

During emergency conditions overexposure to decomposition products may cause a health hazard.  Symptoms may not be immediately apparent.  Obtain medical attention. SPECIAL FIRE FIGHTING PROCEDURES

Full protective equipment including self-contained breathing apparatus should be used.

Water spray may be ineffective.  If water is used, fog nozzles are "Preferable.  Water may be used to cool closed containers to prevent pressure build-up and possible autoignition or explosion when exposed to extreme heat.

Section 6 — ACCIDENTAL RELEASE MEASURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED Remove all sources of ignition.  Ventilate the area. Remove with inert absorbent.

Section 7 — HANDLING AND STORAGE

STORAGE CATEGORY

DOL Storage Class II

230	page 3

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE

Contents are COMBUSTIBLE.  Keep away from heat and open flame.

Consult NFPA Code.  Use approved Bonding and Grounding procedures.

Keep container closed when not in use.  Transfer only to approved containers with complete and appropriate labeling.  Do not take internally. Keep out of the reach of children.

To minimize the possibility of spontaneous combustion: control the accumulation of overspray; soak wiping rags and waste immediately after use in a water-filled, closed metal container; air dry filters outside, far from any combustible material and separated by bricks or other non-combustible spacers; dispose of all contaminated materials and waste properly.  Consult OSHA 29 CFR 1910.107(b)(5) and NFPA 33, Chapter 8 (8-9) for the proper procedures.

Section 8 — EXPOSURE CONTROLS/PERSONAL PROTECTION

PRECAUTIONS TO BE TAKEN IN USE

Use only with adequate ventilation.

Avoid contact with skin and eyes.  Avoid breathing vapor and spray mist.

Wash hands after using.

This coating may contain materials classified as nuisance particulates (listed "as Dust" in Section 2) which may be present at hazardous levels only during sanding or abrading of the dried film.  If no specific dusts are listed in Section 2, the applicable limits for nuisance dusts are ACGIH TLV 10 mg/m3 (total dust), 3 mg/m3 (respirable fraction), OSHA PEL 15 mg/m3 (total dust), 5 mg/m3 (respirable fraction).

Removal of old paint by sanding, scraping or other means may generate dust or fumes that contain lead.  Exposure to lead dust or fumes may cause brain damage or other adverse health effects, especially in children or pregnant women.  Controlling exposure to lead or other hazardous substances requires the use of proper protective equipment, such as a properly fitted respirator (NIOSH approved) and proper containment and cleanup.  For more information, call the National Lead Information Center at 1-800-424-LEAD (in US) or contact your local health authority. VENTILATION

Local exhaust preferable.  General exhaust acceptable if the exposure to materials in Section 2 is maintained below applicable exposure limits. Refer to OSHA Standards 1910.94, 1910.107, 1910.108. RESPIRATORY PROTECTION

If personal exposure cannot be controlled below applicable limits by ventilation, wear a properly fitted organic vapor/particulate respirator approved by NIOSH/MSHA for protection against materials in Section 2.

When sanding or abrading the dried film, wear a dust/mist respirator approved by NIOSH/MSHA for dust which may be generated from this product, underlying paint, or the abrasive. PROTECTIVE GLOVES

Wear gloves which are recommended by glove supplier for protection against materials in Section 2. EYE PROTECTION

Wear safety spectacles with unperforated sideshields. OTHER PRECAUTIONS

Intentional misuse by deliberately concentrating and inhaling the contents can be harmful or fatal.

Section 12 — ECOLOGICAL INFORMATION

CCOTOXICOLOGICAL INFORMATION No data available.

Section 13 — DISPOSAL CONSIDERATIONS

WASTE DISPOSAL METHOD

Waste from this product may be hazardous as defined under the Resource Conservation and Recovery Act (RCRA) 40 CFR 261.

Waste must be tested for ignitability to determine the applicable EPA hazardous waste numbers.

Incinerate in approved facility.  Do not incinerate closed container. Dispose of in accordance with Federal, State/Provincial, and Local regulations regarding pollution.

Section 14 — TRANSPORT INFORMATION No data available.

Section 15 — REGULATORY INFORMATION SARA 313 (40 CFR 372.65C) SUPPLIER NOTIFICATION

CAS No.       CHEMICAL/COMPOUND	% by WT   % Elemei

No ingredients in this product are subject to SARA 313 (40 CFR 372.65C! Supplier Notification.

CALIFORNIA PROPOSITION 65

WARNING:  This product contains chemicals known to the State of California to cause cancer and birth defects or other reproductive harm. TSCA CERTIFICATION

All chemicals in this product are listed, or are exempt from listing, on the TSCA Inventory.

Section 16 — OTHER INFORMATION

This product has been classified in accordance with the hazard criteri, of the Canadian Controlled Products Regulations (CPR) and the MSDS contaii all of the information required by the CPR.

ction 9 — PHYSICAL AND CHEMICAL PROPERTIES

EIGHT

GRAVITY

OINT

OINT

VOLUME

ON RATE

'SITY

Y IN WATER

ORGANIC COMPOUNDS

b/gal  507 g/1

b/gal   507 g/1

7.05  Ib/gal     845 g/1 0.85

300 - 412 F     148 - 211 C Not Available 65  %

Slower than ether Heavier than air N.A.

(VOC Theoretical - As Packaged) Less Water and Federally Exempt Solvents Emitted VOC

:Ction 10 — STABILITY AND REACTIVITY

Stable '0 AVOID rn. ,ITY rn.

•COMPOSITION PRODUCTS Carbon Dioxide, Carbon Monoxide iLYMERIZATION occur

ction 11 — TOXICOLOGICAL INFORMATION

,TH HAZARDS

iient in this product is an IARC, NTP or OSHA listed carcinogen,

i overexposure to solvent ingredients in Section 2 may cause

;cts to the liver and urinary systems.

iave associated repeated and prolonged overexposure to solvents

;nt brain and nervous system damage.

•ATA

Ingredient Name

Mineral Spirits

LC50   RAT     4HR -LD50   RAT Mineral Spirits (Odorless)

LC50   RAT     4HR

LD50   RAT Heavy Naphthenic Petroleum Oil

LC50   RAT     4HR

LD50   RAT Highly refined Naphthenic Oil

LC50   RAT     4HR

LD50   RAT

Not Available Not Available

Not Available Not Available

Not Available Not Available

Not Available Not Available