flinn scientific, inc.

Material Safety Data Sheet (MSDS)	MSDS #: 131 °°

Revision Date: May 16, 2008

SECTION 1 — CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Bromine

Flinn Scientific, Inc. P.O. Box 219 Batavia,IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

SECTION 2 — COMPOSITION. INFORMATION ON INGREDIENTS

Bromine

CAS#: 7726-95-6

SECTION 3 — HAZARDS IDENTIFICATION

Bromine: Dark, reddish-brown, fuming liquid.	FLINN Al-A-GLANCE

Highly toxic by ingestion and inhalation. Corrosive liquid and severe eye and skin irritant, may cause	Health-3

bums, including blindness. Strong oxidizer; heat of reaction may ignite combustibles on contact. Reacts      Flammability-0
with water or steam to produce toxic and corrosive fumes. Extremely hazardous substance.	Reactivity-3

Exposure-3 Storage-2

0 is low hazard, 3 is high hazard

SECTION 4 — FIRST AID MEASURES

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give 1 to 2 cups of water or milk, followed by a gastric antacid, such as milk of magnesia. Do not induce vomiting. Call a

physician or poison control at once.

SECTION 5 — FIRE FIGHTING MEASURES

Non-combustible liquid. Will react with water or steam to produce toxic and corrosive fumes.	NFPA code

Strong oxidizer; heat of reaction may ignite combustibles on contact.	H-3

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE	F-0

and SCBA with full facepiece operated in positive pressure mode.	R-0

OX

SECTION 6 — ACCIDENTAL RELEASE MEASURES

Restrict unprotected personnel from area. Remove all ignition sources and ventilate area. Contain spill with sand and absorbent material; cover spill with a reducing material such as hypo (sodium thiosulfate); deposit in sealed bag or container. See Sections 8 and 13 for further information.

SECTION 7 — HANDLING AND STORAGE

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and phosphates. Store in a cool dry place. Store in a Flinn Chem-Saf bag inside a Flinn Saf-Stor can. Use and dispense in a hood.

SECTION 8 — EXPOSURE CONTROLS. PERSONAL PROTECTION

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. ,<"•"*   Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper

cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire).Exposure guidelines: TWA 0.1 PPM, STEL 0.3 PPM (OSHA, NIOSH)

©Flinn Scientific, Inc. All Rights Reserved.	page 1 OF 2

flinn scientific, inc.

Material Safety Data Sheet     Bromine	MSDS #:

Revision Date: May 16, 2008

;_ .SECTION 9 — PHYSICAL AND CHEMICAL PROPERTIES

Dark, reddish-brown, filming liquid. Pungent odor.	Formula: Br2

Solubility: Water and most common organic solvents.	Formula Weight: 159.83

Specific Gravity: 3.1

Boiling Point: 58.8C

Vapor Density: 7.14

SECTION 10 — STABILITY AND REACTIVITY

Very strong oxidizing agent; heat of reaction may ignite combustible materials on contact. Avoid contact with all other laboratory chemicals. Will react with water or steam to produce toxic and corrosive fumes. Shelf life: Good, if stored properly.

SECTION 11 — TOXICOLOGICAL INFORMATION

Acute effects: Highly toxic, harmful vapor, lachrymator,	ORL-RAT LD50: 2600 mg/kg

corrosive.	IHL-RAT LC50: 2700 mg/m3

Chronic effects: N.A.	SKN-RBT LD50: N.A.
Target organs: Respiratory system, eyes & central nervous

N.A. = Not available, not all health aspects of this substance have been fully investigated.

SECTION 12 — ECOLOGICAL INFORMATION

Toxic to fish and other aquatic organisms. LC50 (rainbow trout) 0.31 mg/L (24 hr). LC50 (water flea) 1.07 mg/L (48 hr).

SECTION 13 — DISPOSAL CONSIDERATIONS

Please consult with state and local regulations. s~  Flinn Suggested Disposal Method #12a is one option.

SECTION 14 — TRANSPORT INFORMATION

Shipping Name: Bromine Hazard Class: 8, Corrosive, Poison UN Number: UN1744 N/A = Not applicable

SECTION 15 — REGULATORY INFORMATION

TSCA-listed, EINECS-listed (231-778-1), RCRA code D002, D003.

SECTION 16 — OTHER INFORMATION

This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific, Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. The data should not be confused with local, state, federal or insurance mandates, regulations, or requirements and CONSTITUTE NO WARRANTY. Any use of this data and information must be determined by the science instructor to be in accordance with applicable local, state or federal laws and regulations. The conditions or methods of handling, storage, use and disposal of the product(s) described are beyond the control of Flinn Scientific, Inc. and may be beyond our knowledge. FOR THIS AND OTHER REASONS, WE DO NOT ASSUME RESPONSIBILITY AND EXPRESSLY DISCLAIM LIABILITY FOR LOSS, DAMAGE OR EXPENSE ARISING OUT OF OR IN ANY WAY CONNECTED WITH THE HANDLING, STORAGE, USE OR DISPOSAL OF THIS PRODUCT(S).

Consult your copy of the Flinn Chemical Catalog/Reference Manual
—	for additional information about laboratory chemicals

©Flinn Scientific, Inc. All Rights Reserved.	PAGE 2 OF 2