MATERIAL SAFETY DATA SHEET 'Complies with OSHA Communication Standard 29 CFR 1910.1200 Dept. of Labor )

— rNTITY:

BRAKE  CLEANER TM-1807

565-1314

Section I

| Manufacturer's Name Aerosol Systems, Inc. |  | Emergency Telephone Number (216)467-4195 |  |
| --- | --- | --- | --- |
| Address | 9150 Valley View Road | Telephone Number for Information (216)467-4195 |  |
| Macedonia. OH 44056 |  | Date Prepared c/-i/qc. |  |
|  |  | Signaturgk^ftmtarer topt»cal)>L, X? |  |
| Section | II • Hazardous Ingredients/Identity Information | / |  |
| Hazardous Components (Specific Chemical Identity, Common Name(s)) OSHA PEL ACGIH TLV Other Limits Recommended |  |  | %(optional |
| 1.1.1 Trichloroethane ('CAS - 71-55-6) 350 |  |  | 47.0 |
|  | Perchloroethvlene (CAS - 127-18-4) | 50 | 50.0 |
|  | Carbon Dioxide (CAS - 124-38-9) | 5000 | 3.0 |
| DOT SHIPPING - Consumer Commodity ORM-D |  |  |  |

Section III - Physical/Chemical Characteristics

| Boiling Point Range | 197°F to ?"500F | Specific Gravity (H2O = 1 ) | 1.3 |
| --- | --- | --- | --- |
| Vapor Pressure PSIG @ 70° F | 100 | Melting Point Liquid | N.A. |
| Vapor Density (AIR •= 1) Heavier than Air | 4.0 | Evaporation Rate (Butyl Acetate = 1) | 3.0 |

Solubility in Water

None

Appearance and Color

Clear Liauid  Solvent Odor

Section IV - Fire and Explosion Hazard Data

| Flash Point (Method Used) | None | Flammable Limits | LEL None | UEL None |
| --- | --- | --- | --- | --- |

Extinguishing Media

Use water fog, dry chemical or carbon dioxide

Special Fire Fighting Procedures

Aerosol cans may rupture when heated

Unusual Fire and Explosion Hazards

Heated cans may burst

rtion V - Reactivity Data

| bility | Unstable |  | Conditions to Avoid |
| --- | --- | --- | --- |
|  | Stable | X | High Temperatures |

ompatibility (Materials to Avoid)

None ratdous Decomposition or Byproducts

In   fire  product  may  decompose   to  carbon  dioxide   and  water.

| zardous ymerization | May Occur |  | Conditions to Avoid |
| --- | --- | --- | --- |
|  | Will not Occur | X | None |

rtion VI - Health Hazard Data

Jte(s) of Entry:

Inhalation?

Yes

Skin?

Yes

Ingestion?

Yes

»lth Hazards (Acute and Chronic)

May cause dizziness or norcosis in high vapor concentrations. Will cause defatting of skin.

Effects are reversible. Long term exposure (years) vapor may cause lung, liver or kidney damage.

cinogenicity:	NTP?

Presently not on any list.

(ARC Monographs?

OSHA Regulated?

ns and Symptoms of Exposure

Inhalation - Difficulty in breathing.     Skin-redness.    Ingestion-vomiting.

dical Conditions

lerally Aggravated by Exposure:   Heart Disease

ergency and First Aid Procedures

-*^	Give oxygen - Do not induce vomiting - Gastric lavage   Wash eyes and skin with water.

tion VII - Precautions for Safe Handling and Use

ps to be Taken in Case Material is Released or Spilled:

Use absorbent sweeping compound to soak up material. Put into container. Dispose as hazardous waste.

'"

ste Disposal Method

Dispose as hazardous waste in accordance with EPA RCRA.

cautions to be Taken in Handling and Storing

Keep away from heat, sparks, or open flame. Store at temperatures below 120° F.

ler Precautions

None

tion VIII • Control Mer'

piratory Protection (Si	/pe)

Self contah.«.o breathing apparatus if above TLV limit exceeding.

| itilation | Local Exhaust Yes | Special | None |
| --- | --- | --- | --- |
|  | Mechanical (General! None | Other | None |
| tective Gloves Eye Protection None reauired if spraying W"ar |  |  | ryp nrnfciion |

Lo::g si"

nd innq pants

rk/Hygienic Practices

Do not smoke

snr3yinq; wash hands after use.

-

80