FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:     282.00

Revision Date:      February 24, 2003

Section 1 — Chemical Product and Company Identification

Cupric Nitrate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition, Information on Ingredients

Cupric Nitrate

Synonym: copper (II) nitrate

CAS#:  19004-19-4

Section 3 — Hazards Identification

Blue crystals, nitric acid odor.

Moderately toxic by ingestion and inhalation. Corrosive. Body tissue irritant. Avoid all body tissue contact.

Strong oxidizer, keep away from organic material.

Section 4 — First Aid Measures

flinn at-a-glance

Health-2

Flammability-0

Reactivity-3

Exposure-2

Storage-1

0 is low hazard, 3 is high hazard

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable, non combustible solid.

Strong oxidizer. Avoid contact with combustible and organic materials. Heating to decomposition may emit

toxic fumes of NOx.

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and

SCBA with full facepiece operated in positive pressure mode.

NFPA code

None Established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #3. Store with amides, nitrates and nitrites. Store in a cool dry place. Deliquescent, store in Flinn Chem-Saf Bag. Use and dispense in a hood.

Section 8 — Exposure Controls, Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

) 2003 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

FLINN SCIENTIFIC INC.

Your Safer Source for Science Supplies"	MSDS #:   282-°°

km   j.     •   i «•»   .r  j     r>i    e+i        a /••<-> rt <-« \	Revision Date:   February 24,2003

Material Safety Data Sheet (MSDS)

Section 9 — Physical and Chemical Properties

Blue crystals, nitric acid odor.	Specific Gravity: 2.0

Solubility: Soluble in water and alcohol.	Melting Point: 115 C

Formula: Cu(NO3)2 3H20 Formula Weight: 241.60

Section 10 — Stability and Reactivity

Avoid contact with organic materials, reducing agents, heat or moisture. Avoid dust. Shelf life: Poor; substance deliquescent.

Section 11 —lexicological Information

Acute effects: Corrosive	ORL-RAT LD50: 940 mg/kg

Chronic effects: N.A.	IF1L-RAT LC50: N.A.

Target organs: N.A.	SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been folly investigated.

Section 12— Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Nitrates, Inorganic, n.o.s. Hazard Class: 5.1, Oxidizer

UN Number: UN 1477

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (221-838-5), RCRA code D001.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemical Packaging Prevents Accidents

flinn@flinnsci.com   www.flinnsci.com P.O. Box 219        BatavialL 60510 (800) 452-1261   Fax (866) 452-1436

© 2003 Flinn Scientific, Inc. All Rights Reserved.	page 2 OF 2