Loctite Corporation

Automotive and Consumer Group

4450 Cranwood Court Cleveland, Ohio 44128

(216) 475-3600

Telex: 6873201 LOCTIT UW

MATERIAL SAFETY DATA SHEET

DATE: 12/28/88 PAGE:  1

* ATTN: SAFETY SUPERVISOR

(79340)-   00000

I.

IDENTIFICATION

PRODUCT NAME: DURO TM SUPER GLUJ

PART NO.:  SUP-l-BK ~7(p$-l2(s2.   UPC CODE:

PRODUCT   TYPE:   CYANOACRYLATE   ADHESIVE

INGREDIENTS ETHYL   CYANOACRYLATE POLYIMETHYL   METHACR^LATE)(11 *HYDROQUINONE

II.   COMPOSITION

CAS.   # 7085-85-0 9011-14-7 123-31-9

%  WT.   ACGIH
95*        N/E
<5	N/E

<0.5      N/E

TLV

OSHA N/E N/E N/E

PEL

,j.)PURE POLY (METHYL METHACRYLATE) HAS BEEN SHOWN TO CAUSE TUHORS IN

EXPERIMENTAL ANIMALS WHEN IMPLANTED BENEATH THE SKIN.  IN LIGHT OF

LOW CONCENTRATION OF THIS COMPONENT IN THE. PRODUCT, IT I S OUR BEST

TECHNICAL JUDGEMENT THAT NORMAL USE OF THIS PRODUCT POSES NO SUCH

HAZARD.

*THIS COMPONENT IS LISTED AS A SARA SECTION 313 TOXIC CHEMICAL,

VAPOR PRESSURE VAPOR DENSITY SOLUBILITY IN WATER APPEARANCE SPECIFIC GRAVITY BOILING POINT PH ODOR

III. CHEMICAL AND PHYSICAL PROPERTIES

<2MM 3 75 OEG F APPROX. 3

POLYMERIZED BY WATER CLEAR LIQUID 1.05 3 75 DEG F >300 DEG F DOES NOT APPLY SHARP, IRRITATING

TOXICITY:

IV, TOXICITY AND HEALTH HAZARD DATA

BONDS SKIN RAPIDLY AND STRONGLY.

SKIN AND EYE IRRITANT

LOW TOXICITY BY INGESTION.

INFORMATION FOR FIRST AID AND CASUALTY ON TREATMENT FOR SKIN ADHESION OF HUMAN SKIN TO ITSELF IF CAUSED BY CYANOACRYLATE ADHESIVES:

Automotive and Consumer Group

4450 Cranwood Court Cleveland. Ohio 44128

(216) 475-3600

Telex: 6873201 LOCTIT UW

#:7651262

PAGE:  2 -

skin adhesion: first immerse the bonded surfaces in warm sdapy water." peel or roll the surfaces apart with the aid of a blunt edget e.g.  ; spatula or teaspoon handle; then remove adhesive from the skin with soap and water. do not try ano pull surfaces apart with a direct opposing action.

eyelid to eyelid or eyeball adhesion: in the even't that eyelids are stuck together or bonded to the eyeball, wash thoroughly with wflrm water and apply a gauze patch. the eye will open without further action, typically in 1-4 days. there will be no residual damage. do not try to open the eyes by manipulation.

adhesive on the eyeball: cyanoacrylates introduced into the eyes will attach itself to the eye protein and will disassociate from it * over intermittent periods, generally covering several hours. this . will cause periods of weeping until clearance is achieved during the period of contamination double vision may be experienced together with a lachrymose effect, and it is important to understand the cause and realize that dis as soc i at ion will normally occur within a matter of hours, even with gross contamination.

mouth: if lips are accidentally stuck together apply lots of warm water to the lips and encourage maximum wetting and pressure from saliva inside the mouth. peel or roll lips apart. do not try and pull the lips with direct opposing action. it is almost impossible to swallow cyanoacrylate. the adhesive solidifies and adheres in the mouth. saliva hill lift the adhesive in 1/2 to 2 days. in case a lump forms in the mouth, position the patient to prevent ingestion of the lump when it'detaches.

burns: cyanoacrylates give off heat on solidification. in rare cases a large drop will increase in temperature enough to cause a burn. burns should be treated normally after the lump of cyanoacrylate is released from the tissue as described above.

surgery: it should never be necessary to use such a drastic method tc
separate accidentally bonded skin.	*

carcinogenicity: ntp        : n

iarc monographs: n osha regulated: n hmis hazard communication codes: health (2)  fire (2)  reactivity (1)

TLV:  2PPM

PRIMARY ROUTES OF ENTRY:

SKIN, INGESTION, INHALATION EXISTING CONDITIONS AGGRAVATED BY EXPOSURE:

NONE KNOWN SYMPTOMS OF OVEREXPCSURE :

VAPOR IS IRRITATING TO EYES AND MUCOUS MEMBRANES ABOVE TLV.

EMERGENCY  TREATMENT   PROCEDURES: INGESTION:

..,     SEE    INFORMATION   ABOVE   ON   EMERGENCY   PROCEDURES.

Loctite Corporation

Automotive and Consumer Group

4450 Cranwood Court Cleveland, Ohio 44128

(216)475-3600

Telex: 6873201 LOCTIT UW

ART   #:7651262     -/	PAGE:      3

inhalation:

remove to fresh air. treat symptomatically. skin contact:

see information above on emergency procedures. eye contact:

see information above on emergency procedures. personal protection:

eyes:    safety glasses recommended.

skin:    polyethylene gloves recommended. do not use cotton gloves.

ventilation:

provide adequate local ventilation , maintain concentration below tlv.

v. flammability and explosive properties

flash point: >200 deg f	method: t.c.c.

explosive limitst? by volume  in air)        lower: unk z upper: unk % recommended extinguishing agents: c02, foam, dry chemical hazardous products formed by fire or thermal decomposition:

none known unusual fire or explosion hazards:

: ^	NONE

I   OMPRESSED GASES: NAME: NONE PRESSURE AT ROO.V TEMPERATURE:

VI. REACTIVITY DATA

stability:	ixi    stable

I I  UNSTABLE

IX 1  WILL NOT. OCCUR

hazardous polymerization:	i   |    may occur

hazardous decomposition productstnon-thermal):

none known incompatability:

polymerized by contact with water, alcohol, amines, alkalies.

vii. spill or leak and disposal procedures

steps to  be taken  in case of spill or leak:

flood with water to polymerize. soak up with an inert absorbent. recommended methods of disposal:

polymerize as  above. landfill or incinerate following epa and local

regulations.

cercla reportable qtycin lbs.j: rcra hazardous waste no.:

VIII. STORAGE AND HANDLING PROCEDURES

STORAGE: STORE AT OR BELOW 75 DEG F TO MAXIMIZE SHELF LIFE. HANDLING:AVOID CONTACT WITH SKIN AND EYES. AVOID BREATHING VAPOR.

™na __	_

Automotive and Consumer Group	,,,„. ,,t. ,„„„

(/id) 475-36OO

Telex: 6873201 LOCTIT UW

ART   *:7651262	PAGE:

ix. shipping regulations

type or class dot: unrestricted iata: unrestricted proper shipping name

DOT:

iata:

PREPARED BY: KATHRYN A. HOLEN

TITLE: ENVIR. HEALTH AND SAFETY SPECIALIST

DATE: 12/16/88

SUPERSEDES:  NEW ISSUE

N/A = NOT APPLICABLE

EST = ESTIMATE

DNA = DOES NOT APPLY

UNK = UNKNOWN

N/E   =   NOT   ESTABLISHED