FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

MSDS #:    62.00
Material Safety Data Sheet (MSDS)	Revision Date:   November 25, 2002

Section 1 — Chemical Product and Company Identification

Ammonium Oxalate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Ammonium Oxalate

Synonym: Ethanedioic Acid Diammonium Salt

CAS#: 6009-70-7

Section 3 — Hazards Identification

White odorless powder.	flinn Ar-A-GLANCE

Toxic by ingestion and inhalation. Can cause severe kidney damage. Moderately corrosive to body	Health-2

tissues. Avoid body tissue contact.	Flammability-0

React! vity-1 Exposure-2 Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non-flammable solid.	NFPA code

When heated to decomposition, emits toxic fumes of ammonia and oxalic acid.	None Established

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and SCBA with full facepiece operated in positive pressure mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and

phosphates.

Use and dispense in a hood. Store in a cool dry place.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Ammonium Oxalate

MSDS #:    62.00

Revision Date:   November 25, 2002

*>

Section 9 — Physical and Chemical Properties

White odorless powder. Solubility: Water soluble (2.54 g/100 ml). Formula: (NH4)2C2O4 H2O Formula Weight: 142.11

Specific Gravity:  1.5 Melting Point: 133 C (dec.)

Section 10 — Stability and Reactivity

May react violently with NaOCl and ammonium acetate. Shelf life: indefinite

Section 11 —Toxicoloqical Information

Acute effects: Corrosive, toxic, irritant

Chronic effects: N.A.

Target organs: Kidneys, nerves

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been folly investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not Regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

Not listed.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn— "Your Safer Source for Chemicals "

^

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   BatavialL 60510

(800) 452-1261 Fax (866) 452-1436

2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2