MATERIAL SAFETY	AMOFUEL NO. 2 OIL

DATA SHEET

MSDS NO:  02000910

MANUFACTURER/SUPPLIER:	EMERGENCY HEALTH INFORMATION:  (800) 447-8735

Amoco Oil Company	EMERGENCY SPILL INFORMATION:  (800) 424-9300

200 East Randolph Drive	OTHER PRODUCT SAFETY INFORMATION:  (312) 856-3907
Chicago, Illinois 60601

IMPORTANT COMPONENTS:  Petroleum distillate (CAS 68476-30-2).

No exposure limit established.

WARNING STATEMENT:  Warning!  Combustible.  Harmful if swallowed and/or aspirated into

the lungs.  Can cause skin irritation upon prolonged or repeated contact.

HMIS/NFPA CODES:(HEALTH;!)(FLAMMABILITY;2)(REACTIVITY;0) APPEARANCE AND ODOR:  Clear, bright liquid.

HEALTH HAZARD INFORMATION

EYE

EFFECT:     No significant irritation expected. "^

FIRST AID:  Flush eyes with plenty of water.

PROTECTION: None required, however, use of safety glasses is good industrial practice.

SKIN

EFFECT:     Can cause skin irritation upon prolonged or repeated contact.  See Toxicology Section.

FIRST AID:  Wash exposed skin with soap and water. Remove contaminated clothing, including shoes, and thoroughly clean and dry before reuse.

• PROTECTION: Avoid prolonged or repeated skin contact. Wear protective gloves if prolonged or repeated contact is likely.

INHALATION EFFECT:     None expected under usual conditions of use.

"

FIRST AID:  Not applicable.

PROTECTION: None required for usual conditions of use.  Use with adequate ventilation.

AMOFUEL NO. 2 OIL MSDS NO:  02000910

f

PAGE 02 OF 03

FIRE AND EXPLOSION INFORMATION

FLASHPOINT:      120°F TO    180°F, (TCC) FLAMMABLE LIMITS:  UPPER:  7.5%  LOWER:  0.6%

EXTINGUISHING MEDIA:  Agents approved for Class B hazards (e.g., dry chemical, carbon

dioxide, halogenated agents, foam, steam) or water fog.

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Not applicable.

PRECAUTIONS:  Keep away from ignition sources (e.g., heat and open flames).

REACTIVITY INFORMATION

STABILITY:  Stable.

CHEMICAL AND PHYSICAL PROPERTIES

BOILING POINT:      340°F TO     675°F, Range approx. SOLUBILITY IN WATER:  Negligible, below 0.1%. SPECIFIC GRAVITY (WATER =1):       0.88 VISCOSITY:  1.8-3.6 cS @ 100°F

STORAGE AND ENVIRONMENTAL PROTECTION

STORAGE REQUIREMENTS:  Store away from heat, ignition sources, and open flame, in

accordance with applicable regulations.

SPILLS AND LEAKS:  Remove mechanically or contain on absorbent material.

WASTE DISPOSAL:  Disposal must be in accordance with applicable federal, state, or

local regulations. Enclosed-controlled incineration unless directed otherwise by applicable ordinances.

SPECIAL PRECAUTIONS:  Avoid strong oxidizers.
		TOXICOLOGICAL INFORMATION

EYE:  Similar products have produced maximum eye irritation scores ranging from 0.33 to 1.0/110.0; 24 hours (rabbits).

SKIN:  Similar products have produced primary skin irritation scores ranging from 0.67 to 3.83/8.0 (rabbits).  Dermal LD50 for similar products was greater than 2g/kg; practically non-toxic for acute exposures by this route.

INGESTION:  For a similar product oral LD50 was greater than 5g/kg; practically non-toxic for acute exposures by this route.

A chronic skin-painting study has shown that this product is capable of producing skin cancer in laboratory animals.  Ten out of the original 50 test animals developed on-site tumors following repeated and prolonged skin applications of furnace oil over their lifetime.  The mean latency period was 90.0 weeks.

Occasional skin contact with this product is not expected to have serious

AMOFUEL NO. 2 OIL MSDS NO:  02000910

PAGE 03 OF 03

TOXICOLOGICAL INFORMATION - CONTINUED

effects, but good personal hygiene should be practiced; prolonged and unnecessary skin contact avoided. This product can also produce moderate skin irritation upon prolonged or repeated skin contact.  Personal hygiene measures taken to prevent skin irritation are expected to be adequate to prevent undue risk of skin cancer.

No component of this product has been identified as a carcinogen by NTP, IARC or OSHA.

REGULATORY INFORMATION

DOT PROPER SHIPPING NAME:  Fuel Oil, No. 2, Combustible Liquid, NA1993.

OSHA HAZARD COMMUNICATION STANDARD:  Combustible liquid.

TSCA STATUS:  All of the components of this product are listed on the TSCA Inventory.

ISSUE INFORMATION

BY:

Richard A. Symuleski Director, Product Safety