103018-01

MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLE, IWRTH CAROLINA 28078

- material safety oata sheet -

these material safety data sheets are sent to you in compliance with 29 cfr 1910.1200 which became effective november 25, 1985.

emergency phone: (7o4) 875-6513 msds reference:   20500 revision date:   6/11/90

section 1 - identification

manufactured for: maintenance supply company. inc,

post office box 496 huntersville, n.c. 23073

product:        akti-static

trade name:      snap-shock anti-static

package type:    aerosol container

hmis symbol: health, 1 (slight); flammability. 3 (serious)!

reactivity. 1 (slight) nfpa symbol: toxicity. i (slight)i fire, 3 (high);

reactivity, i (slight)

TLV	PEL	Approx. % Wt.

1,1.1-TRICHLOROETHAWE	350     350       <45£

(CAS # 71-55-6)

ISOPROPYL ALCOHOL	400     400

(CAS » 67-63-0)

PROPELLANT: PHOPANE/BUTANE	1000    1000

(CAS # 74-98-6/106-97-8)

SECTION 3 - PHYSICAL DATA

BOILING POINT OF CONCENTRATE: NOT DETERMINED VAPOR PRESSURE IN CAW PSI6 3 75 DEGREES F.: 50 VAPOR DENSITY (AIR = 1): NOT DETERMINED

SOLUBILITY IN «ATER OF CONCENTRATE: 1,07 GM/100 GM 3t 25 DEGREES SPECIFIC GRAVITY OF CONC, 3 75 DEGREES F, (H20=l): 1.3 PERCENT VOLATILE 8Y VOLUME IN CAM: 100% FLASH PQIMT OF CONCENTRATE:

APPEARANCE AND      Of SPRAY: CLEAR WATER WHITE LIQUID/MILD SWEETISH 000ft

SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

FLAMMABILITY AS PER CPSC FLAME EXTENSION TEST: EXTREMELY

FLAMMABLE SPRAY, FLAMMABLE LIMITS: N/A

EXTINGUISHING MEDIA: FOAM, DRY CHEMICAL, CARSON DIOXIDE SPECIAL FIRE FIGHTING PROCEDURE: KEEP CONTAINERS COOL,  USE

EQUIPMENT GR SHIELDING REQUIRED TO PROTECT PERSONNEL AGAINST BURSTING, RUPTURING OR VEHTING CONTAINERS, yHAR SELF-CONTAINED BREATHING APPARATUS,

UNUSUAL FIRE AND EXPLOSION HAZARDS: AT ELEVATED TEMPERATURES (ABOVE 120 DEGREES F.) CONTAINERS MAY VENT. RUPTURE OR BURST.

SECTION 5 - REACTIVITY DATA

CHEMICAL STABILITY: STABLE

CONDITIONS TO AVOID:  DO HQ7 EXPOSE TO TEMPERATURES AROVE

120 DEGREES F, INCOMPATIBILITY (MATERIALS TO AVOID): STRONG ALKALIES, OXIOIZER.f

ALUMINUM AND OTHER REACTIVE METALS,

HAZARDOUS DECOMPOSITION PRODUCTS: AT ABNORMALLY HIGH TEMPERATURE POSSIBLE TRACE AMOUNTS OF PHOSGENE OR HYDROGEN CHLQRIDi HAZARDOUS POLYMERIZATION: WILL WOT OCCUR CONDITIONS TO AVOID:

SECTION 6 - HEALTH HAZARD DATA

routes of entry: skin primary

effects of over exposure (acute and chronic):

inhalation: minimal effects below 1000 ppmj dizziness, drowsiness ano throat irritation at levels above 1000 ppm, unconsciousness and death possible at levels above 10,000 ppm, blooo pressure depression cardiac sensitization. ventricular arrhythmia caf4 result fro« exposure to ^ear anesthetic levels,

skin contact/absorption: prolonged or repeated contact cam cause moderate irritation, oefatting and dermatitis,

insestion: can cause gastrointestinal irritation. nausea, vomiting and diarrhea, aspiration of material into the lungs cam cause chemical

PfiiEUWQNITIS,

EYES: CAH CAUSE SEVERE IRRITATION, REDNESS, TEARING ANC BLURRED VISION*

carcino&inicity: no

NTP: MO

IARC MONOGRAPHS: NO

OSMA REGULATED: NO

EYES ANO SKIM:  FLUSH EYES WITH WATER, LIFTING UPPER AND

LIDS,  GET MEDICAL ATTENTION,  WASH EXPOSED SKIN AREAS WITH SOAP ANO WATER,

INHALATION: REMOVE TO FRESH AIR,  IF BREATHING IS DIFFICULT, ADMINISTER OXYGEN,  IF BREATHING STOPS GIVE ARTIFICIAL RESPIRATION,  GET MEDICAL ATTENTION,

INGESTION: 00 NOT INDUCE VOMITING,  GET MEDICAL ATTENTION, ASPIRATION OF MATERIAL INTO THE LUNGS DUE TO VOMITING CAN CAUSE CHEMICAL PNEUHONITIS,

SECTION  7 - PRECAUTIONS FOR SAFE HANDLING AND USE STEPS TO 8E TAKEN IN CASE CONTAINER IS PUNCTURED AMD MATERIAL

is released:

CLEAN UP AREA BY HOPPING OR WITH ABSORBENT MATERIALS ANO PLACE IN CLOSED CONTAINERS FOR DISPOSAL,  CONTACT FEDERAL, STATE OR LOCAL AUTHORITIES FOR APPROVED DISPOSAL PROCEDURES,