MATERIAL SAFETY DATA SHEET	12/09/88

For Coatings* Resins and Related Materials

SECTION  I  - PRODUCT INDENTIF ICATION

Manufacturer: GUARDIAN CHEMJ^Ur-GOMEANY       Information Phone: (404) 873-1692
ATTN: W.K^-Fawcett Jr	Emergency   Phone: (404) 873-1692

6,94 Je t-Person Street N. W.
'^ATLANTA	ga  30318 NHazard Ratings:      Health - 1

Product C}4.ss: VARNISH	! )none -> extreme        Fire - 1

Trade Namfe   : TOP-LOC, URETHANE GYM FINISH  !/ 0  	>  4      Reactivity - 0

Product CoNJe : 12735		y(

C.A.S. Numbe><_ NONE	!         Personal Protection - A

SECTION  II  - HAZARDOUS INGREDIENTS

Weight  Exposure      Vapor Pr.
Inatiadients	CAS_tt	  	7.	Limits	mm_bla

aliphatic: hydrocarbon	64742-88-7	so.	100.      ppm	.3

OSHA--PEL/TWA = 50O ppm ACGIH-TLWTWA = 100

--TLV/STEL = 125 PETROLEUM HYDROCARBON       8008-2O-6       1O.     500.   ppm      2.

SECTION  III - PHYSICAL DATA

filing Range:    330 - 50O Deg.	F         Vapor  Density:	Heavier	than Air.

.p. Rate: Slower than  n-Butyl	Acetate   Liquid Density:	Lighter	than Water,

Volatile* volume: 53.0    "/.	Wgt per gallon!	7.28	Pounds.
Appearance: AMBER LIQUID

SECTION  IV  - FIRE. AND EXPLOSION HAZARD DATA

Flammability Class: II	Flash Point: 114 F tec       LEL :

-EXTINGUISHING MEDIA:

Carbon Dioxide	Dry Chemical	Foam	Water Fog

Use water for cooling material stored in vicinity of fire.

-SPECIAL FIREFIGHTING PROCEDURES:

Use self-contained breathing apparatus with a. full facepiece operated in pressure—demand or other positive pressure mode.

-UNUSUAL FIRE & EXPLOSION HAZARDS:

Vapors are heavier than air and may travel along the ground to an ignition source some distance from material handling point. Ignition sources include pilot lights* smoking* heaters* electric motors* sparks from electrical switches and static discharges.

CAUTION: Never use cutting torch on empty containers! Residual solvent vapor in empty container may explode.

SECTION  V   • HEALTH HAZARD DATA

-PERMISSIBLE EXPOSURE LEVEL:

Unknown.

SECTION  V  - HEALTH HAZARD DATA (cont.)

-EFFECTS OF OVEREXPOSURE:

Eves: can cause severe irritation, redness, tearing, blurred vision.

Skin: Prolonged or repeated contact can cause moderate irritation, defatting, dermatitis.

Breathing! Excessive inhalation of vapors and/or spravmist can

cause respiratory irritation, dizziness, weakness, fatigue nausea, headache, unconsciousness and even asphyxiation.

8 wa 1 1 owing: Ca n cause g a s t r o i n t e s t :i. n a I i r r i t a t i o n , n a u sea,

vomiting and diarrhea. -FIRST AID!

Eves: Flush with large amounts of water lifting eyelids occasionally" set prompt medical attention.

Skin: Wash thoroughly with soap and water-, remove contaminated clothing promptly" wash clothing before reuse.

Swallowing: DO NOT induce vomiting! Keep person warm, quiet and set medical attention. Aspiration of material into the lungs due to vomiting can cause chemical pneumonitis which can be fatal.

Breathing: Move affected person to fresh air. If breath ins is difficult, administer oxygen. If breathing has stopped, give artificial respiration. Keep person warm, quiet, and GET MEDICAL ATTENTION.

SECTION  VI  - REACTIVITY DATA

STABLIT Y:  C 3 Unstable       C x3 St ab1e

HAZARDOUS POLYMERIZATION:    C "J May occur       Cx3 Will not occur

-INCOMPATIBILITY

-CONDITIONS TO AVOID:

KEEP AWAY FROM HEAT and open flame.

DO NOT USE AROUND EXPOSED FOODS, odor of product may be absorbed -HAZARDOUS DECOMPOSITION PRODUCTS:

May form carbon monoxide and dioxide, various hydrocarbons, etc.

SECTION  VII   SPILL OR LEAK PROCEEOURES

-STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

SMALL SPILL: absorb liquid with rags, floor absorbent, verrnicu-1 i t e or o t h e r a b s o r b e n t mat e r i a. 1 a n d t r a n s f e r • t o h o o d . -WASTE DISPOSAL METHOD:

SMALL SPILL: allow volatile portion to evaporate in hood

dispose of remaining material in accordance with applicable r e g u 1 a t i o n s .

SECTION VIII - SPECIAL PROTECTION INFORMATION:

-RESPIRATORY PROTECTION:

If TLV of the product is exceeded, a NIOSH/MSHA Jointly approved air supply respirator is advised in absence of proper environmental control. OSHA regulations also permit other NIOSH/MSHA r e s p i r a t o r s under specifie d c o n d :i. t i o n s „ ( s e e y o u r s a f e t y e <=i u i p m t.