J   MM  ^  $  7fifl? MATERIAL SAFETY DATA SHEET i(JU^

This MSDS complies with OSHA'S Hazard Communication Standard 29 CFR 1910.1200 and OSHA Form 174

IDENTITY AND MANUFACTURER'S INFORMATION

HMIS Rating: Health-2;   Flammability-3;   Reaclivity-O;   Personal Proteciion-B

NFPA Rating: Heallh-2;   Flammability-3:   Reaclivity-0;   Special-B

Winzer Corporation 10560 Markison Road Dallas, Texas 75238

Prepared By: PL

Supplier's Name: Address:

Date Prepared: 10/07/96

Information Calls: (800) 527-4126

DOT Hazard Classification:   ORM-0 Identity (trade name as used on label):

BRAKE CLEANER - LOW V.O.C.  891.4522

MSDS Number: 891.4522.7

Revision-1

NOTICE: JUDGMENT BASED ON INDIRECT TEST DATA

SECTION 1 - MATERIAL IDENTIFICATION AND INFORMATION

;OMPONENTS-CHEMICAL NAMES AND COMMON NAMES

CAS Number

SARA III LIST

Hazardous Components 1% or greater; Carcinogens 0.1% or greater)

HEXANE

Yes

110-54-3

ACETONE

67-64-1

NO

CARBON DIOXIDE

No

124-38-9

OSHA PEL

(ppm)

50

1000

5000

ACGIH TLV (ppm)

50

750

5000

Carcinogen Ret Source *

SECTION 2 - PHYSICAL/CHEMICAL CHARACTERISTICS

Specific Gravity (H2O1):   Concentrate Only = 0.728

Boiling Point:   N/A

Vapor Pressure (Non-Aerosols)(mm Hg and Temperature):   N/A

Vapor Pressure: PSIG @ 70°F (Aerosols):   80-100

Vapor Density (Air=1):   N/E

Evaporation Rate (

•1):    N/E

Solubility in Water:None

Water Reactive:   No

Appearance and Odor:   Clear forceful spray with aliphatic/ketone solvent odor.

SECTION 3 - FIRE AND EXPLOSION HAZARD DATA

FLAMMABILITY as per USA FLAME PROJECTION TEST
(aerosols)       EXTREMELY FLAMMABLE

Flammability Limits in Air by % in Volume:
%LEL:   N/E	% UEL:   N/E

Auto Ignition Temperature N/E

FLASH POINT AND METHOD USED (non-aerosols):    N/A

EXTINGUISHER MEDIA: Foam, dry chemical, carbon dioxide.

SPECIAL FIRE FIGHTING PROCEDURES: Self-contained breathing apparatus.

Unusual Fire & Explosion Hazards: Do not expose aerosols to temperatures above 130°F or the container may rupture.

SECTION 4 - REACTIVITY HAZARD DATA

HAZARDOUS POLYMERIZATION   f  ]   WILL    [ X ] WILL NOT     OCCUR

STABILITY      [ X] STABLE    [    ]   UNSTABLE

Conditions to Avoid: Open flame, welding arcs, heat, sparks.

'ncompatibility (Mai, to avoid): Strong oxidizing agents.

Hazardous Decomposition Products: Carbon dioxide, carbon monoxide.

SECTION 5 - HEALTH HAZARD DATA

PRIMARY ROUTES OF ENTRY:    [ X ] INHALATION    [   ] INGESTION    [ X ] SKIN ABSORPTION    [   ] EYE    [  ] NOT HAZARDOUS

ACUTE EFFECTS

Inhalation: Excessive inhalation of vapors can cause nasal & respiratory irritation, dizziness, weakness, nausea, headache, possible unconsciousness or
asohvxiation.

[Skin Contact: Slight irritation due to defatting of skin.

:ye Contact: Slight irritation.

Ingestion: Irritation.

CHRONIC EFFECTS: (Effects due to excessive exposure to the raw materials of this mixture) Excessive inhalation may result in central nervous system effects.

Medical Conditions Generally Aggravated by Exposure: May aggravate existing eye, skin, or upper respiratory conditions.

EMERGENCY FIRST AID PROCEDURES

Eye Contact: Flush with water for 15 minutes. If irritated, seek medical attention.

jSkin Contact: Wash with soap and water. If irritated, seek medical attention.

Inhalation: Remove to fresh air. Resuscitate if necessary. Get medical attention.

Ingestion: DO NOT INDUCE VOMITING.  Drink two large glasses of water. Get immediate medical attention.

SECTION 6 - CONTROL AND PROTECTIVE MEASURES

jRespiratory Protection (specify type): If vapor concentration exceeds TLV, use respirator approved by NIOSH approved for organic vapor.
[Protective Gloves^ Neqprene	|Eye Protection: Safety glasses recommended.

Ventilation Requirements: Adequate ventilation to keep vapor concentration below TLV.

Other Protective Clothing & Equipment: None

Hygienic Work Practices: Wash with soap and water before handling food.

SECTION 7 - PRECAUTIONS FOR SAFE HANDLING AND USE

Steps To Be Taken If Material Is Spilled Or Released: Absorb with suitable medium. Incinerate or landfill according to local, state or federal regulations. DO NOT FLUSH TO SEWER

Waste Disposal Methods: Aerosol cans when vented to atmospheric pressure through normal use, pose no disposal hazard.

Precautions To Be Taken In Handling & Storage: Do not puncture or incinerate containers. Do not store at temperatures above 130'F. Other Precautions &/or Special Hazards: KEEP OUT OF REACH OF CHILDREN. Avoid food contamination.

We believe the statements, technical information and recommendations contained herein are reliable, but they are given without warranty or guarantee of any kind. ** Chemical Listed as Carcinogen or Potential Carcinogen, [a] NTP [b] (ARC Monograph [c] OSHA [d] Not Listed [e] Animal Data Only