c

TEL 910-73B-7224

MATERIAL?     VTY DATA SHEET AMERICAN mOUSTRIES, INC.

FAX 910-738-2083 * 4300 KAHN DRIVE • PO BOX 1405 - LUMBERTON, NC 28359-1405

HAZARD RATINGS

PRODUCT IDENTIFICATION

Product Id:	1606.06

Trade Name;	HYTEMP (A)	Health:

Generic Name:	High Temperature Ami-Seize Aerosol Lubricant	Flammability:

Date:	August 20,1998	Reactivity:

Emergency #:	CHEMTREC1 800 424-9300	Personal Protection:

2 3 0 B

SECTION 5 - EMERGENCY AND FIRST AID PROCEDURES

Skin: Wash with soap and water. If irritation persists seek medical attention.

Eyes: Flush with large amounts of cool running water (or at least 15 minutes while holding upper and

lower lids open. Get medical attention,

If Ingested: DO NOT INDUCE VOMITING, Seek medical attention immediately.,

Inhajatioji: Remove to fresh air. Seek medical attention immediately. If breathing stops give artificial

respiration.

SECTION 1 - HAZARDOUS INGREDIENTS & MAJOR COMPONENTS

Chemical And/Or Common Name: 'Methylene Chloride Petroleum Distillate Liquified Petroleum Gas

GAS#:

75-09-2

64742-89-8

68476-85-7

40-60

1 Subject to SARA Title III Section 313 Reporting Requirements.

SECTION 2 - PHYSICAL DATA

Data Below Based on Aerosol Concentrate Only;

Boiling Point:	92'F

Vapor Density (air = 1):	>1

Specific Gravity (HaO - 1)@75°F:	1.195

Solubility In Water:	Negligible

Appearance and Odor:	Dark gray viscous liquid, solvent odor,

Vapor Pressure Of Can (psig @ 70°F):	62

TLVIPEL:

50 ppm

400 ppm

1000 ppm

SECTION 6 - SPECIAL PROTECTION DATA

Resgiratory_ProtecUon: None needed for proper use In accordance with label directions.

EyjJirotecjjorK None needed unless it is anticipated that a splash or spray back will occur, then wear

safety glasses or chemical proof goggles.

Pl9tgct!.Y.§....S.!2ies: None needed for proper use in accordance with label directions. Use chemical

resistant gloves if hand contact will be made,

Ventilation Requirements:  Provide local exhaust to keep TLV below acceptable limits.

SECTION 7 - REACTIVITY DATA

teJateriaiStable? Yes

iM&iUfi&tMHy.! Avoid contact with strong oxidizing agents.

Hazardous Decomposition Products: Carbon Dioxice, Carbon Monoxide.

Hazardous Polymerization Occur: Will not occur.

SECTION 3 - FIRE AND EXPLOSION DATA

Flash Pgintfof concentrate only): 80°F

E!S!ffia8HBfidiI-Bai^MJ!iMjl!^§^[2BJfett Extremely Flammable Spray. Extinguislung Media: Foam, C02, dry media,

UnusuaLFire oiJxjIosjojiJiazards: Exposure to temperature above 120*F may cause bursting, §fi^!iLEl£SfisMllfl£l5cedures: Wear self-contained breathing apparatus and protective clothing. Cool tire exposed containers to prevent rupturing.

SECTION 8 - SPILL OR LEAK PROCEDURES

StegsTo Je JaJ^jf^ajeii^lJte_SByi^_Or_Reieased: Allow propellant to evaporate. Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources, and open flame far away from spill or leak. Cover with absorbent material and sweep up. Wash area to prevent slipping. Dispose of soaked absorbent material in accordance with Federal, State and local laws/

osal Method- Aerosol cans when emptied and depressurized through normal use, pose no

disposal hazard and should be recycled. Consult Federal, State, and local authorities for approved procedures,

SECTION 4 - HEALTH HAZARD INFORMATION (Hazard By Routes Of Exposure)

Eyes: May cause slight irritation but does not injure eye tissue. Skin: Frequent or prolonged contact may cause irritation. Inhalation: Inhalation of rnist can cause irritation of nasal and respiratory passages. Abusive or excessive inhalation may cause irritation to the upper respiratory tract, dizziness, nausea and other central nervous system effects. Ingestion: Can cause gastrointestinal irritation, nausea, vomiting and diarrhea. Aspiration of material into the lungs can cause chemical pneumonitis. Any of Part 1 Listed As Carcinogens?   (NTP, (ARC, OSHA): Yes,

SECTION 9 - SPECIAL PRECAUTIONS

SgecjaJ_Pj5£ajjtipjn^ForJte|j^Sgjg^^j»aij Store in a cool dry area away from heat or open

flame. Do not store at temperatures above 120"F.

Other Precautions: Keep out of reach of children, For industrial and institutional use only.

The information contained heiein is based on the data available to us and is believed to he correct. However, Ai makes no warranty, expressed or implied, regarding the accuracy of these data or the results to be obtained from the use

thereof. Ai assumes no responsibility for injury from the use of the products described herein.